3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220347

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2404 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR test case 8.1.4.1.8.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR test case 8.1.4.1.8.3 to the IWD_21wk49 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.1.4.1.8.3 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.8.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	4

2.1	Function f_TC_8_1_4_1_8_x_NR_Common	4

3	Branches executed	6

4	Execution Log Files	6

MediaTek MT6983 UE	6

5	References	6

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.1.4.1.8.3 which is part of the NR5GC test suite in iWD_TTCN3-B2020-09_D21wk49.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	8.1.4.1.8.3

ATS Version:	iwd-TTCN3-B2020-09_D21wk49

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	MediaTek MT6983

Verification Status:	PASS

# Corrections required

## Function f_TC_8_1_4_1_8_x_NR_Common

| Function name | f_TC_8_1_4_1_8_x_NR_Common |
| --- | --- |
| Reason for change | Common initialization all SCell with the same timing as the PSCell is not taken care ; at step 6 , when moving from cell 1 to cell 2 as as result of handover , cell 2 and cell 3 will be having different time offset. |
| Summary of change | Pcell ( Cell 1 ) , target Pcell ( Cell 2 ) and Scell ( Cell 3/10 ) are initialized to have same Timing Offset |
| TTCN module | RRC_NR_CA_IntraNR_Handover_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_4_1_8_x_NR_Common (NR_CellId_Type p_SourcePCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested, NR_CellId_Type p_TargetPCell) runs on NR5GC_PTC { f_NR5GC_Init_CA(NR_4, {p_SourcePCell, p_SCell}, p_NR_CA_Tested); // Create and configure cells f_NR_CellConfig_PCell_Def(p_SourcePCell); f_NR_CellConfig_SCell(p_SCell); f_NR_CellConfig_PCell_Def(p_TargetPCell); // Preamble: If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3. // Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3. f_NR5GC_Preamble (p_SourcePCell, STATE_CONNECTED_3A, PING_Or_TEST_LOOPModeB_ON); f_NR_TestBody_Set(true); f_TC_8_1_4_1_8_x_NR5GC_TestBody(p_SourcePCell, p_SCell, p_TargetPCell); f_NR_TestBody_Set(false); if (not pc_IP_Ping) { f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(p_TargetPCell); } // Switch/Power off UE f_NR_Postamble(p_TargetPCell, STATE_CONNECTED_3A); } //end of f_TC_8_1_4_1_8_x_NR_Common

After Change:

function f_TC_8_1_4_1_8_x_NR_Common (NR_CellId_Type p_SourcePCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested, NR_CellId_Type p_TargetPCell) runs on NR5GC_PTC { f_NR5GC_Init_CA(NR_4, {p_SourcePCell, p_SCell,p_TargetPCell}, p_NR_CA_Tested); // Create and configure cells f_NR_CellConfig_PCell_Def(p_SourcePCell); f_NR_CellConfig_SCell(p_SCell); f_NR_CellConfig_PCell_Def(p_TargetPCell); // Preamble: If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3. // Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3. f_NR5GC_Preamble (p_SourcePCell, STATE_CONNECTED_3A, PING_Or_TEST_LOOPModeB_ON); f_NR_TestBody_Set(true); f_TC_8_1_4_1_8_x_NR5GC_TestBody(p_SourcePCell, p_SCell, p_TargetPCell); f_NR_TestBody_Set(false); if (not pc_IP_Ping) { f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(p_TargetPCell); } // Switch/Power off UE f_NR_Postamble(p_TargetPCell, STATE_CONNECTED_3A); } //end of f_TC_8_1_4_1_8_x_NR_Common

# Branches executed

## This NR TC was executed in 5GS Stand Alone mode (Option 2) on Intra-Band Non-Contiguous CA band “CA_n66_2A”.

# Execution Log Files

## MediaTek MT6983 UE

The MediaTek MT6983 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_8_1_4_1_8_3_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s220348: NR5GC FR1 : Supporting information for addition of NR testcase 8.1.4.1.8.3 |
| --- | --- |