3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220354

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2406 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the NR5GC testcases 8.1.4.1.9.1, 8.1.4.1.9.2 and 8.1.4.1.9.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN Implementation, 1. At Step 7 , SS transmits Reconfiguration message on Nr cell 1 , to release scell ( Nr Cell3 / 10) and UE to perform Inter frequency handover to Nr Cell 3 / 10 . But the network configuration is missing for the same , release scell has to be done at network end as well so that Dci switch can happen, in order to reconfigure DAI as single serving cell 2.At Step 7 and 18 , reconfiguration is transmitted to do Handover to Cell 3 / 10 for which UE may not send Harq ACK, which leads to network doing retransmissions; To prevent this retransmission RB release and reconfiguration is required ; this problem point is already addressed with R5s220201 3. At Step 9 , 10, 11 , 12 and 13 p_MacBearerRoutingCA has to be set to Omit as CA is not configured during these steps 4.At Step 12-13 , reconfiguration message is sent to resume existing radio bearers on NR cell 1 and to add Nr cell 3 / 10 as Scell; It is required to make sure , in addition to scell addition , reconfiguration DAI as multiple serving cell is also required . It is proposed to send reconfiguration to resume existing radio beares on Nr cell 1 at steps 12-13, and at step 12A-12B ( new Steps ) , SS can be configured with addition of scells and send peer message to UE for addition of Scells . this will make sure reconfiguration DAI as multiple serving cell is done before sending Step14 , Reconfiguration message to setup Event A3 – Prose will be raised for this change 5. At step 17 , SS is configured for NO rach response for cell 3 / 10 ; at step 18 , reconfiguration message on Nr cell 1 is sent to do Handover to cell 3 / 10 ; as per the test case prose , UE has to initiate reeestablishment request on Cell 3 / 10 . With immeidate timing at step 20 , Nr cell 3 / 10 is configured with Rar response without waiting for T304 duration . It is required to wait for the duration of T304 before calling f_NR5GC_RRCReestablishment_Def at step 20 , so that Rach sent by UE is responded back for reestablishment request . Providing rar grant immediately at step 20 will make the SS to respond for the rach procedure triggered for reconfiguration complete HO) procedure 6.In function f_NR5GC_RRCReestablishment_Steps8To13 at step 13, f_NR_GetActiveDRBs has to be queried for Source cell id instead of target cell id ; so that rb release and configuration can be done for source cell id |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Release Scell is done at Network end as well at step 7 which is also helping to reconfigure DAI as single serving cell 2. RB release and reconfiguration is done at Step 7 and 18 , so that network can stop doing the retransmission for handover Message 3. At Step 9,10,11,12 and 13 p_MacBearerRoutingCA is set as 'Omit' as CA is not configured during these steps 4. At step 12-13 , reconfiguration message is sent only to resume existing radio bearers on NR cell 1 as a new step 12A-13A , addition of Scell ( NR cell 3/10 ) is done which is also helping to reconfigure DAI as multiple serving cell 5. At step 20 , f_NR5GC_RRCReestablishment_Def is called after the duration of T304 , so that Network will respond to the rach which is received for the purpose of reestablishment request ( not for handover reconfiguration complete ) 6. In function f_NR5GC_RRCReestablishment_Steps8To13 , at step 13 , f_NR_GetActiveDRBs is called for source cell id |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.9.1.NR5GC, 8.1.4.1.9.2.NR5GC and 8.1.4.1.9.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to the function f_TC_8_1_4_1_9_x_NR5GC_TestBody()	3

1.2	Correction to the function f_NR5GC_RRCReestablishment_Steps8To13	15

1.3	Correction to the module RRC_NR_CA_IntraNR_Handover_NR5GC	16

# Corrections required

## Correction to the function f_TC_8_1_4_1_9_x_NR5GC_TestBody()

| Function name | f_TC_8_1_4_1_9_x_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current TTCN Implementation, 1. At Step 7 , SS transmits Reconfiguration message on Nr cell 1 , to release scell ( Nr Cell3 / 10) and UE to perform Inter frequency handover to Nr Cell 3 / 10 . But the network configuration is missing for the same , release scell has to be done at network end as well so that Dci switch can happen, in order to reconfigure DAI as single serving cell 2.At Step 7 and 18 , reconfiguration is transmitted to do Handover to Cell 3 / 10 for which UE may not send Harq ACK, which leads to network doing retransmissions; To prevent this retransmission RB release and reconfiguration is required ; this problem point is already addressed with R5s220201 3. At Step 9 , 10, 11 , 12,13 p_MacBearerRoutingCA has to be set to Omit as CA is not configured during these steps 4.At Step 12-13 , reconfiguration message is sent to resume existing radio bearers on NR cell 1 and to add Nr cell 3 / 10 as Scell; It is required to make sure , in addition to scell addition , reconfiguration DAI as multiple serving cell is also required . It is proposed to send reconfiguration to resume existing radio beares on Nr cell 1 at steps 12-13, and at step 12A-12B ( new Steps ) , SS can be configured with addition of scells and send peer message to UE for addition of Scells . this will make sure reconfiguration DAI as multiple serving cell is done before sending Step14 , Reconfiguration message to setup Event A3 – Prose will be raised for this change 5. At step 17 , SS is configured for NO rach response for cell 3 / 10 ; at step 18 , reconfiguration message on Nr cell 1 is sent to do Handover to cell 3 / 10 ; as per the test case prose , UE has to initiate reeestablishment request on Cell 3 / 10 . With immeidate timing at step 20 , Nr cell 3 / 10 is configured with Rar response without waiting for T304 duration . It is required to wait for the duration of T304 before calling f_NR5GC_RRCReestablishment_Def at step 20 , so that Rach sent by UE is responded back for reestablishment request . Providing rar grant immediately at step 20 will make the SS to respond for the rach procedure triggered for reconfiguration complete HO) procedure |
| Summary of change | 1. Release Scell is done at Network end as well at step 7 which is also helping to reconfigure DAI as single serving cell 2. RB release and reconfiguration is done at Step 7 and 18 , so that network can stop doing the retransmission for handover Message 3. At Step 9,10,11,12 and 13 p_MacBearerRoutingCA is set as 'Omit' as CA is not configured during these steps 4. At step 12-13 , reconfiguration message is sent only to resume existing radio bearers on NR cell 1 as a new step 12A-13A , addition of Scell ( NR cell 3/10 ) is done which is also helping to reconfigure DAI as multiple serving cell 5. At step 20 , f_NR5GC_RRCReestablishment_Def is called after the duration of T304 , so that Network will respond to the rach which is received for the purpose of reestablishment request ( not for handover reconfiguration complete ) |
| TTCN module | NR5GC\8_1_4\RRC_NR_CA_IntraNR_Handover_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_4_1_9_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { var template (value) DL_DCCH_Message v_RRCReconfigurationHO; var template(value) CellGroupConfig v_SCGConfig; var template (value) NR_DRBInfoList_Type v_DRBInfoList; var template (value) NR_RadioBearerList_Type v_RadioBearerList; var integer v_Rsrp; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (omit) MeasConfig v_MeasConfig; var PhysCellId v_PhysCellIdServing; var PhysCellId v_PhysCellIdNeighbour; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0 :={cs_NR_CellPower(p_PCell, -85, -85), //FR2 power levels are FFS cs_NR_CellPower(p_SCell, -91, -91) //FR2 power levels are FFS }; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1orT3 :={cs_NR_CellPower(p_PCell, -85, -85), //FR2 power levels are FFS cs_NR_CellPower(p_SCell, -79, -79) //FR2 power levels are FFS }; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 :={cs_NR_CellPower(p_PCell, -85, -85), //FR2 power levels are FFS cs_NR_CellPower(p_SCell, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT4 :={cs_NR_CellPower(p_PCell, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(p_SCell, -79, -79) //FR2 power levels are FFS }; //The SS changes Cell parameters according to the row "T0" in table 8.1.4.1.8.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to configure NR Cell 3/10 as an SCell. //The UE transmits the RRCReconfigurationComplete message on NR Cell 1? f_NR5GC_AddMod_1SCell(p_PCell, p_SCell); // NR Cell1, NR Cell3/10 //@siclog "Steps 3-4" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to setup event A3 reporting configuration //The UE transmits an RRCReconfigurationComplete message on NR Cell 1. if(f_NR_CellInfo_GetIsFR1(p_PCell)) { v_Rsrp := 2; // @sic R5-211391 sic@ } else { v_Rsrp := 0; //FFS dummy value FR2 } v_MeasuredObjectList := {cs_MeasObjectId1(p_PCell), cs_MeasObjectId2(p_SCell)}; v_ReportConfigList := {cs_NR_ReportConfigEventA3(tsc_NR_IdReportConfigId1, v_Rsrp, -, -, -, cs_NR_MeasReportQuantity_All, ms2560)}; v_MeasConfig := f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, {cs_NR_MeasIdToAddMod(-, tsc_NR_MeasObjectId2)}); //@sic R5-211391 sic@ //@sic R5-213516 sic@ f_NR_SendRRCReconfiguration_MeasNR(p_PCell, v_MeasConfig, tsc_NR_MeasObjectId2, tsc_NR_MeasObjectId1, -, -, -, crs_MacBearerRouting(p_PCell)); //@sic R5-2135166, R5-217997 (added tsc_NR_MeasObjectId2 as third parameter) sic@ //@siclog "Step 5" siclog@ //The SS changes NR Cell 1 and NR Cell 3/10 parameters according to the row "T1" in table 8.1.4.1.9.1.3.2-1/1A f_NR_SetCellPowerList(v_CellPowerList_AtT1orT3); //@siclog "Step 6" siclog@ //The UE transmits a MeasurementReport message on NR Cell 1 to report event A3 with the measured RSRP, RSRQ value for NR Cell 3/10. v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(p_PCell); v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(p_SCell); f_NR_ReceiveMeasurementReports(p_PCell, v_PhysCellIdServing, tsc_NR_MeasId1, -, -, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour, f_NR_Get_MeasQuantityResults())}), -, -, -, -, -, -, crs_MacBearerRouting(p_PCell), //@sic R5-213516 sic@ true); //p_MeasResultServingMOList_NotChecked @sic R5-217997 sic@ //Config target cell for no RACH response f_NR_SS_RachProcedureConfig(p_SCell, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 7" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform inter frequency handover to NR Cell 3/10 and to release SCell NR Cell 3. v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(p_SCell, -,-,-,-,-,-,-,-,-, {tsc_SCellIndex_1} //SCellToReleaseList ); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO, crs_MacBearerRouting(p_PCell))); //@sic R5-213516 sic@ //@siclog "Step 8" siclog@ //The SS changes NR Cell 1 and NR Cell 3/10 parameters according to the row "T2" in table 8.1.4.1.9.1.3.2-1/1A f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@siclog "Step 9" siclog@ //Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 1? // @sic R5-211391 sic@ f_NR5GC_RRCReestablishment_Steps1To2(p_PCell, p_PCell, handoverFailure, tsc_C_RNTI_Value4, "Step 9", crs_MacBearerRouting(p_PCell)); //@sic R5-211391 R5-213516 R5-213516 sic@ //@siclog "Steps 10-11" siclog@ //The SS transmits an RRCReestablishment message to resume SRB1 operation and re-activate security on NR Cell 1. //The UE transmits an RRCReestablishmentComplete message f_NR5GC_RRCReestablishment_Steps3To7(p_PCell, p_PCell, 0, crs_MacBearerRouting(p_PCell)); //@sic R5-213516 sic@ //@siclog "Step 11A" siclog@ //The SS changes NR Cell 1 and NR Cell 3/10 parameters according to the row "T3" in table 8.1.4.1.9.1.3.2-1/1A f_NR_SetCellPowerList(v_CellPowerList_AtT1orT3); //@siclog "Steps 12-13" siclog@ //The SS transmits an RRCReconfiguration message to resume existing radio bearer on NR Cell 1 and configure NR Cell 3/10 as an SCell //The UE transmits an RRCConnectionReconfigurationComplete message on NR Cell 1 v_SCGConfig := f_NR_GetSCGConfig_SCellAdd(p_SCell, tsc_SCellIndex_1); f_NR5GC_RRCReestablishment_Steps8To13(p_PCell, p_PCell, v_SCGConfig.sCellToAddModList, omit, crs_MacBearerRouting(p_PCell)); //@sic R5-213516 sic@ //@siclog "Steps 14-15" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to setup event A3 reporting configuration //The UE transmits an RRCReconfigurationComplete message on NR Cell 1. f_NR_SendRRCReconfiguration_MeasNR(p_PCell, v_MeasConfig, tsc_NR_MeasObjectId2, tsc_NR_MeasObjectId1, -, -, -, crs_MacBearerRouting(p_PCell)); //@sic R5-213516, R5-217997 (added tsc_NR_MeasObjectId2 as third parameter) sic@ //@siclog "Step 16" siclog@ //@sic R5-211391 sic@ //@siclog "Step 17" siclog@ //The UE transmits a MeasurementReport message on NR Cell 1 to report event A3 with the measured RSRP, RSRQ value for NR Cell 3/10. v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(p_PCell); v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(p_SCell); f_NR_ReceiveMeasurementReports(p_PCell, v_PhysCellIdServing, tsc_NR_MeasId1, -, -, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour, f_NR_Get_MeasQuantityResults())}), -, -, -, -, -, -, crs_MacBearerRouting(p_PCell), true); //p_MeasResultServingMOList_NotChecked @sic R5-217997 sic@ //Config target cell for no RACH response f_NR_SS_RachProcedureConfig(p_SCell, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 18" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 3/10 and to release SCell NR Cell3/10. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO, crs_MacBearerRouting(nr_Cell1))); //@sic R5-213516 sic@ //Reconfigure SCell to normal cell v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); v_RadioBearerList := f_NR5GC_GetRadioBearerList(v_DRBInfoList); f_NR_ReconfigSCellToNormalCell(p_SCell, v_RadioBearerList, -); //@siclog "Step 19" siclog@ //The SS changes NR Cell 1 and NR Cell 3/10 parameters according to the row "T4" in table 8.1.4.1.9.1.3.2-1/1A f_NR_SetCellPowerList(v_CellPowerList_AtT4); //@siclog "Step 20" siclog@ //Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 3/10? //@siclog "Steps 21-22" siclog@ //The SS transmits an RRCReestablishment message to resume SRB1 operation and re-activate security on NR Cell 3/10. //The UE transmits an RRCReestablishmentComplete message //@siclog "Steps 23-24" siclog@ //The SS transmits an RRCReconfiguration message to resume existing radio bearer on NR Cell 3/10 //The UE transmits an RRCConnectionReconfigurationComplete message on NR Cell 3/10 f_NR5GC_RRCReestablishment_Def(p_PCell, p_SCell, -, -, -,-, "Step 20"); } }

After Change:

function f_TC_8_1_4_1_9_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { var template (value) DL_DCCH_Message v_RRCReconfigurationHO; var template(value) CellGroupConfig v_SCGConfig; var template (value) NR_DRBInfoList_Type v_DRBInfoList; var template (value) NR_RadioBearerList_Type v_RadioBearerList; var integer v_Rsrp; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (omit) MeasConfig v_MeasConfig; var PhysCellId v_PhysCellIdServing; var PhysCellId v_PhysCellIdNeighbour; var IntegerList_Type v_DrbIdList; //WA#WI=929545 var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0 :={cs_NR_CellPower(p_PCell, -85, -85), //FR2 power levels are FFS cs_NR_CellPower(p_SCell, -91, -91) //FR2 power levels are FFS }; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1orT3 :={cs_NR_CellPower(p_PCell, -85, -85), //FR2 power levels are FFS cs_NR_CellPower(p_SCell, -79, -79) //FR2 power levels are FFS }; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 :={cs_NR_CellPower(p_PCell, -85, -85), //FR2 power levels are FFS cs_NR_CellPower(p_SCell, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT4 :={cs_NR_CellPower(p_PCell, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(p_SCell, -79, -79) //FR2 power levels are FFS }; var float v_T304Min_1sec := f_NR_SetTimerToleranceMin (nr_Cell1, rrcTimer, 1.0); //WA#WI=929545 timer t_T304Expiry; //WA#WI=929545 //The SS changes Cell parameters according to the row "T0" in table 8.1.4.1.8.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT0); //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to configure NR Cell 3/10 as an SCell. //The UE transmits the RRCReconfigurationComplete message on NR Cell 1? f_NR5GC_AddMod_1SCell(p_PCell, p_SCell); // NR Cell1, NR Cell3/10 //@siclog "Steps 3-4" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to setup event A3 reporting configuration //The UE transmits an RRCReconfigurationComplete message on NR Cell 1. if(f_NR_CellInfo_GetIsFR1(p_PCell)) { v_Rsrp := 2; // @sic R5-211391 sic@ } else { v_Rsrp := 0; //FFS dummy value FR2 } v_MeasuredObjectList := {cs_MeasObjectId1(p_PCell), cs_MeasObjectId2(p_SCell)}; v_ReportConfigList := {cs_NR_ReportConfigEventA3(tsc_NR_IdReportConfigId1, v_Rsrp, -, -, -, cs_NR_MeasReportQuantity_All, ms2560)}; v_MeasConfig := f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, {cs_NR_MeasIdToAddMod(-, tsc_NR_MeasObjectId2)}); //@sic R5-211391 sic@ //@sic R5-213516 sic@ f_NR_SendRRCReconfiguration_MeasNR(p_PCell, v_MeasConfig, tsc_NR_MeasObjectId2, tsc_NR_MeasObjectId1, -, -, -, crs_MacBearerRouting(p_PCell)); //@sic R5-2135166, R5-217997 (added tsc_NR_MeasObjectId2 as third parameter) sic@ //@siclog "Step 5" siclog@ //The SS changes NR Cell 1 and NR Cell 3/10 parameters according to the row "T1" in table 8.1.4.1.9.1.3.2-1/1A f_NR_SetCellPowerList(v_CellPowerList_AtT1orT3); //@siclog "Step 6" siclog@ //The UE transmits a MeasurementReport message on NR Cell 1 to report event A3 with the measured RSRP, RSRQ value for NR Cell 3/10. v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(p_PCell); v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(p_SCell); f_NR_ReceiveMeasurementReports(p_PCell, v_PhysCellIdServing, tsc_NR_MeasId1, -, -, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour, f_NR_Get_MeasQuantityResults())}), -, -, -, -, -, -, crs_MacBearerRouting(p_PCell), //@sic R5-213516 sic@ true); //p_MeasResultServingMOList_NotChecked @sic R5-217997 sic@ //Config target cell for no RACH response f_NR_SS_RachProcedureConfig(p_SCell, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 7" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform inter frequency handover to NR Cell 3/10 and to release SCell NR Cell 3. v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(p_SCell, -,-,-,-,-,-,-,-,-, {tsc_SCellIndex_1} //SCellToReleaseList ); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO, crs_MacBearerRouting(p_PCell))); //@sic R5-213516 sic@ f_NR_Rel_SCell_Common( nr_Cell1, p_SCell, tsc_PCellIndex_0, cs_TimingInfo_Now); //WA#WI=929545 //@siclog "Step 8" siclog@ //The SS changes NR Cell 1 and NR Cell 3/10 parameters according to the row "T2" in table 8.1.4.1.9.1.3.2-1/1A f_NR_SetCellPowerList(v_CellPowerList_AtT2); v_DrbIdList := f_NR_GetActiveDRBs(nr_Cell1); //WA#WI=929545 f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, v_DrbIdList); //@sic R5-211417 R5s211169 sic@ //WA#WI=929545 f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit); //WA#WI=929545 //@siclog "Step 9" siclog@ //Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 1? // @sic R5-211391 sic@ f_NR5GC_RRCReestablishment_Steps1To2(p_PCell, p_PCell, handoverFailure, tsc_C_RNTI_Value4, "Step 9"/*, crs_MacBearerRouting(p_PCell)*/); //@sic R5-211391 R5-213516 R5-213516 sic@ //WA#WI=929545 //@siclog "Steps 10-11" siclog@ //The SS transmits an RRCReestablishment message to resume SRB1 operation and re-activate security on NR Cell 1. //The UE transmits an RRCReestablishmentComplete message f_NR5GC_RRCReestablishment_Steps3To7(p_PCell, p_PCell, 0/*, crs_MacBearerRouting(p_PCell)*/); //@sic R5-213516 sic@ //WA#WI=929545 //@siclog "Step 11A" siclog@ //The SS changes NR Cell 1 and NR Cell 3/10 parameters according to the row "T3" in table 8.1.4.1.9.1.3.2-1/1A //f_NR_SetCellPowerList(v_CellPowerList_AtT1orT3); //WA#WI=929545 //@siclog "Steps 12-13" siclog@ //The SS transmits an RRCReconfiguration message to resume existing radio bearer on NR Cell 1 and configure NR Cell 3/10 as an SCell //The UE transmits an RRCConnectionReconfigurationComplete message on NR Cell 1 v_SCGConfig := f_NR_GetSCGConfig_SCellAdd(p_SCell, tsc_SCellIndex_1); f_NR5GC_RRCReestablishment_Steps8To13(p_PCell, p_PCell, /*v_SCGConfig.sCellToAddModList*/-, omit /*crs_MacBearerRouting(p_PCell)*/); //@sic R5-213516 sic@ //WA#WI=929545 //@siclog "Step 11A" siclog@ //The SS changes NR Cell 1 and NR Cell 3/10 parameters according to the row "T3" in table 8.1.4.1.9.1.3.2-1/1A f_NR_SetCellPowerList(v_CellPowerList_AtT1orT3); //WA#WI=929545 //@siclog "Steps 12A-13A" siclog@ f_NR5GC_AddMod_1SCell(p_PCell, p_SCell); //WA#WI=929545 //@siclog "Steps 14-15" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to setup event A3 reporting configuration //The UE transmits an RRCReconfigurationComplete message on NR Cell 1. f_NR_SendRRCReconfiguration_MeasNR(p_PCell, v_MeasConfig, tsc_NR_MeasObjectId2, tsc_NR_MeasObjectId1, -,-, -, crs_MacBearerRouting(p_PCell)); //@sic R5-213516, R5-217997 (added tsc_NR_MeasObjectId2 as third parameter) sic@ //@siclog "Step 16" siclog@ //@sic R5-211391 sic@ //@siclog "Step 17" siclog@ //The UE transmits a MeasurementReport message on NR Cell 1 to report event A3 with the measured RSRP, RSRQ value for NR Cell 3/10. v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(p_PCell); v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(p_SCell); f_NR_ReceiveMeasurementReports(p_PCell, v_PhysCellIdServing, tsc_NR_MeasId1, -, -, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour, f_NR_Get_MeasQuantityResults())}), -, -, -, -, -, -, crs_MacBearerRouting(p_PCell), true); //p_MeasResultServingMOList_NotChecked @sic R5-217997 sic@ //Config target cell for no RACH response f_NR_SS_RachProcedureConfig(p_SCell, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 18" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 3/10 and to release SCell NR Cell3/10. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO, crs_MacBearerRouting(nr_Cell1))); //@sic R5-213516 sic@ //Reconfigure SCell to normal cell v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); v_RadioBearerList := f_NR5GC_GetRadioBearerList(v_DRBInfoList); f_NR_ReconfigSCellToNormalCell(p_SCell, v_RadioBearerList, -); //@siclog "Step 19" siclog@ //The SS changes NR Cell 1 and NR Cell 3/10 parameters according to the row "T4" in table 8.1.4.1.9.1.3.2-1/1A f_NR_SetCellPowerList(v_CellPowerList_AtT4); v_DrbIdList := f_NR_GetActiveDRBs(nr_Cell1); //WA#WI=929545 f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, v_DrbIdList); //@sic R5-211417 R5s211169 sic@ //WA#WI=929545 f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit); //WA#WI=929545 t_T304Expiry.start(v_T304Min_1sec); //WA#WI=929545 t_T304Expiry.timeout; //WA#WI=929545 //@siclog "Step 20" siclog@ //Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 3/10? //@siclog "Steps 21-22" siclog@ //The SS transmits an RRCReestablishment message to resume SRB1 operation and re-activate security on NR Cell 3/10. //The UE transmits an RRCReestablishmentComplete message //@siclog "Steps 23-24" siclog@ //The SS transmits an RRCReconfiguration message to resume existing radio bearer on NR Cell 3/10 //The UE transmits an RRCConnectionReconfigurationComplete message on NR Cell 3/10 f_NR5GC_RRCReestablishment_Def(p_PCell, p_SCell, -, -, -,-, "Step 20"); }

## Correction to the function f_NR5GC_RRCReestablishment_Steps8To13

| Function name | f_NR5GC_RRCReestablishment_Steps8To13 |
| --- | --- |
| Reason for change | In the current TTCN Implementation, In function f_NR5GC_RRCReestablishment_Steps8To13 at step 13, f_NR_GetActiveDRBs has to be queried for Source cell id instead of target cell id ; so that rb release and configuration can be done for source cell id |
| Summary of change | In function f_NR5GC_RRCReestablishment_Steps8To13 , at step 13 , f_NR_GetActiveDRBs is called for source cell id |
| TTCN module | Common\NR5GC\NR5GC_RRCSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_RRCReestablishment_Steps8To13(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template (omit) NR_SCellToAddModList_Type p_SCellToAddModList := omit, template(omit) SpCellConfig.rlf_TimersAndConstants p_RlfTimersAndConstants := omit, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit, boolean p_ReceiveForRRCreconfigurationComplete := true ) runs on NR5GC_PTC { << SKIPPED CODE >> // 13. Source Cell: If the Source Cell is different from the Target Cell: // Release security. // Reset SRBs and release DRBs. // (Re-)configure SRBs. // Stop periodic UL grant and periodic TA. if (p_NR_SourceCellId != p_NR_TargetCellId) { f_NR_SS_RRC_ReleaseSecurity(p_NR_SourceCellId, cs_TimingInfo_Now); //@sic R5s211165 sic@ v_DrbIdList := f_NR_GetActiveDRBs(p_NR_TargetCellId); f_NR_SS_SRBs_DRBs_Release(p_NR_SourceCellId, cs_TimingInfo_Now, v_DrbIdList, tsc_NoCnfReq); //@sic R5-211417 R5s211165 sic@ f_NR_SS_SRBs_DRBs_Config(p_NR_SourceCellId, cs_TimingInfo_Now, omit, tsc_FollowOnFlag); f_NR_ULGrantConfiguration_Stop(p_NR_SourceCellId, -, cs_NR_UplinkTimeAlignment_Stop); } }

After Change:

function f_NR5GC_RRCReestablishment_Steps8To13(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template (omit) NR_SCellToAddModList_Type p_SCellToAddModList := omit, template(omit) SpCellConfig.rlf_TimersAndConstants p_RlfTimersAndConstants := omit, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit, boolean p_ReceiveForRRCreconfigurationComplete := true ) runs on NR5GC_PTC { << SKIPPED CODE >> // 13. Source Cell: If the Source Cell is different from the Target Cell: // Release security. // Reset SRBs and release DRBs. // (Re-)configure SRBs. // Stop periodic UL grant and periodic TA. if (p_NR_SourceCellId != p_NR_TargetCellId) { f_NR_SS_RRC_ReleaseSecurity(p_NR_SourceCellId, cs_TimingInfo_Now); //@sic R5s211165 sic@ v_DrbIdList := f_NR_GetActiveDRBs(p_NR_SourceCellId); //WA#WI=929545 f_NR_SS_SRBs_DRBs_Release(p_NR_SourceCellId, cs_TimingInfo_Now, v_DrbIdList, tsc_NoCnfReq); //@sic R5-211417 R5s211165 sic@ f_NR_SS_SRBs_DRBs_Config(p_NR_SourceCellId, cs_TimingInfo_Now, omit, tsc_FollowOnFlag); f_NR_ULGrantConfiguration_Stop(p_NR_SourceCellId, -, cs_NR_UplinkTimeAlignment_Stop); } }

## Correction to the module RRC_NR_CA_IntraNR_Handover_NR5GC

| Module name | RRC_NR_CA_IntraNR_Handover_NR5GC |
| --- | --- |
| Reason for change | For the compilation purpose of the changes made in section 1.1 |
| Summary of change | Modules are imported for the compilation purpose of the changes made in section 1.1 |
| TTCN module | \NR5GC\8_1_4\RRC_NR_CA_IntraNR_Handover_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

module RRC_NR_CA_IntraNR_Handover_NR5GC { import from CommonDefs all; import from NR_RRC_ASN1_Definitions language "ASN.1:2002" all with {encode "UNALIGNED_PER_OctetAligned"}; import from NR_Component_Base all; import from NR_Component_NR5GC all; import from NR_CommonDefs all; import from NG_NAS_Common all; import from EUTRA_NR_Parameters all; import from NR_ASP_TypeDefs all; import from NR_ConfigurationSteps all; import from NR5GC_CommonFunctions all; import from NR5GC_NASSteps all; import from NR_FrequencyCommon all; import from NG_NASTemplateFunctions all; import from NR_CellInfo all; import from NR5GC_LoopBack all; import from NR_ASP_Templates_SRB all; import from Common4G5G_Templates all; import from NR5GC_RRCSteps all; import from NR_Measurement_Functions all; import from NR_RRC_Templates all; import from NR_CellCfg_Templates all; import from NR_Measurement_Templates all; import from NR_ASP_Templates_CommonPart all; import from NR_CommonFunctions_CA all; import from NR5GC_CommonFunctions_CA all; import from NR5GC_Handover all;

After Change:

module RRC_NR_CA_IntraNR_Handover_NR5GC { import from CommonDefs all; import from NR_RRC_ASN1_Definitions language "ASN.1:2002" all with {encode "UNALIGNED_PER_OctetAligned"}; import from NR_Component_Base all; import from NR_Component_NR5GC all; import from NR_CommonDefs all; import from NG_NAS_Common all; import from EUTRA_NR_Parameters all; import from NR_ASP_TypeDefs all; import from NR_ConfigurationSteps all; import from NR5GC_CommonFunctions all; import from NR5GC_NASSteps all; import from NR_FrequencyCommon all; import from NG_NASTemplateFunctions all; import from NR_CellInfo all; import from NR5GC_LoopBack all; import from NR_ASP_Templates_SRB all; import from Common4G5G_Templates all; import from NR5GC_RRCSteps all; import from NR_Measurement_Functions all; import from NR_RRC_Templates all; import from NR_CellCfg_Templates all; import from NR_Measurement_Templates all; import from NR_ASP_Templates_CommonPart all; import from NR_CommonFunctions_CA all; import from NR5GC_CommonFunctions_CA all; import from NR5GC_Handover all; import from NR_SecuritySteps all; //WA#WI=929545 import from NR_Timing all; //WA#WI=929545