3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220410

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2247 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of Rel-16 SON-MDT test case 8.1.6.1.4.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies, Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add Rel-16 SON-MDT test case 8.1.6.1.4.6 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to Rel-16 SON-MDT test case 8.1.6.1.4.6 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.4.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

3	Branches executed	5

4	Execution Log Files	5

4.1	Qualcomm SM 8450, SDX65	5

4.2	Mediatek MT6983	5

5	References	6

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.6.1.4.6 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

Ramakrishnan Thiruvathirai

Ramakrishnan.Thiruvathirai@rohde-schwarz.com

## Verification Test Summary

Test Case: 	8.1.6.1.4.6.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk49

System Simulator used:	Keysight S8704A Protocol Conformance Toolset and R&S® 5G Protocol Conformance Test platform

UE used:	Qualcomm SM 8450, SDX65 and Mediatek MT6983

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_GetTestcaseAttrib_EarlyContentionResolution() |
| --- | --- |
| Reason for change | Add this test case to early contention attribute list due to SS not responding to RRCSetupRequest at step 5 |
| Summary of change | Added test case to use early contention Resolution. |
| TTCN module | TestcaseProperties |
| MCC160 Comment |  |

Before Change:

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { … … case ("TC_8_1_6_1_4_1_NR5GC") { return true; } case ("TC_8_1_6_1_4_2_NR5GC") { return true; } case ("TC_8_1_6_1_4_3_NR5GC") { return true; } case ("TC_8_1_6_1_4_4_NR5GC") { return true; } case ("TC_8_1_6_1_4_5_NR5GC") { return true; } }

After Change:

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { … … case ("TC_8_1_6_1_4_1_NR5GC") { return true; } case ("TC_8_1_6_1_4_2_NR5GC") { return true; } case ("TC_8_1_6_1_4_3_NR5GC") { return true; } case ("TC_8_1_6_1_4_4_NR5GC") { return true; } case ("TC_8_1_6_1_4_5_NR5GC") { return true; } case ("TC_8_1_6_1_4_6_NR5GC") { return true; } }

## Change 2

| Function name | cr_MeasResultListNR_1Entry |
| --- | --- |
| Reason for change | At Step 17 the UEInformationResponse when received will include the IE measResultNeighCellListNR, within this IE certain IEs are set to Not present. According to 38.331 sec 5.3.3.7 if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell reselection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following: 4> for each neighbour cell included, include the optional fields that are available |
| Summary of change | Modified the template to “anyoromit” for the IEs that may possibly be available for reporting by the UE Note : An associated prose CR on 38.523-1 will be raised at RAN5#94. |
| TTCN module | RRC_MDT_IntraNR_CEF_NR5GC |
| MCC160 Comment |  |

Before Change:

template (present) MeasResultListNR cr_MeasResultListNR_1Entry(template (present) PhysCellId p_PhysCellId) := //@sic R5-217924 sic@ { { physCellId := p_PhysCellId, measResult := { cellResults := { resultsSSB_Cell := { rsrp := ?, rsrq := omit, sinr := omit }, resultsCSI_RS_Cell := omit }, rsIndexResults := omit }, cgi_Info := omit } };

After Change:

template (present) MeasResultListNR cr_MeasResultListNR_1Entry(template (present) PhysCellId p_PhysCellId) := //@sic R5-217924 sic@ { { physCellId := p_PhysCellId, measResult := { cellResults := { resultsSSB_Cell := { rsrp := ?, rsrq := *, sinr := * }, resultsCSI_RS_Cell := * }, rsIndexResults := * }, cgi_Info := * } };

# Branches executed

The test case was executed on NR band n78 uing NR ciphering nea1 and integrity algorithm nia1

# Execution Log Files

## Qualcomm SM 8450, SDX65

The Qualcomm SM 8450, SDX65 UE passed this test case on Keysight S8704A Protocol Conformance Toolset solution. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_8_1_6_1_4_6_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_8_1_6_1_4_6_PIXIT.xml

## Mediatek MT6983

The Mediatek MT6983  UE passed this test case on R&S 5G Protocol Conformance Test Platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

R&S/ TC_8_1_6_1_4_6_NR5GC_MTK

# References

| [1] | R5s220008 : NR5GC FR1 : Supporting inforamtion for addition of Rel-16 SON-MDT test case 8.1.6.1.4.6 |
| --- | --- |