3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220478

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2454 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the function f_EUTRA38_MultiPDN_InitialRegistration_Step5_16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In 22wk12 Implementation , in the function f_EUTRA38_MultiPDN_InitialRegistration_Step5_16 , v_UERadioCapIdToSend is initialized with f_EUTRA38_MobileInfo_GetRACS_id without checking for pc_EPC_RACS. This can result in run time error due to vc_EUTRA_Multi5GS_Global.MobileInfo.UERadioCapId being Omit for the UEs which don’t support pc_EPC_RACS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | v_UERadioCapIdToSend is initialized with f_EUTRA38_MobileInfo_GetRACS_id() only if pc_EPC_RACS is supported |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All IRAT scenarios where LTE registration happens |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function f_EUTRA38_MultiPDN_InitialRegistration_Step5_16 ()	2

# Corrections required

## Correction to function f_EUTRA38_MultiPDN_InitialRegistration_Step5_16 ()

| Function name | f_EUTRA38_MultiPDN_InitialRegistration_Step5_16() |
| --- | --- |
| Reason for change | In 22wk12 Implementation , in the function f_EUTRA38_MultiPDN_InitialRegistration_Step5_16 , v_UERadioCapIdToSend is initialized with f_EUTRA38_MobileInfo_GetRACS_id without checking for pc_EPC_RACS. This can result in run time error due to vc_EUTRA_Multi5GS_Global.MobileInfo.UERadioCapId being Omit for the UEs which don’t support pc_EPC_RACS. |
| Summary of change | v_UERadioCapIdToSend is initialized with f_EUTRA38_MobileInfo_GetRACS_id() only if pc_EPC_RACS is supported |
| TTCN module | ttcn/develop/NR5GC_IRAT/Common/EUTRA38_NR5GC_InitialRegistration.ttcn |
| MCC160 Comment |  |

Before Change:

function f_EUTRA38_MultiPDN_InitialRegistration_Step5_16(EUTRA_CellId_Type p_CellId, NR_TESTMODE_STATE_Type p_State, NAS_MSG_Indication_Type p_NAS_Ind, template (omit) PLMN_List p_EquivalentPlmnList := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduSessionInfoList := omit, boolean p_FallbackSignalling := false) runs on EUTRA_Multi5GS_PTC { var SRB_COMMON_IND v_ReceivedAsp; var template (omit) AdditionalUpdateType v_AdditionalUpdateType; var NAS_UL_Message_Type v_PdnConnectivityRequest; var ProcedureTransactionIdentifier v_EPS_TI; var template (omit) ProtocolConfigOptions v_PcoFromUE := omit; // may be omit var template (omit) ExtdProtocolConfigOptions v_ExtdPcoFromUE := omit; var EPS_BearerIdentity v_EpsBearerId; var boolean v_IPv4AllocationViaNasFlag; var APNandPCOs_Type v_APNandPCOs; var NasCount_Type v_NasCountUL; var NR_Configuration_Type v_NR_ConfigType := f_EUTRA38_NR_Config_Get(); var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling := noDelay; // Default: no IP signalling possible var template (omit) UERadioCapId v_UERadioCapIdToSend := f_EUTRA38_MobileInfo_GetRACS_id(); var UERadioCapId v_UERadioCapIdRxd; if (not ispresent (p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap)){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueAddSecurityCap should be present"); } select (v_NR_ConfigType) { case (EN_DC) { if (not match (p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_EUTRA_UENetworkCap(omit))){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for ENDC"); } } case else { // other options FFS if (not match (p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap(omit))){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for EUTRA38"); } } } v_AdditionalUpdateType := p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.addUpdateType; v_PdnConnectivityRequest := p_NAS_Ind.Pdu.PiggybackedPduList[0].Msg; v_EPS_TI := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.procedureTransactionIdentifier; v_PcoFromUE := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.protocolConfigurationOptions; v_ExtdPcoFromUE := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.extdProtocolConfigurationOptions; << SKIPPED CODE >> }

After Change:

function f_EUTRA38_MultiPDN_InitialRegistration_Step5_16(EUTRA_CellId_Type p_CellId, NR_TESTMODE_STATE_Type p_State, NAS_MSG_Indication_Type p_NAS_Ind, template (omit) PLMN_List p_EquivalentPlmnList := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduSessionInfoList := omit, boolean p_FallbackSignalling := false) runs on EUTRA_Multi5GS_PTC { var SRB_COMMON_IND v_ReceivedAsp; var template (omit) AdditionalUpdateType v_AdditionalUpdateType; var NAS_UL_Message_Type v_PdnConnectivityRequest; var ProcedureTransactionIdentifier v_EPS_TI; var template (omit) ProtocolConfigOptions v_PcoFromUE := omit; // may be omit var template (omit) ExtdProtocolConfigOptions v_ExtdPcoFromUE := omit; var EPS_BearerIdentity v_EpsBearerId; var boolean v_IPv4AllocationViaNasFlag; var APNandPCOs_Type v_APNandPCOs; var NasCount_Type v_NasCountUL; var NR_Configuration_Type v_NR_ConfigType := f_EUTRA38_NR_Config_Get(); var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling := noDelay; // Default: no IP signalling possible var template (omit) UERadioCapId v_UERadioCapIdToSend := omit /*f_EUTRA38_MobileInfo_GetRACS_id()*/; var UERadioCapId v_UERadioCapIdRxd; if (not ispresent (p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap)){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueAddSecurityCap should be present"); } select (v_NR_ConfigType) { case (EN_DC) { if (not match (p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_EUTRA_UENetworkCap(omit))){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for ENDC"); } } case else { // other options FFS if (not match (p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap(omit))){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for EUTRA38"); } } } if (pc_EPC_RACS) { v_UERadioCapIdToSend := f_EUTRA38_MobileInfo_GetRACS_id(); } v_AdditionalUpdateType := p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.addUpdateType; v_PdnConnectivityRequest := p_NAS_Ind.Pdu.PiggybackedPduList[0].Msg; v_EPS_TI := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.procedureTransactionIdentifier; v_PcoFromUE := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.protocolConfigurationOptions; v_ExtdPcoFromUE := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.extdProtocolConfigurationOptions; << SKIPPED CODE >> }