3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220486

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2461 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR_RRC_CipherActTime_GetCurrent |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It has been observed that some of the TCs are calling this function while cell is in switched off state and as per TS 36.523-3 section 7.4.5 cases where cells being switched off need not to be mapped to physical resources. Therefore, SS is not expected to fill the list if that cell is in switched off state which in turn leads to runtime error for v_SecurityActTimeList as it remains uninitialized for such cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Initialized variable v_SecurityActTimeList := {} so that behavior of such test cases can be handled here with empty list. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Idle mode test cases. i.e., 6.4.2.1.NR5GC, 6.4.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to f_NR_RRC_CipherActTime_GetCurrent ()	2

# Corrections required

## Correction to f_NR_RRC_CipherActTime_GetCurrent ()

| Function name | f_NR_RRC_CipherActTime_GetCurrent () |
| --- | --- |
| Reason for change | It has been observed that some of the TCs are calling this function while cell is in switched off state and as per TS 36.523-3 section 7.4.5 cases where cells being switched off need not to be mapped to physical resources. Therefore, SS is not expected to fill the list if that cell is in switched off state which in turn leads to runtime error for v_SecurityActTimeList as it remains uninitialized for such cases. |
| Summary of change | Initialized variable v_SecurityActTimeList := {} so that behavior of such test cases can be handled here with empty list. |
| TTCN module | \Common\NR\NR_SecuritySteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_RRC_CipherActTime_GetCurrent(NR_CellId_Type p_NR_CellId, boolean p_IncrSRB1, boolean p_IncrSRB1_DL := false, template(omit) NR_PdcpCountInfoList_Type p_PdcpCountInfoList:= omit ) runs on NR_BASE_PTC return NR_SecurityActTimeList_Type { //@sic R5s220123: added p_PdcpCountInfoList sic@ var NR_PdcpCountInfoList_Type v_PdcpCountInfoList; var NR_PdcpCountInfo_Type v_PdcpCountInfo; var NR_PDCP_ActTime_Type v_PDCP_ActTimeUL; var NR_PDCP_ActTime_Type v_PDCP_ActTimeDL; var NR_RadioBearerId_Type v_RadioBearerId; var NR_SecurityActTime_Type v_SecurityActTime; var integer I; var NR_SecurityActTimeList_Type v_SecurityActTimeList; if (isvalue(p_PdcpCountInfoList)) { v_PdcpCountInfoList := valueof(p_PdcpCountInfoList); } else { v_PdcpCountInfoList := f_NR_SS_PdcpCount_Get(p_NR_CellId); } for (I:=0; I < lengthof(v_PdcpCountInfoList); I:=I+1) { v_PdcpCountInfo := v_PdcpCountInfoList[I]; if ((not ispresent(v_PdcpCountInfo.UL)) or (not ispresent(v_PdcpCountInfo.DL))) { FatalError(__FILE__, __LINE__, "missing PdcpCountInfo for UL or DL"); } v_RadioBearerId := v_PdcpCountInfo.RadioBearerId; // Convert PdcpCount to integer value (UL an DL) v_PDCP_ActTimeUL.SQN := f_NR_SQN_ConvertFromPdcpCount(v_PdcpCountInfo.UL); v_PDCP_ActTimeDL.SQN := f_NR_SQN_ConvertFromPdcpCount(v_PdcpCountInfo.DL); // increment SQN for SRB1 (UL and DL) if (p_IncrSRB1 and (ischosen(v_RadioBearerId.Srb) and (v_RadioBearerId.Srb == tsc_SRB1))) { // @sic R5s190214 sic@ v_PDCP_ActTimeUL.SQN := f_NR_SQN_Increment(v_PDCP_ActTimeUL.SQN); v_PDCP_ActTimeDL.SQN := f_NR_SQN_Increment(v_PDCP_ActTimeDL.SQN); } // When transmitting RRCReestablishment to UE on DL SRB1 ciphering is not applied // When receiving RRCReestablishemntComplete on UL SRB1 ciphering is applied by UE. // So the new ciphering keys needs to be applied at SN+1 in DL and at SN in UL => increment DL SN only // @sic R5s201221 sic@ if ((p_IncrSRB1_DL) and (ischosen(v_RadioBearerId.Srb) and (v_RadioBearerId.Srb == tsc_SRB1))) { v_PDCP_ActTimeDL.SQN := f_NR_SQN_Increment(v_PDCP_ActTimeDL.SQN); } // set activation time v_SecurityActTime.RadioBearerId := v_RadioBearerId; v_SecurityActTime.UL := v_PDCP_ActTimeUL; v_SecurityActTime.DL := v_PDCP_ActTimeDL; v_SecurityActTimeList[I] := v_SecurityActTime; } return v_SecurityActTimeList; }

After Change:

function f_NR_RRC_CipherActTime_GetCurrent(NR_CellId_Type p_NR_CellId, boolean p_IncrSRB1, boolean p_IncrSRB1_DL := false, template(omit) NR_PdcpCountInfoList_Type p_PdcpCountInfoList:= omit ) runs on NR_BASE_PTC return NR_SecurityActTimeList_Type { //@sic R5s220123: added p_PdcpCountInfoList sic@ var NR_PdcpCountInfoList_Type v_PdcpCountInfoList; var NR_PdcpCountInfo_Type v_PdcpCountInfo; var NR_PDCP_ActTime_Type v_PDCP_ActTimeUL; var NR_PDCP_ActTime_Type v_PDCP_ActTimeDL; var NR_RadioBearerId_Type v_RadioBearerId; var NR_SecurityActTime_Type v_SecurityActTime; var integer I; var NR_SecurityActTimeList_Type v_SecurityActTimeList := {}; // WA#WI=950222 if (isvalue(p_PdcpCountInfoList)) { v_PdcpCountInfoList := valueof(p_PdcpCountInfoList); } else { v_PdcpCountInfoList := f_NR_SS_PdcpCount_Get(p_NR_CellId); } for (I:=0; I < lengthof(v_PdcpCountInfoList); I:=I+1) { v_PdcpCountInfo := v_PdcpCountInfoList[I]; if ((not ispresent(v_PdcpCountInfo.UL)) or (not ispresent(v_PdcpCountInfo.DL))) { FatalError(__FILE__, __LINE__, "missing PdcpCountInfo for UL or DL"); } v_RadioBearerId := v_PdcpCountInfo.RadioBearerId; // Convert PdcpCount to integer value (UL an DL) v_PDCP_ActTimeUL.SQN := f_NR_SQN_ConvertFromPdcpCount(v_PdcpCountInfo.UL); v_PDCP_ActTimeDL.SQN := f_NR_SQN_ConvertFromPdcpCount(v_PdcpCountInfo.DL); // increment SQN for SRB1 (UL and DL) if (p_IncrSRB1 and (ischosen(v_RadioBearerId.Srb) and (v_RadioBearerId.Srb == tsc_SRB1))) { // @sic R5s190214 sic@ v_PDCP_ActTimeUL.SQN := f_NR_SQN_Increment(v_PDCP_ActTimeUL.SQN); v_PDCP_ActTimeDL.SQN := f_NR_SQN_Increment(v_PDCP_ActTimeDL.SQN); } // When transmitting RRCReestablishment to UE on DL SRB1 ciphering is not applied // When receiving RRCReestablishemntComplete on UL SRB1 ciphering is applied by UE. // So the new ciphering keys needs to be applied at SN+1 in DL and at SN in UL => increment DL SN only // @sic R5s201221 sic@ if ((p_IncrSRB1_DL) and (ischosen(v_RadioBearerId.Srb) and (v_RadioBearerId.Srb == tsc_SRB1))) { v_PDCP_ActTimeDL.SQN := f_NR_SQN_Increment(v_PDCP_ActTimeDL.SQN); } // set activation time v_SecurityActTime.RadioBearerId := v_RadioBearerId; v_SecurityActTime.UL := v_PDCP_ActTimeUL; v_SecurityActTime.DL := v_PDCP_ActTimeDL; v_SecurityActTimeList[I] := v_SecurityActTime; } return v_SecurityActTimeList; }