3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220534

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2480 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR PDCP test case 7.1.3.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test steps 3 and 3E are duplicated |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed step 3E |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE might fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.2.ENDC, 7.1.3.5.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	4

2	Corrections required	4

2.1 Correction to function f_TC_7_1_3_5_2_NR_CheckULPrimaryPath	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk12 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Corrections required

## 2.1 Correction to function f_TC_7_1_3_5_2_NR_CheckULPrimaryPath

| Function name | f_TC_7_1_3_5_2_NR_CheckULPrimaryPath |
| --- | --- |
| Reason for change | Test Step duplicated |
| Summary of change | Removed test step 3E and renamed 3A-3D to 2A-2D |
| TTCN module | PDCP_TC_Common_NR.ttcn |

Before

function f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(NR_PDCP_SS_State_Type p_NR_PDCP_State,

template (value) RlcBearerRouting_Type p_SCG_RoutingRlcRouting,

template (value) RlcBearerRouting_Type p_PrimaryPath,

template (value) RlcBearerRouting_Type p_SecondaryPath,

charstring p_TestStep)  runs on NR_BASE_PTC

{// @sic R5-220641 sic@

var NR_PDCP_SDUList_Type v_SduList;

var template (present) NR_PDCP_PDU_Type v_ExpectedPdu;

timer t_WaitForPdu:= 2.0;

v_SduList := f_GenerateSDU (p_NR_PDCP_State, {64, 164});

//@siclog "Step 1 or 3C" siclog@

//The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell Data PDU = 64 bytes.

f_NR_PDCP_Send_SCGCell(p_NR_PDCP_State,nr_Cell1,v_SduList[0], p_SCG_RoutingRlcRouting);

v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT, v_SduList[0]);

t_WaitForPdu.start;

alt{

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_PrimaryPath, -, {v_ExpectedPdu}))

{  //@siclog "Step 2 or 3D" siclog@ Does UE transmit a PDCP Data PDU on the AM RLC entity configured for SCG on NR Cell 1 (PSCell)?

f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestStep);

p_NR_PDCP_State.RX_NEXT := p_NR_PDCP_State.RX_NEXT + 1;

t_WaitForPdu.stop;

}

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_SecondaryPath, -, {v_ExpectedPdu}))

{ //Does UE transmit a PDCP Data PDU on the AM RLC secondary entity in next two seconds?

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestStep);

}

[] t_WaitForPdu.timeout{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestStep);

}

}

if(p_TestStep =="3D")//@siclog "Step 3E" siclog@

{ //The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell.Data PDU = 164 bytes.

f_NR_PDCP_Send_SCGCell(p_NR_PDCP_State,nr_Cell1,v_SduList[1], p_SCG_RoutingRlcRouting);

}

} }

After

function f_TC_7_1_3_5_2_NR_CheckULPrimaryPath(NR_PDCP_SS_State_Type p_NR_PDCP_State,

template (value) RlcBearerRouting_Type p_SCG_RoutingRlcRouting,

template (value) RlcBearerRouting_Type p_PrimaryPath,

template (value) RlcBearerRouting_Type p_SecondaryPath,

charstring p_TestStep)  runs on NR_BASE_PTC

{// @sic R5-220641 sic@

var NR_PDCP_SDUList_Type v_SduList;

var template (present) NR_PDCP_PDU_Type v_ExpectedPdu;

timer t_WaitForPdu:= 2.0;

v_SduList := f_GenerateSDU (p_NR_PDCP_State, {64, 164});

//@siclog "Step 1 or 3C" siclog@

//The SS sends a PDCP Data PDU on the split DRB on AM RLC entity configured for SCG on PSCell Data PDU = 64 bytes.

f_NR_PDCP_Send_SCGCell(p_NR_PDCP_State,nr_Cell1,v_SduList[0], p_SCG_RoutingRlcRouting);

v_ExpectedPdu := f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT, v_SduList[0]);

t_WaitForPdu.start;

alt{

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_PrimaryPath, -, {v_ExpectedPdu}))

{  //@siclog "Step 2 or 2D" siclog@ Does UE transmit a PDCP Data PDU on the AM RLC entity configured for SCG on NR Cell 1 (PSCell)?

f_NR_PreliminaryPass(__FILE__, __LINE__, p_TestStep);

p_NR_PDCP_State.RX_NEXT := p_NR_PDCP_State.RX_NEXT + 1;

t_WaitForPdu.stop;

}

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id , p_SecondaryPath, -, {v_ExpectedPdu}))

{ //Does UE transmit a PDCP Data PDU on the AM RLC secondary entity in next two seconds?

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestStep);

}

[] t_WaitForPdu.timeout{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestStep);

}

}

//Test step removed