3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220552

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2491 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC PS data off test case 11.6.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - When the UE is activated with PS Data Off, it may initiate the IMS SIP re-REGISTER in parallel to the PDU SESSION MODIFICATION REQUEST. Currently, these are expected to occur in sequence. - When performing re-registration following PS data off, the UE may not include feature tag for support of video callin (even if PICS settings indicate this is normally supported). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Move coordination signalling to later point to allow IMS and MR5GC procedures to run in parallel Note : An associated prose CR on 38.523-1 will be raised at RAN5#95 - Pass parameters to checking functions indicating IMS vidoe not supported (to override checking based on PICS) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.6.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

2.2 Change 2	4

2.3 Change 3	6

2.4 Change 4	7

2.5 Change 5	8

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_11_6_2_Body() |
| --- | --- |
| Reason for change | When the UE is activated with PS Data Off, it may initiate the IMS SIP re-REGISTER in parallel to the PDU SESSION MODIFICATION REQUEST. Currently, these are expected to occur in sequence. |
| Summary of change | Remove the sending of trigger message to IMS to expect re-registration.Instead send the trigger later after the MO video call is initiated, to start the timer at IMS side. Note : An associated prose CR on 38.523-1 will be raised at RAN5#95 |
| TTCN module | PS_Data_OFF_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_11_6_2_Body() runs on NR5GC_PTC

{

…

…

…

//@siclog "Steps 4" siclog@

//receive PDU Session Modification Complete

SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToModify)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2");

}

//Now expect the re-register on IMS PTC

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]);

//@siclog "Steps 7" siclog@

//Receive the coordination to trigger a Video call

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]);

//Make the UE attempt IMS voice call.

f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri);

//Postamble after receiving coordination from IMS PTC

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]);

} //End of test body of TC_11_6_2

After Change:

function fl_TC_11_6_2_Body() runs on NR5GC_PTC

{

…

…

…

//@siclog "Steps 4" siclog@

//receive PDU Session Modification Complete

SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToModify)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2");

}

//Now expect the re-register on IMS PTC

//MOVED f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]);

//@siclog "Steps 7" siclog@

//Receive the coordination to trigger a Video call

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]);

//Make the UE attempt IMS voice call.

f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri);

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]);

//Postamble after receiving coordination from IMS PTC

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]);

} //End of test body of TC_11_6_2

## 2.2 Change 2

| Function name | f_TC_11_6_2_NR5GC_IMS1() |
| --- | --- |
| Reason for change | When the UE is activated with PS Data Off, it may initiate the IMS SIP re-REGISTER in parallel to the PDU SESSION MODIFICATION REQUEST. Currently, these are expected to occur in sequence. |
| Summary of change | Remove the wait for trigger message from NR5GC to expect re-registration.Instead wait for the trigger later, to start the t_wait timer |
| TTCN module | PS_Data_OFF_NR5GC_IMS.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_11_6_2_NR5GC_IMS1()runs on IMS_PTC

{

timer t_Wait := 30.0;

f_IMS_PTC_Init(PDN_1);

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

f_IMS_Registration();

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

//Now ready for the Re-REGISTER

//@siclog "Steps 5-6" siclog@

f_IMS_IPCAN_WaitForTrigger(IPCAN);

fl_IMS_ReRegistration_DataOff();

//Now trigger the Video call

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

//@siclog "Step 8" siclog@

t_Wait.start;

alt {

[] IMS_Server.receive(car_IMS_Invite_Request(cr_INVITE_Request(?, ?)))

{

f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8");

}

[]t_Wait.timeout{}

}

//Ready for postamble

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

f_IMS_IpcanReleaseWithOptionalDeregistration();

}

After Change:

function f_TC_11_6_2_NR5GC_IMS1()runs on IMS_PTC

{

timer t_Wait := 30.0;

f_IMS_PTC_Init(PDN_1);

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

f_IMS_Registration();

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

//Now ready for the Re-REGISTER

//@siclog "Steps 5-6" siclog@

//MOVED f_IMS_IPCAN_WaitForTrigger(IPCAN);

fl_IMS_ReRegistration_DataOff();

//Now trigger the Video call

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

f_IMS_IPCAN_WaitForTrigger(IPCAN);

//@siclog "Step 8" siclog@

t_Wait.start;

alt {

[] IMS_Server.receive(car_IMS_Invite_Request(cr_INVITE_Request(?, ?)))

{

f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8");

}

[]t_Wait.timeout{}

}

//Ready for postamble

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

f_IMS_IpcanReleaseWithOptionalDeregistration();

}

## 2.3 Change 3

| Function name | fl_IMS_ReRegistration_DataOff() |
| --- | --- |
| Reason for change | When performing re-registration following PS data off, the UE may not include feature tag for support of video callin (even if PICS settings indicate this is normally supported). |
| Summary of change | Pass false into f_IMS_REGISTER_SubsequentRequest() to indicate IMS video not supported (see change 4) |
| TTCN module | PS_Data_OFF_NR5GC_IMS.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_IMS_ReRegistration_DataOff(integer p_NonceCount := 2) runs on IMS_PTC //anupams@11.6.2

{

var REGISTER_Request v_RegisterReq;

var IMS_DATA_REQ v_IMS_DATA_REQ;

var template (present) SemicolonParam_List v_MandatoryContactParams := { cr_GenericParam("+g.3gpp.ps-data-off", "active") };

v_IMS_DATA_REQ := f_IMS_REGISTER_SubsequentRequest(-, v_MandatoryContactParams, -, p_NonceCount);

v_RegisterReq := v_IMS_DATA_REQ.Request.Register;

IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq))));

}

//===

After Change:

function fl_IMS_ReRegistration_DataOff(integer p_NonceCount := 2) runs on IMS_PTC

{

var REGISTER_Request v_RegisterReq;

var IMS_DATA_REQ v_IMS_DATA_REQ;

var template (present) SemicolonParam_List v_MandatoryContactParams := { cr_GenericParam("+g.3gpp.ps-data-off", "active") };

v_IMS_DATA_REQ := f_IMS_REGISTER_SubsequentRequest(-, v_MandatoryContactParams, -, p_NonceCount, -, -, false);

v_RegisterReq := v_IMS_DATA_REQ.Request.Register;

IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq))));

}

//===

## 2.4 Change 4

| Function name | f_IMS_REGISTER_SubsequentRequest() |
| --- | --- |
| Reason for change | When performing re-registration following PS data off, the UE may not include feature tag for support of video calling (even if PICS settings indicate this is normally supported). |
| Summary of change | Pass new parameter f_IMS_Register_MessageHeader_CommonChecks () to indicate IMS video supported/not supported (see change 5) |
| TTCN module | IMS_Procedures_RegistrationCommon.ttcn |
| MCC160 Comment |  |

Before Change:

function f_IMS_REGISTER_SubsequentRequest(template (omit) IMS_DATA_REQ p_IMS_DATA_REQ := omit,

template (present) SemicolonParam_List p_MandatoryContactParams := {},

boolean p_CondA7 := false,

integer p_NonceCount := 1,

template (omit) charstring p_ExplicitExpiry := omit,

boolean p_IMS_SMSoverIMS := pc_SMS_IP_MT) runs on IMS_PTC  return IMS_DATA_REQ

{ /* p_CondA7  .. condition A7 acc. to 34.229-1 Annex A.1.1 (Initial IMS emergency registration)

…

…

…

f_IMS_PTC_ImsInfo_SetContactUrl(f_MessageHeader_GetContactSipUrl(v_MessageHeader));

f_IMS_Register_MessageHeader_CommonChecks(v_MessageHeader, v_Protocol, p_CondA7, v_UE_Address, p_ExplicitExpiry, -, p_IMS_SMSoverIMS); // @sic R5s120945 sic@  @sic R5-201458 sic@ */

if (f_IMS_PTC_ImsInfo_SigCompStarted()) {

f_IMS_MessageHeader_CheckSigComp(v_MessageHeader);

}

return v_IMS_DATA_REQ;

}

After Change:

function f_IMS_REGISTER_SubsequentRequest(template (omit) IMS_DATA_REQ p_IMS_DATA_REQ := omit,

template (present) SemicolonParam_List p_MandatoryContactParams := {},

boolean p_CondA7 := false,

integer p_NonceCount := 1,

template (omit) charstring p_ExplicitExpiry := omit,

boolean p_IMS_SMSoverIMS := pc_SMS_IP_MT,

boolean p_VideoFeatureTag := pc_IMS_Video_FeatureTag) runs on IMS_PTC  return IMS_DATA_REQ //anupams@11.6.2

{ /* p_CondA7  .. condition A7 acc. to 34.229-1 Annex A.1.1 (Initial IMS emergency registration)

…

…

…

f_IMS_PTC_ImsInfo_SetContactUrl(f_MessageHeader_GetContactSipUrl(v_MessageHeader));

f_IMS_Register_MessageHeader_CommonChecks(v_MessageHeader, v_Protocol, p_CondA7, v_UE_Address, p_ExplicitExpiry, -, p_IMS_SMSoverIMS, p_VideoFeatureTag); // @sic R5s120945 sic@  @sic R5-201458 sic@ */ //anupams@11.6.2

if (f_IMS_PTC_ImsInfo_SigCompStarted()) {

f_IMS_MessageHeader_CheckSigComp(v_MessageHeader);

}

return v_IMS_DATA_REQ;

}

## 2.5 Change 5

| Function name | f_IMS_Register_MessageHeader_CommonChecks() |
| --- | --- |
| Reason for change | When performing re-registration following PS data off, the UE may not include feature tag for support of video calling (even if PICS settings indicate this is normally supported). |
| Summary of change | Add new parameter and pass into fl_IMS_Register_ContactHeader_CheckParams to control IMS video support checking |
| TTCN module | IMS_Procedures_RegistrationCommon.ttcn |
| MCC160 Comment |  |

Before Change:

function f_IMS_Register_MessageHeader_CommonChecks(MessageHeader p_MessageHeader,

InternetProtocol_Type p_Protocol,

boolean p_CondA7 := false,

template (omit) IP_AddrInfo_Type p_UE_Address := omit,

template (omit) charstring p_ExplicitExpiry := omit,

boolean p_CheckAuthorization := true,

boolean p_IMS_SMSoverIMS := pc_SMS_IP_MT) runs on IMS_PTC //anupams@11.6.2

{ /* @sic R5-201458 (34.229-5): p_IMS_SMSoverIMS sic@ */

…

…

…

if (not v_IsMCX) {                                                          /* @sic R5s200000: MCX parameters are directly checked by setting of p_MandatoryContactParams for Contact template sic@ */

if (pc_IMS_GRUUsInSIP) {                                                    // check 'c-p-instance'

// @sic R5-153766: Check of instance-id shall be done for GRUU too sic@

if (not match(v_ContactAddress.contactParams, cr_CPInstanceInContactParams)) {

f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid instance id in Contact header");

}

}

if (not v_IsDeregistration and not f_GetTestcaseAttrib_IMS_UTRAN (testcasename())) {    /* @sic R5-142956: in case of deregistration no further contact parameters need to be checked, R5-169130 or if UTRAN_IMS Testsuite sic@ */

fl_IMS_Register_ContactHeader_CheckParams(v_ContactAddress, p_CondA7, -, p_IMS_SMSoverIMS);

}

}

if (isvalue(p_UE_Address)) {

fl_IMS_Register_ContactAddress_CheckHostname(v_ContactAddress, valueof(p_UE_Address));

}

}

// Check 'expires'

f_IMS_MessageHeader_CheckExpires(p_MessageHeader, p_ExplicitExpiry);           /* @sic R5-196983: check is common for MCX and IMS sic@ */

}

After Change:

function f_IMS_Register_MessageHeader_CommonChecks(MessageHeader p_MessageHeader,

InternetProtocol_Type p_Protocol,

boolean p_CondA7 := false,

template (omit) IP_AddrInfo_Type p_UE_Address := omit,

template (omit) charstring p_ExplicitExpiry := omit,

boolean p_CheckAuthorization := true,

boolean p_IMS_SMSoverIMS := pc_SMS_IP_MT,

boolean p_VideoFeatureTag := pc_IMS_Video_FeatureTag) runs on IMS_PTC //anupams@11.6.2

{ /* @sic R5-201458 (34.229-5): p_IMS_SMSoverIMS sic@ */

…

…

…

if (not v_IsMCX) {                                                          /* @sic R5s200000: MCX parameters are directly checked by setting of p_MandatoryContactParams for Contact template sic@ */

if (pc_IMS_GRUUsInSIP) {                                                    // check 'c-p-instance'

// @sic R5-153766: Check of instance-id shall be done for GRUU too sic@

if (not match(v_ContactAddress.contactParams, cr_CPInstanceInContactParams)) {

f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid instance id in Contact header");

}

}

if (not v_IsDeregistration and not f_GetTestcaseAttrib_IMS_UTRAN (testcasename())) {    /* @sic R5-142956: in case of deregistration no further contact parameters need to be checked, R5-169130 or if UTRAN_IMS Testsuite sic@ */

fl_IMS_Register_ContactHeader_CheckParams(v_ContactAddress, p_CondA7, -, p_IMS_SMSoverIMS, p_VideoFeatureTag);

}

}

if (isvalue(p_UE_Address)) {

fl_IMS_Register_ContactAddress_CheckHostname(v_ContactAddress, valueof(p_UE_Address));

}

}

// Check 'expires'

f_IMS_MessageHeader_CheckExpires(p_MessageHeader, p_ExplicitExpiry);           /* @sic R5-196983: check is common for MCX and IMS sic@ */

}