3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220553

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2492 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC NRDC test case 8.2.3.14.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test , 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TTCN Implementation for test case 8.2.3.14.2, function f_NRDC_RRC_PSCellChange_InterCell_SplitMCGReestablish is called at step 2-3. This function in turn calls f_NR_SS_PdcpCount_Handover() to reset the PDCP SN of different RBs. However, in test case 8.2.3.14.2, it is only required to Reset PDCP SN of DRB-3 (SCG DRB) before the PDCP Handover. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A New function is defined f_NR_SS_Reset_PdcpCount_DRB_Handover() which will only reset PDCP SN of DRB-3 (SCG DRB). Function f_NRDC_RRC_PSCellChange_InterCell_SplitMCGReestablish() is updated to call newly defined function f_NR_SS_Reset_PdcpCount_DRB_Handover() replacing the old function call f_NR_SS_PdcpCount_Handover() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not execute correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.14.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NRDC_RRC_PSCellChange_InterCell_SplitMCGReestablish () |
| --- | --- |
| Reason for change | In TTCN Implementation for test case 8.2.3.14.2, function f_NRDC_RRC_PSCellChange_InterCell_SplitMCGReestablish is called at step 2-3. This function in turn calls f_NR_SS_PdcpCount_Handover() to reset the PDCP SN of different RBs. However, in test case 8.2.3.14.2, it is only required to Reset PDCP SN of DRB-3 (SCG DRB) before the PDCP Handover. |
| Summary of change | A New function is defined f_NR_SS_Reset_PdcpCount_DRB_Handover() which will only reset PDCP SN of DRB-3 (SCG DRB). Function f_NRDC_RRC_PSCellChange_InterCell_SplitMCGReestablish() is updated to call newly defined function f_NR_SS_Reset_PdcpCount_DRB_Handover() replacing the old function call f_NR_SS_PdcpCount_Handover() |
| TTCN module | NR5GC_RRCSteps_NRDC.ttcn |
| MCC160 Comment |  |

Before Change

function f_NRDC_RRC_PSCellChange_InterCell_SplitMCGReestablish( NR_CellId_Type p_MCG_PCellId, NR_CellId_Type p_Source_PSCellId, NR_CellId_Type p_Target_PSCellId, boolean p_RBConfig_KeyChange := true, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, template(omit) SK_Counter p_SK_Counter := omit ) runs on NR5GC_PTC { … //1. NR Target PSCell: Configuration of SRB3 (if necessary) and DRBs f_NR_SS_CommonRadioBearerConfig(p_Target_PSCellId, v_SS_TargetSCG_RadioBearerList); // 2. Transfer of the PDCP Count for DRBs and SRB3 (if necessary) from NR source to NR target PSCell: //a) NR Source PSCell: Get PDCP COUNT. //b) NR Target PSCell: Set PDCP COUNT. //NOTE 1: No further sending/receiving of DRB data before the PSCell change is done. //NOTE 2: For AM DRBs the PDCP count is maintained. For SRB3 (if applied) and UM DRBs, the PDCP count is maintained or reset depending on the RRCReconfiguration message content. f_NR_SS_PdcpCount_Handover(p_Source_PSCellId, p_Target_PSCellId, v_SS_TargetSCG_RadioBearerList, p_RBConfig_KeyChange); //3. NR Target PSCell: Inform the SS about the PSCell change and about the source cell id. f_NR_SS_PdcpHandoverCtrl(p_Target_PSCellId, cas_NR_PdcpHandoverInit_REQ(p_Source_PSCellId, p_Target_PSCellId)); //4. NR Target PSCell: Configure RACH procedure either dedicated or C-RNTI based. //NOTE 3: The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI. f_NR_CellInfo_SetRNTI(p_Target_PSCellId, p_RNTI_Value); f_NR_SS_C_RNTI_Config(p_Target_PSCellId, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag); f_NR_SS_RachProcedureConfigHO(p_Target_PSCellId, v_RachProcedureConfig); … }

After Change

function f_NRDC_RRC_PSCellChange_InterCell_SplitMCGReestablish( NR_CellId_Type p_MCG_PCellId, NR_CellId_Type p_Source_PSCellId, NR_CellId_Type p_Target_PSCellId, boolean p_RBConfig_KeyChange := true, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value3, template(omit) SK_Counter p_SK_Counter := omit ) runs on NR5GC_PTC { … //1. NR Target PSCell: Configuration of SRB3 (if necessary) and DRBs f_NR_SS_CommonRadioBearerConfig(p_Target_PSCellId, v_SS_TargetSCG_RadioBearerList); // 2. Transfer of the PDCP Count for DRBs and SRB3 (if necessary) from NR source to NR target PSCell: //a) NR Source PSCell: Get PDCP COUNT. //b) NR Target PSCell: Set PDCP COUNT. //NOTE 1: No further sending/receiving of DRB data before the PSCell change is done. //NOTE 2: For AM DRBs the PDCP count is maintained. For SRB3 (if applied) and UM DRBs, the PDCP count is maintained or reset depending on the RRCReconfiguration message content. f_NR_SS_Reset_PdcpCount_DRB_Handover(p_Source_PSCellId, p_Target_PSCellId, v_SS_TargetSCG_RadioBearerList); //New Function Call //3. NR Target PSCell: Inform the SS about the PSCell change and about the source cell id. f_NR_SS_PdcpHandoverCtrl(p_Target_PSCellId, cas_NR_PdcpHandoverInit_REQ(p_Source_PSCellId, p_Target_PSCellId)); //4. NR Target PSCell: Configure RACH procedure either dedicated or C-RNTI based. //NOTE 3: The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI. f_NR_CellInfo_SetRNTI(p_Target_PSCellId, p_RNTI_Value); f_NR_SS_C_RNTI_Config(p_Target_PSCellId, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag); f_NR_SS_RachProcedureConfigHO(p_Target_PSCellId, v_RachProcedureConfig); … }

## Change 2

| Function name | (New) f_NR_SS_Reset_PdcpCount_DRB_Handover () |
| --- | --- |
| Reason for change | See change 1 |
| Summary of change | A New function is defined f_NR_SS_Reset_PdcpCount_DRB_Handover() which will only reset PDCP SN of DRB-3 (SCG DRB). For change 1 |
| TTCN module | NR5GC_RRCSteps_NRDC.ttcn |
| MCC160 Comment |  |

New function

function f_NR_SS_Reset_PdcpCount_DRB_Handover(NR_CellId_Type p_SourceCellId, NR_CellId_Type p_TargetCellId, template( value) NR_RadioBearerList_Type p_RadioBearerList, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now ) runs on NR_BASE_PTC { var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList:=f_NR_SS_PdcpCount_Get(p_SourceCellId); var NR_PdcpCountInfoList_Type v_NR_PdcpCountInfoList_DRB; var integer i; var integer k := 0; for (i := 0; i < lengthof(v_NR_PdcpCountInfoList); i:= i + 1) { if ((fl_CheckAM_Drb(v_NR_PdcpCountInfoList[i], p_RadioBearerList ))) { v_NR_PdcpCountInfoList[i].DL.Value := int2bit(0,32); v_NR_PdcpCountInfoList[i].UL.Value := int2bit(0,32); v_NR_PdcpCountInfoList_DRB[k] := v_NR_PdcpCountInfoList[i]; k := k+1; } } f_NR_SS_PdcpCount_Set(p_TargetCellId, v_NR_PdcpCountInfoList_DRB, p_TimingInfo); }