3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220558

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2494 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR MAC test cases 7.1.1.4.2.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In 7.1.1.4.2.x case with 64QAM in UL (7.1.1.4.2.5), UL TB would be bigger than DL TB in some MCS/RB combinations. For MCS/RB (eg MCS=27 and NPRB = 270), DL PDUs(generated based on UL TB size) cannot scheduled in the same DL slot. Therefore, SS have to split PDUs in DL into two slots. If maxMCS=28 is configured in the DL rather than v_Max_Imcs, then, DL PDUs(generated based on UL TB Size) can be transmitted in the same DL slot. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To avoid such mismtach in DL and UL TBS size, configure DL maxMCS=28 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.2.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_4_2_x_DCI0_0_NR_TestBody |
| --- | --- |
| Reason for change | To avoid early detection issue for an active PLMN when UE is switched to Manual PLMN Selection Mode. |
| Summary of change | In 7.1.1.4.2.x case with 64QAM in UL (7.1.1.4.2.5), UL TB would be bigger than DL TB in some MCS/RB combinations. For MCS/RB (eg MCS=27 and NPRB = 270), DL PDUs(generated based on UL TB size) cannot scheduled in the same DL slot. Therefore, SS have to split PDUs in DL into two slots. If maxMCS=28 is configured in the DL rather than v_Max_Imcs, then, DL PDUs(generated based on UL TB Size) can be transmitted in the same DL slot. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_4_2_x_DCI0_0_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters, integer p_Max_Imcs := 28, //@sic R5s201144sic@ boolean p_TransformPrecoder := false) runs on NR_BASE_PTC { /* L2/MAC/7.1 * 7.1.1.4.2.x: UL-SCH Transport Block Size selection */ var integer v_TBSize:=0; // transport block size in bits var integer v_I_MCS:=0; // Modulation and Coding Scehme Index var integer v_N_PRB:=1; // Number of physical resource Blocks var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); // to store the max allowed PRB resources that can be allocated var integer v_TDRA ; // Time Domain resource allocation var NR_PDCP_SDUList_Type v_NR_PDCP_SDUList := {''O}; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var IntegerList_Type v_NoOfSymbols := {4,14}; var integer v_Max_Imcs :=p_Max_Imcs;//@sic R5s201144sic@ var SubFrameNumber_Type v_SF := 2; //@sic R5s210884 sic@ var default v_IgnoreNRSR:= null; //var integer v_NumSR_Rcvd :=0; //@sic R5s220320 sic@ timer t_Watchdog := 12.0; if (v_SCS ==kHz30) {v_SF:= 1;} // Configure SS for DL grant for Max TB size f_NR_DLGrantConfiguration(nr_Cell1, cs_TimingInfo_SubFrame(v_SF),//@sic R5s210884 sic@ resourceAllocationType1, cs_NR_FreqDomainResourceAssignmentDL_Explicit(0, v_Max_nPRB), int2bit(1, 4), //@sic R5s211325 sic@ cs_NR_TransportBlockRetransmissionList_Def(v_Max_Imcs), -, -, cs_NR_DciFormat_1_0_Params(v_SCS)); /* @sic R5s190442: v_SCS sic@ */ //Steps 1 to 5 are repeated for values of nPRB from 1 to v_Max_nPRB and iMCS from 0 to 28, time domain resource allocation v_TDRA 0..1 //f_NR_ConfigureULGrant_MaxTBSupported(); for (v_N_PRB := 1; v_N_PRB <= v_Max_nPRB; v_N_PRB := v_N_PRB + 1) { //for nPRB 1 to v_Max_nPRB if (p_TransformPrecoder) { if (f_NR_Check_This_N_PRB(v_N_PRB) == false) { // nPRB check in EXCEPTION Step continue; // nprb not allowed } } for (v_I_MCS := 0; v_I_MCS <= v_Max_Imcs ; v_I_MCS := v_I_MCS + 1) { //for iMCS 0 to 28 //@sic R5s190827r1 sic@ for (v_TDRA := 0; v_TDRA <= 1 ; v_TDRA := v_TDRA + 1) { //for Time Domain Resource Allocation 0 to 1 //@sic R5s190827 sic@ /* if(v_TDRA == 0) { v_NoOfSymbols := 4; } else{ v_NoOfSymbols := 14 } */ //SS looks up iTBS in table 7.1.7.1-1 in TS 36.213 based on the value of iMCS //v_I_TBS := f_NR_Get_iTBS_For_iMCS(v_I_MCS); //The SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L, and nPRB. v_TBSize := f_NR_TBS_Calculation(p_CalculationParameters, v_NoOfSymbols[v_TDRA], v_N_PRB, v_I_MCS); //@sic R5-198884 R5s210002 R5s210884 sic@ if ((v_I_MCS >= 27 and v_N_PRB <= 5) or (v_I_MCS == 28 and v_N_PRB == 17)) { //For IMCS= Max IMCS and < 4, the resulting TBS is very small leading to CRC errors in decoding UL data continue; } // skip this nprb, Imcs, TDRA combination. if (v_TBSize < 136) { //this results in zero number of PDCP SDUs continue; } // skip this nprb, Imcs, TDRA combination. // Steps 2 to 5 are performed if TBsize is More than minimum //@siclog "Step 2" siclog@ // SS creates one or more PDCP SDUs, depending on TBsize, in accordance with Table 7.1.1.4.1.1.3.2-2 //Configure SS for DCI Format x with RA type y and a Resource block assignment (RBA) correspondent //to nPRB as specified in 7.1.6.1 in TS 36.213 and Modulation and coding scheme iMCS //@siclog "Step 3" siclog@ // SS sends MAC PDU (NxPDCP SDUs) v_NR_PDCP_SDUList := fl_NR_GetPdcpSduList_1TB(v_TBSize); //calculates v_N, v_PDCPsize and returns f_NR_Build_N_PDCP_SDUList f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@sic R5s211325 sic@ v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); //@sic R5s210884 sic@ //v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); //300 ms in future DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, //crs_RlcBearerRouting_NR(nr_Cell1), cs_TimingInfo_Now, v_NR_PDCP_SDUList)); //@siclog "Step 4" siclog@ //v_Timing := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_Timing, 60); // v_NumSR_Rcvd:= 0; t_Watchdog.start; alt { //@sic R5s220320 sic@ /*[v_NumSR_Rcvd == 0] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { v_NumSR_Rcvd :=1; repeat; } */ [] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { //@sic R5s220095 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); f_NR_ULGrantConfiguration_ULTBS(nr_Cell1, cs_TimingInfo_Now, v_I_MCS, v_N_PRB, int2bit(v_TDRA, 4), resourceAllocationType1); //@sic R5s190895 sic@ //@siclog "Step 5" siclog@ // SS receives NxPDCP SDUs f_NR_Rx_N_PDCP_SDUs_InOneTTI(v_NR_PDCP_SDUList, p_NR_DRB_Id); deactivate(v_IgnoreNRSR); t_Watchdog.stop; } []t_Watchdog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } }; }//End for TDRA 0 to 13 }//End for iMCS 0 to 28 }//End for nPRB 1 to 110 }//f_NR_7_1_1_4_2_1

After Change

function f_TC_7_1_1_4_2_x_DCI0_0_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters, integer p_Max_Imcs := 28, //@sic R5s201144sic@ boolean p_TransformPrecoder := false) runs on NR_BASE_PTC { /* L2/MAC/7.1 * 7.1.1.4.2.x: UL-SCH Transport Block Size selection */ var integer v_TBSize:=0; // transport block size in bits var integer v_I_MCS:=0; // Modulation and Coding Scehme Index var integer v_N_PRB:=1; // Number of physical resource Blocks var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); // to store the max allowed PRB resources that can be allocated var integer v_TDRA ; // Time Domain resource allocation var NR_PDCP_SDUList_Type v_NR_PDCP_SDUList := {''O}; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var IntegerList_Type v_NoOfSymbols := {4,14}; var integer v_Max_Imcs :=p_Max_Imcs;//@sic R5s201144sic@ var SubFrameNumber_Type v_SF := 2; //@sic R5s210884 sic@ var default v_IgnoreNRSR:= null; //var integer v_NumSR_Rcvd :=0; //@sic R5s220320 sic@ timer t_Watchdog := 12.0; if (v_SCS ==kHz30) {v_SF:= 1;} // Configure SS for DL grant for Max TB size f_NR_DLGrantConfiguration(nr_Cell1, cs_TimingInfo_SubFrame(v_SF),//@sic R5s210884 sic@ resourceAllocationType1, cs_NR_FreqDomainResourceAssignmentDL_Explicit(0, v_Max_nPRB), int2bit(1, 4), //@sic R5s211325 sic@ cs_NR_TransportBlockRetransmissionList_Def(28), -, -, cs_NR_DciFormat_1_0_Params(v_SCS)); /* @sic R5s190442: v_SCS sic@ */ //Steps 1 to 5 are repeated for values of nPRB from 1 to v_Max_nPRB and iMCS from 0 to 28, time domain resource allocation v_TDRA 0..1 //f_NR_ConfigureULGrant_MaxTBSupported(); for (v_N_PRB := 1; v_N_PRB <= v_Max_nPRB; v_N_PRB := v_N_PRB + 1) { //for nPRB 1 to v_Max_nPRB if (p_TransformPrecoder) { if (f_NR_Check_This_N_PRB(v_N_PRB) == false) { // nPRB check in EXCEPTION Step continue; // nprb not allowed } } for (v_I_MCS := 0; v_I_MCS <= v_Max_Imcs ; v_I_MCS := v_I_MCS + 1) { //for iMCS 0 to 28 //@sic R5s190827r1 sic@ for (v_TDRA := 0; v_TDRA <= 1 ; v_TDRA := v_TDRA + 1) { //for Time Domain Resource Allocation 0 to 1 //@sic R5s190827 sic@ /* if(v_TDRA == 0) { v_NoOfSymbols := 4; } else{ v_NoOfSymbols := 14 } */ //SS looks up iTBS in table 7.1.7.1-1 in TS 36.213 based on the value of iMCS //v_I_TBS := f_NR_Get_iTBS_For_iMCS(v_I_MCS); //The SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L, and nPRB. v_TBSize := f_NR_TBS_Calculation(p_CalculationParameters, v_NoOfSymbols[v_TDRA], v_N_PRB, v_I_MCS); //@sic R5-198884 R5s210002 R5s210884 sic@ if ((v_I_MCS >= 27 and v_N_PRB <= 5) or (v_I_MCS == 28 and v_N_PRB == 17)) { //For IMCS= Max IMCS and < 4, the resulting TBS is very small leading to CRC errors in decoding UL data continue; } // skip this nprb, Imcs, TDRA combination. if (v_TBSize < 136) { //this results in zero number of PDCP SDUs continue; } // skip this nprb, Imcs, TDRA combination. // Steps 2 to 5 are performed if TBsize is More than minimum //@siclog "Step 2" siclog@ // SS creates one or more PDCP SDUs, depending on TBsize, in accordance with Table 7.1.1.4.1.1.3.2-2 //Configure SS for DCI Format x with RA type y and a Resource block assignment (RBA) correspondent //to nPRB as specified in 7.1.6.1 in TS 36.213 and Modulation and coding scheme iMCS //@siclog "Step 3" siclog@ // SS sends MAC PDU (NxPDCP SDUs) v_NR_PDCP_SDUList := fl_NR_GetPdcpSduList_1TB(v_TBSize); //calculates v_N, v_PDCPsize and returns f_NR_Build_N_PDCP_SDUList f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@sic R5s211325 sic@ v_IgnoreNRSR := activate(a_NR5GC_SR_Handler(nr_Cell1)); //@sic R5s210884 sic@ //v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); //300 ms in future DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, //crs_RlcBearerRouting_NR(nr_Cell1), cs_TimingInfo_Now, v_NR_PDCP_SDUList)); //@siclog "Step 4" siclog@ //v_Timing := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_Timing, 60); // v_NumSR_Rcvd:= 0; t_Watchdog.start; alt { //@sic R5s220320 sic@ /*[v_NumSR_Rcvd == 0] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { v_NumSR_Rcvd :=1; repeat; } */ [] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { //@sic R5s220095 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); f_NR_ULGrantConfiguration_ULTBS(nr_Cell1, cs_TimingInfo_Now, v_I_MCS, v_N_PRB, int2bit(v_TDRA, 4), resourceAllocationType1); //@sic R5s190895 sic@ //@siclog "Step 5" siclog@ // SS receives NxPDCP SDUs f_NR_Rx_N_PDCP_SDUs_InOneTTI(v_NR_PDCP_SDUList, p_NR_DRB_Id); deactivate(v_IgnoreNRSR); t_Watchdog.stop; } []t_Watchdog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 5"); } }; }//End for TDRA 0 to 13 }//End for iMCS 0 to 28 }//End for nPRB 1 to 110 }//f_NR_7_1_1_4_2_1