3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220560

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2495 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_ReleaseCallWithOptionalDeregister |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation, the timer t_Wait is never started. As a result, the testcase execution stalls at Step 8a1 because the TTCN keeps waiting until guard timer expiry for some UL message. This needs to be fixed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Started the time t_Wait before Step 8a1. Please se below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	fl_ReleaseCallWithOptionalDeregister	3

# Corrections required

## fl_ReleaseCallWithOptionalDeregister

| Function name | fl_ReleaseCallWithOptionalDeregister |
| --- | --- |
| Reason for change | In the current TTCN implementation, the timer t_Wait is never started. As a result, the testcase execution stalls at Step 8a1 because the TTCN keeps waiting until guard timer expiry for some UL message. This needs to be fixed. |
| Summary of change | Started the time t_Wait before Step 8a1. Please se below. |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_ReleaseCallWithOptionalDeregister (NR_CellId_Type p_NR_CellId, float p_WaitTime) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var boolean v_RxdMsg := false; var default v_MyDefaultVar := null; timer t_Wait := p_WaitTime; f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(p_NR_CellId)); //@siclog "Step 5-6" siclog@ //Make the UE release the emergency call. f_NR5GC_EmergencyCallRelease(p_NR_CellId); deactivate(v_MyDefaultVar); if (f_NR5GC_MsgInDefault_GetNumOfNASMessageReq()>0) { v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(1); v_RxdMsg := true; } else { //@siclog "Step 7" siclog@ //@siclog "Step 8a1-8b3" siclog@ //EXCEPTION: Step 8a1 and 8b3 describes optional behaviour that depends on the UE implementation. alt { //@siclog "Step 8a1" siclog@ //The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration". [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, ?, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { v_RxdMsg := true; //@siclog "Step 8a3" siclog@ //Stop Timer. t_Wait.stop; } //@siclog "Step 8b1" siclog@ //Timer expires. [] t_Wait.timeout {} //@sic R5-216198 sic@ } } if (v_RxdMsg and f_Check_NG_DeregistrationReqMOMsg (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, crs_DeregisterType('0'B, '0'B,'01'B))) { //@siclog "Step 8a2" siclog@ //The SS transmits a DEREGISTRATION ACCEPT message. SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB2, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT))); } }

After Change:

function fl_ReleaseCallWithOptionalDeregister (NR_CellId_Type p_NR_CellId, float p_WaitTime) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var boolean v_RxdMsg := false; var default v_MyDefaultVar := null; timer t_Wait := p_WaitTime; f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(p_NR_CellId)); //@siclog "Step 5-6" siclog@ //Make the UE release the emergency call. f_NR5GC_EmergencyCallRelease(p_NR_CellId); deactivate(v_MyDefaultVar); if (f_NR5GC_MsgInDefault_GetNumOfNASMessageReq()>0) { v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(1); v_RxdMsg := true; } else { //@siclog "Step 7" siclog@ //@siclog "Step 8a1-8b3" siclog@ //EXCEPTION: Step 8a1 and 8b3 describes optional behaviour that depends on the UE implementation. t_Wait.start; //WA#11_4_2 alt { //@siclog "Step 8a1" siclog@ //The UE transmits a DEREGISTRATION REQUEST message with De-registration type IE set to "Normal de-registration". [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, ?, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { v_RxdMsg := true; //@siclog "Step 8a3" siclog@ //Stop Timer. t_Wait.stop; } //@siclog "Step 8b1" siclog@ //Timer expires. [] t_Wait.timeout {} //@sic R5-216198 sic@ } } if (v_RxdMsg and f_Check_NG_DeregistrationReqMOMsg (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, crs_DeregisterType('0'B, '0'B,'01'B))) { //@siclog "Step 8a2" siclog@ //The SS transmits a DEREGISTRATION ACCEPT message. SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB2, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT))); } }