3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220594

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0821 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS NR5GC testcase 7.22 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The RequestURI in the step 1 reINVITE needs to be checked against contact header from the first INVITE which is default value defined in tsc_IMS_CallerSipAddrStr. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change value of the contact header that is used to check RequestURI in the step 1 reINVITE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.22.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_7_22_NR5GC_IMS1 |
| --- | --- |
| Reason for change | The RequestURI in the step 1 reINVITE needs to be checked against contact header from the first INVITE which is default value defined in tsc_IMS_CallerSipAddrStr. |
| Summary of change | Change value of the contact header that is used to check RequestURI in the step 1 reINVITE. |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_22_NR5GC_IMS1() runs on IMS_PTC { // MTSI MT Voice Call / add video and remove video / with preconditions at both originating UE and terminating UE / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var SDP_Message v_LatestSdpOfferFromUE; var template (value) SDP_Message v_LatestSdpAnswerFromSS; var charstring v_CallIdentification; var boolean v_DontIncrementSessVers := true; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); f_IMS_MTCallSetup_NR5GC_A_5_1(); v_CallIdentification := f_UT_GetCallId(MMI); f_IMS_TestBody_Set(true); f_UT_AddVideoToIMSCall(MMI, v_CallIdentification); // @siclog "Step 1" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_Video_Step2(AddVideo, -, tsc_IMS_SDP_AudioCodecList_EVS_only, tsc_IMS_SDP_VideoCodecList_H265_H264); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 21"); // @siclog "Step 2 - 3" siclog@ 100 Trying, 183 Session Progress v_ResponseWithSdp := f_IMS_MOCallSetup_Video_Step3_4(AddVideo, v_InviteRequestWithSdp, EVS_16000, H265_90000); // @siclog "Step 4 - 5" siclog@ PRACK, 200 OK f_IMS_MOCallSetup_ReceivePRACK_Send200OK(v_InviteRequestWithSdp, v_ResponseWithSdp.Response.msgHeader); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5A" siclog@ f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MO_AddVideo); // @siclog "Step 6 - 7" siclog@ UPDATE, 200 OK v_LatestSdpOfferFromUE := f_IMS_MOCallSetup_NR5GC_Video_Step6_7(AddVideo, v_InviteRequestWithSdp); v_LatestSdpAnswerFromSS := f_IMS_BuildSDP_UpdatedAnswer_TX(v_LatestSdpOfferFromUE, v_DontIncrementSessVers); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); f_IMS_IPCAN_WaitForTrigger(IPCAN); // @siclog "Step 8 - 9" siclog@ 200 OK, ACK f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 9"); // @siclog "Step 10 - 14" siclog@ Release Video f_IMS_VideoCall_NR5GC_ReleaseVideoMT(v_LatestSdpAnswerFromSS); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 12"); f_IMS_TestBody_Set(false); // Call release and postamble: f_IMS_CC_PostambleWithCallReleaseMT(IPCAN_MT_SpeechCall); }

After Change

function f_TC_7_22_NR5GC_IMS1() runs on IMS_PTC { // MTSI MT Voice Call / add video and remove video / with preconditions at both originating UE and terminating UE / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var SDP_Message v_LatestSdpOfferFromUE; var template (value) SDP_Message v_LatestSdpAnswerFromSS; var charstring v_CallIdentification; var boolean v_DontIncrementSessVers := true; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_CC_StartCall(IPCAN_MT_SpeechCall); f_IMS_MTCallSetup_NR5GC_A_5_1(); v_CallIdentification := f_UT_GetCallId(MMI); f_IMS_TestBody_Set(true); f_UT_AddVideoToIMSCall(MMI, v_CallIdentification); // @siclog "Step 1" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_Video_Step2(AddVideo, f_SIP_BuildSipUri_TX(tsc_IMS_CallerSipAddrStr, tsc_IMS_CallerPort), tsc_IMS_SDP_AudioCodecList_EVS_only, tsc_IMS_SDP_VideoCodecList_H265_H264); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 21"); // @siclog "Step 2 - 3" siclog@ 100 Trying, 183 Session Progress v_ResponseWithSdp := f_IMS_MOCallSetup_Video_Step3_4(AddVideo, v_InviteRequestWithSdp, EVS_16000, H265_90000); // @siclog "Step 4 - 5" siclog@ PRACK, 200 OK f_IMS_MOCallSetup_ReceivePRACK_Send200OK(v_InviteRequestWithSdp, v_ResponseWithSdp.Response.msgHeader); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5A" siclog@ f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MO_AddVideo); // @siclog "Step 6 - 7" siclog@ UPDATE, 200 OK v_LatestSdpOfferFromUE := f_IMS_MOCallSetup_NR5GC_Video_Step6_7(AddVideo, v_InviteRequestWithSdp); v_LatestSdpAnswerFromSS := f_IMS_BuildSDP_UpdatedAnswer_TX(v_LatestSdpOfferFromUE, v_DontIncrementSessVers); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); f_IMS_IPCAN_WaitForTrigger(IPCAN); // @siclog "Step 8 - 9" siclog@ 200 OK, ACK f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 9"); // @siclog "Step 10 - 14" siclog@ Release Video f_IMS_VideoCall_NR5GC_ReleaseVideoMT(v_LatestSdpAnswerFromSS); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 12"); f_IMS_TestBody_Set(false); // Call release and postamble: f_IMS_CC_PostambleWithCallReleaseMT(IPCAN_MT_SpeechCall); }