3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220609

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0827 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS emergency call test case 10.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add Emergency Call test case 10.9. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to Emergency Call test case 10.9 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.9.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	6

2.3	Change 3	7

3	Branches executed	8

4	Execution Log Files	9

4.1	Qualcomm SM 8450+SDX65 5G UE	9

5	References	9

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 10.9 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	10.9

ATS Version:	iwd-TTCN3-B2020-09_D22wk12

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR,

UE used:	Qualcomm SM 8450+SDX65 5G UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_10_9_NR5GC_IMS2 |
| --- | --- |
| Reason for change | - Resource reservation, PDU Session Modification, will happen during call stablishment. With current implementation it may happen that PDU Session Modification procedure has not finished before Call is released (BYE is sent from SS). - After the call is released it is necessary to inform IPCAN and also to handle coordination trigger sent by IPCAN – both done by f_IMS_CC_EndCall. - Another coordination trigger to IPCAN is needed to coordinate PDU session release. |
| Summary of change | - Added a delay of 5s to give some time for PDU Session Modification Complete to NR PTC. - Added f_IMS_CC_EndCall in f_TC_10_9_NR5GC_IMS2 after the call is released. - Added coordination trigger to IPCAN |
| TTCN module | IMS\NR5GC_10\IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_10_9_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var SipUrl v_ContactUrlUE; var template (value) MessageHeader v_MessageHeader; var template (value) MessageBody v_MessageBody; var IMS_DATA_REQ v_IMS_DATA_REQ; var REGISTER_Request v_RegisterRequest; var boolean v_CondA7 := true; var boolean v_GeolocationUsage := pc_IMS_Geolocation; /* the test case intends to use GeoLocationInformation if supported by the UE (PICS) @sic R5s150846 sic@ INVITE (step 1 of Annex C.22) refers to condition A8 of A.2.1 */ f_IMS_PTC_Init(PDN_2); // Test Case Body f_IMS_TestBody_Set(true); f_IMS_IMS_WaitForTrigger(OtherPDN); // @siclog "Steps 2-9" siclog@ // Steps 1-8 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed. // EXCEPTION: In parallel to the events described in steps 9-10 below the events specified in step of Table 4.9.11.2.2-2 of TS 38.508-1 [21] take place // @siclog "Step 11-12" siclog @ // Steps 9-10 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed. //EXCEPTION: In parallel to the events described in steps 12-20 below the events specified in 11-13 of annex Table 4.9.11.2.2-1 of TS 38.508-1[21] are performed. f_IMS_CC_StartCall(IPCAN_EmergencyCall_NoRegistration); // 4.9.11.2.2-2 of TS 38.508-1 //@siclog "Step 12" siclog@ //UE sends REGISTER(Step 1 of Annex A.3) v_IMS_DATA_REQ := f_IMS_REGISTER_InitialRequest(-, omit, -, -, -, v_CondA7); v_RegisterRequest := v_IMS_DATA_REQ.Request.Register; // Reconfig of UE client ports in case the UE uses UDP and different than default (5060) ports f_IMS_Register_SecurityInit(v_RegisterRequest); //@siclog "Step 13" siclog@ //SS sends 401 Unauthorized(Step 2 of Annex A.3) IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterRequest)))); //@siclog "Step 14" siclog@ //UE sends REGISTER(Step 3 of Annex A.3) v_IMS_DATA_REQ := f_IMS_REGISTER_SubsequentRequest(-, -, v_CondA7); v_RegisterRequest := v_IMS_DATA_REQ.Request.Register; //@siclog "Step 15" siclog@ //SS sends 403 Forbidden v_MessageHeader := f_IMS_RegisterResponse_403_MessageHeaderTX(v_RegisterRequest); v_MessageHeader.contentType := cs_ContentType(c_ims3gppApplication); v_MessageBody := cs_MessageBody_XML_Ims_3gpp(cs_Ims_3gpp_Emergency("anonymous-emergencycall")); IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine403, v_MessageHeader, v_MessageBody))); f_IMS_GlobalRelease (oldestProtectedContextOnly); // Prose CR needed //@siclog "Steps 16 - 20" siclog@ //Check: Does the UE send a correctly composed INVITE request?(Step 1 of annex A.6) v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(A_2_1_A6, v_GeolocationUsage); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Steps 16"); // Emergency call setup: 34.229-5 Annex A.6 f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ, v_GeolocationUsage); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20"); //@siclog "Steps 21-22" siclog@ //SS sends BYE.(Step 1 of annex A.8) //UE sends 200 OK.(Step 1 of annex A.8) v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_TestBody_Set(false); f_IMS_IMS_SendCoOrdMsg(OtherPDN); }

After Change

function f_TC_10_9_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var SipUrl v_ContactUrlUE; var template (value) MessageHeader v_MessageHeader; var template (value) MessageBody v_MessageBody; var IMS_DATA_REQ v_IMS_DATA_REQ; var REGISTER_Request v_RegisterRequest; var boolean v_CondA7 := true; var boolean v_GeolocationUsage := pc_IMS_Geolocation; /* the test case intends to use GeoLocationInformation if supported by the UE (PICS) @sic R5s150846 sic@ INVITE (step 1 of Annex C.22) refers to condition A8 of A.2.1 */ f_IMS_PTC_Init(PDN_2); // Test Case Body f_IMS_TestBody_Set(true); f_IMS_IMS_WaitForTrigger(OtherPDN); // @siclog "Steps 2-9" siclog@ // Steps 1-8 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed. // EXCEPTION: In parallel to the events described in steps 9-10 below the events specified in step of Table 4.9.11.2.2-2 of TS 38.508-1 [21] take place // @siclog "Step 11-12" siclog @ // Steps 9-10 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed. //EXCEPTION: In parallel to the events described in steps 12-20 below the events specified in 11-13 of annex Table 4.9.11.2.2-1 of TS 38.508-1[21] are performed. f_IMS_CC_StartCall(IPCAN_EmergencyCall_NoRegistration); // 4.9.11.2.2-2 of TS 38.508-1 //@siclog "Step 12" siclog@ //UE sends REGISTER(Step 1 of Annex A.3) v_IMS_DATA_REQ := f_IMS_REGISTER_InitialRequest(-, omit, -, -, -, v_CondA7); v_RegisterRequest := v_IMS_DATA_REQ.Request.Register; // Reconfig of UE client ports in case the UE uses UDP and different than default (5060) ports f_IMS_Register_SecurityInit(v_RegisterRequest); //@siclog "Step 13" siclog@ //SS sends 401 Unauthorized(Step 2 of Annex A.3) IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterRequest)))); //@siclog "Step 14" siclog@ //UE sends REGISTER(Step 3 of Annex A.3) v_IMS_DATA_REQ := f_IMS_REGISTER_SubsequentRequest(-, -, v_CondA7); v_RegisterRequest := v_IMS_DATA_REQ.Request.Register; //@siclog "Step 15" siclog@ //SS sends 403 Forbidden v_MessageHeader := f_IMS_RegisterResponse_403_MessageHeaderTX(v_RegisterRequest); v_MessageHeader.contentType := cs_ContentType(c_ims3gppApplication); v_MessageBody := cs_MessageBody_XML_Ims_3gpp(cs_Ims_3gpp_Emergency("anonymous-emergencycall")); IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine403, v_MessageHeader, v_MessageBody))); f_IMS_GlobalRelease (oldestProtectedContextOnly); // Prose CR needed //@siclog "Steps 16 - 20" siclog@ //Check: Does the UE send a correctly composed INVITE request?(Step 1 of annex A.6) v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(A_2_1_A6, v_GeolocationUsage); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Steps 16"); // Emergency call setup: 34.229-5 Annex A.6 f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ, v_GeolocationUsage); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20"); f_Delay(5.0); //@siclog "Steps 21-22" siclog@ //SS sends BYE.(Step 1 of annex A.8) //UE sends 200 OK.(Step 1 of annex A.8) v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_CC_EndCall(IPCAN_EmergencyCall_NormalService, IPCAN_SpeechAndEmergencyCall); //signal to IPCAN that call is released and wait for coordination from there f_IMS_TestBody_Set(false); // Inform NR that PDU session can be released (caught by f_NR5GC_EmergencyCallRelease) f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IMS_SendCoOrdMsg(OtherPDN); }

## Change 2

| Function name | f_TC_10_9_NR5GC_IMS1 |
| --- | --- |
| Reason for change | The f_IMS_CC_Postamble (if called with TestProcedure parameter) is informing IPCAN to release PDU session, however that was already done from IMS2 and doing it again from IMS1 would lead to runtime error as there are no more PDU sessions. |
| Summary of change | f_IMS_CC_Postamble is not called with the TestProcedure parameter. |
| TTCN module | IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_10_9_NR5GC_IMS1() runs on IMS_PTC { // Emergency call without emergency registration / UE credentials are not accepted / 5GS var boolean v_GeolocationUsage := pc_IMS_Geolocation; f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED); // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); // wait for trigger after IMS call release f_IMS_CC_Postamble(IPCAN_EmergencyCall_NoRegistration); }

After Change

function f_TC_10_9_NR5GC_IMS1() runs on IMS_PTC { // Emergency call without emergency registration / UE credentials are not accepted / 5GS var boolean v_GeolocationUsage := pc_IMS_Geolocation; f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED); // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); // wait for trigger after IMS call release f_IMS_CC_Postamble(); //IPCAN was already informed to end the call from IMS2 }

## Change 3

| Function name | fl_IP_IMS_Server_SendDL |
| --- | --- |
| Reason for change | In scenarios where IMS security establishment failed but SIP message needs to be sent in DL nonetheless (emergency call with failed/rejected 401 procedure, e.g. tests 10.9 and 10.10) it is needed to assign UE/NW addresses for routing. |
| Summary of change | Added routing addresses into fl_IP_IMS_Server_SendDL also when only unprotected context is present. |
| TTCN module | IP_PTC\IP_PTC_IMS_Handler.ttcn |
| MCC160 Comment |  |

Before Change

function fl_IP_IMS_Server_SendDL(inout IMS_Server_Type p_ImsServer, IMS_RoutingInfo_Type p_RoutingInfo, IMS_RequestResponse_Type p_RequestResponse, bitstring p_Data) runs on IP_PTC { … if (isvalue(p_RoutingInfo.NW_Address) and isvalue(p_RoutingInfo.UE_Address)) { v_NW_Address := p_RoutingInfo.NW_Address; v_UE_Address := p_RoutingInfo.UE_Address; } else if (ispresent(p_ImsServer.RegistrationAddress)) { /* @sic R5s160735: fl_ImsServer_UeIsRegistered removed sic@ */ v_NW_Address := p_ImsServer.RegistrationAddress.NW; v_UE_Address := p_ImsServer.RegistrationAddress.UE; } else { FatalError(__FILE__, __LINE__, "missing IP addresses"); } …

After Change

function fl_IP_IMS_Server_SendDL(inout IMS_Server_Type p_ImsServer, IMS_RoutingInfo_Type p_RoutingInfo, IMS_RequestResponse_Type p_RequestResponse, bitstring p_Data) runs on IP_PTC { … if (isvalue(p_RoutingInfo.NW_Address) and isvalue(p_RoutingInfo.UE_Address)) { v_NW_Address := p_RoutingInfo.NW_Address; v_UE_Address := p_RoutingInfo.UE_Address; } else if (ispresent(p_ImsServer.RegistrationAddress)) { /* @sic R5s160735: fl_ImsServer_UeIsRegistered removed sic@ */ v_NW_Address := p_ImsServer.RegistrationAddress.NW; v_UE_Address := p_ImsServer.RegistrationAddress.UE; } else if (ispresent(p_ImsServer.UnprotectedContext.TcpServer)) { v_NW_Address := p_ImsServer.UnprotectedContext.TcpServer.Local.IpAddr; v_UE_Address := p_ImsServer.UnprotectedContext.TcpServer.Remote.IpAddr; } else { FatalError(__FILE__, __LINE__, "missing IP addresses"); } …

# Branches executed

On Anritsu, the test case was executed on NR band n41 using NR ciphering nea1 and integrity algorithm nia1.

# Execution Log Files

## Qualcomm SM 8450+SDX65 5G UE

The Qualcomm SDX UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_10_9_LOG.xml

- PICS/PIXIT parameter file:

TC_10_9_PICS.txt

# References

| [1] | R5s220610 - Supporting information for addition of IMS 5GS emergency call test case 10.9 |
| --- | --- |