3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220615

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2525 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR MAC test cases 7.1.1.4.2.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | With the current PDSCH Time Domain resource allocation, with K0 value 1 and SLIV 27, there may be an overlap between PDCCH and PDSCH symbols, which would cause interference and may subsequently cause the UE to not decode the DL PDUs (DATA or RLC ACK) resulting in test case failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the SLIV to 53 (S=2, L=12) Note : An associated prose CR R5-222749 is raised at RAN5#95 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.2.1.ENDC, 7.1.1.4.2.1.NR5GC, 7.1.1.4.2.3.ENDC, 7.1.1.4.2.3.NR5GC, 7.1.1.4.2.4.ENDC, 7.1.1.4.2.4.NR5GC, 7.1.1.4.2.5.ENDC, 7.1.1.4.2.5.NR5GC, 7.1.1.4.2.6.ENDC, 7.1.1.4.2.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | R5-222749 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | Template cs_NR_PDSCH_TimeDomainResourceAllocationList_71142x |
| --- | --- |
| Reason for change | With the current PDSCH Time Domain resource allocation, with K0 value 1 and SLIV 27, there may be an overlap between PDCCH and PDSCH symbols, which would cause interference and may subsequently cause the UE to not decode the DL PDUs (DATA or RLC ACK) resulting in test case failure. |
| Summary of change | Changed the SLIV to 53 (S=2, L=12) Note : An associated prose CR R5-222749 is raised at RAN5#95 |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

template (value) PDSCH_TimeDomainResourceAllocationList cs_NR_PDSCH_TimeDomainResourceAllocationList_71142x := { /* @status APPROVED (ENDC, NR5GC) */ // Acc to 38.523-1 Table 7.1.1.4.1.0-1: PDSCH-TimeDomainResourceAllocationList //@sic R5-190677 sic@ cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53), // S=2, L=12 cs_NR_PDSCH_TimeDomainResourceAllocation(1, typeA, 27) // K0=1, S=0, L=14 //@sic R5s211325 sic@ };

After Change:

template (value) PDSCH_TimeDomainResourceAllocationList cs_NR_PDSCH_TimeDomainResourceAllocationList_71142x := { /* @status APPROVED (ENDC, NR5GC) */ // Acc to 38.523-1 Table 7.1.1.4.1.0-1: PDSCH-TimeDomainResourceAllocationList //@sic R5-190677 sic@ cs_NR_PDSCH_TimeDomainResourceAllocation(omit, typeA, 53), // S=2, L=12 cs_NR_PDSCH_TimeDomainResourceAllocation(1, typeA, 53) // K0=1, S=0, L=14 //@sic R5s211325 sic@ };