3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220676

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2534 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 7.1.1.3.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UL Grant of 40 bit is already configured at Step 3 so configuring it again at STEP 15 is redundant, similar to Step 7 and Step 11 the SS will automatically take care of this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed f_NR_ULGrantConfiguration_Start function call at STEP 15. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.4.NR5GC, 7.1.1.3.4.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to f_TC_7_1_1_3_4_NR_TestBody ()	2

# Corrections required

## Correction to f_TC_7_1_1_3_4_NR_TestBody ()

| Function name | f_TC_7_1_1_3_4_NR_TestBody () |
| --- | --- |
| Reason for change | UL Grant of 40 bit is already configured at Step 3 so configuring it again at STEP 15 is redundant, similar to Step 7 and Step 11 the SS will automatically take care of this. |
| Summary of change | Removed f_NR_ULGrantConfiguration_Start function call at STEP 15. |
| TTCN module | \ NR_TC_Common\7_1_1\ MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_3_4_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { //TB sizes 40, 80, 576, 256, for FR1 var NR_ResourceAllocation_Type v_NR_ResourceAllocation[4]; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var integer v_BSR1; var integer v_BSR2; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_MAC_SDU_Type v_EncodedRlcPdu1_Poll; var NR_MAC_SDU_Type v_EncodedRlcPdu2; var NR_MAC_SDU_Type v_EncodedRlcPdu2_Poll; var NR_MAC_SDU_Type v_EncodedRlcPdu3; var NR_MAC_SDU_Type v_EncodedRlcPdu3_Poll; var NR_MAC_SDU_Type v_EncodedRlcPdu4; var NR_MAC_SDU_Type v_EncodedRlcPdu5; var NR_MAC_SDU_Type v_EncodedRlcPdu5_Poll; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_SubFrameTimingTa; var SubFrameTiming_Type v_SubFrameTimingTb; var integer v_Duration; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL; var template (present) NR_MAC_CE_SubPDU_UL_List_Type v_CE_SubPDUListUL; var template (present) NR_MAC_CE_SubPDU_UL_List_Type v_CE_SubPDUListUL2; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL2; var integer v_SBSR; //@sic R5s200511 sic@ //@sic R5-201519 sic@ var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6);//@sic R5-201519 sic@ var B6_Type v_LCID2 := int2bit(p_NR_DRB_Id+4, 6);//@sic R5-201519 sic@ var B6_Type v_LCID3 := int2bit(p_NR_DRB_Id+5, 6);//@sic R5-201519 sic@ timer t_Watchdog := 5.0; // Stop UL Grant configuration //@siclog "Step 1" siclog@ f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@sic R5s190899 sic@ v_NR_ResourceAllocation[0] := f_NR_ResourceAllocation_UplinkGrant(40, v_NR_UplinkBWP); v_NR_ResourceAllocation[1] := f_NR_ResourceAllocation_UplinkGrant(608, v_NR_UplinkBWP); v_NR_ResourceAllocation[2] := f_NR_ResourceAllocation_UplinkGrant(256, v_NR_UplinkBWP); //@siclog "Step 2" siclog@ //3 B PDCP H + 3 Byte RLC H + 9B PDCP SDU = 12Byte RLC SDU, 15 B RLC PDU +2 Byte MAC SH =17 Byte MAC Sub PDU // 2* 17 =34 B =272 bit an allowed TBsize v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_9B))); v_EncodedRlcPdu1_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_9B))); v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_9B))); v_EncodedRlcPdu2_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_9B))); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1),//@sic R5-201519 sic@ cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu2) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 3" siclog@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20); // 20 ms later f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation[0].Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation[0].Lrbs)); //40 bits // @siclog "Step 4" siclog@ t_Watchdog.start; v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload('010'B, int2bit(4, 5))) }; //LCG 2, value =4 v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('??'O)) }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)) -> value v_NR_DRB_COMMON_IND; // 2byte Padding header and 1 byte padding payload v_SubFrameTimingTa := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5-7" siclog@ //Automatically taken by SS and duration check // @siclog "Step 8" siclog@ v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload('010'B, int2bit(4, 5))) }; //LCG 2, value =4 v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('??'O)) }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)) -> value v_NR_DRB_COMMON_IND; // 1byte Padding header and 2 byte padding payload v_SubFrameTimingTb := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; //@sic R5s200511 sic@ v_SBSR := bit2int(v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.MacPdu[0].UL.CE_SubPDUList[0].ControlElement.ShortBSR.BufferSize); if (v_SBSR >= 4) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } v_Duration := f_SubFrameTiming_Duration(v_SubFrameTimingTa, v_SubFrameTimingTb); if (f_NR_SubFrameTimingCheckDuration(nr_Cell1, l2Timer, v_Duration, 320)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } // @siclog "Step 9" siclog@ //3 B PDCP H + 3 Byte RLC H + 9B PDCP SDU = 12Byte RLC SDU, 15 B RLC PDU +2 Byte MAC SH =17 Byte =136 bit an allowed TB size v_EncodedRlcPdu3 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_9B))); v_EncodedRlcPdu3_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_9B))); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID2, v_EncodedRlcPdu3) };//@sic R5-201519 sic@ v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); // @siclog "Step 10-11" siclog@ //Automatically taken by SS // @siclog "Step 12" siclog@ v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload('010'B, int2bit(5, 5))) }; //LCG 2, value =5 v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('??'O)) }; // 1byte Padding header and 2 byte padding payload DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); //@siclog "Step 13" siclog@ //3 B PDCP H + 3 B RLC H + 2B PDCP SDU = 5Byte RLC SDU, 8 B RLC PDU +2 Byte MAC SH =10 Byte MAC Sub PDU // 2* 10 =20 B =160 bit an allowed TBsize v_EncodedRlcPdu4 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_2B))); v_EncodedRlcPdu5 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_2B))); v_EncodedRlcPdu5_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_2B))); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID3, v_EncodedRlcPdu4),//@sic R5-201519 sic@ cs_NR_MAC_SDU_SubPDU_LI8(v_LCID3, v_EncodedRlcPdu5) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 14-15" siclog@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20); // 20 ms later f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation[0].Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation[0].Lrbs)); //40 bits //@siclog "Step 16" siclog@ v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_LongBSR_SubHeader(int2bit(3, 8)), cr_LongBSR_Payload) }; //LCG 2,1 present v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL) }; //@sic R5s190552 sic@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)) -> value v_NR_DRB_COMMON_IND; v_BSR1 := oct2int(v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.MacPdu[0].UL.CE_SubPDUList[0].ControlElement.LongBSR.BufferSizeList[0]); v_BSR2 := oct2int(v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.MacPdu[0].UL.CE_SubPDUList[0].ControlElement.LongBSR.BufferSizeList[1]); v_SubFrameTimingTa := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; if ((v_BSR1 >= 1) and (v_BSR2 >= 20)) { //@sic R5-197002 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 16"); } // @siclog "Step 17-19" siclog@ f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation[1].Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation[1].Lrbs)); //576 bits //@siclog "Step 20" siclog@ //@sic R5-201519 sic@ v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID3, int2bit(8, 8), '0'B), v_EncodedRlcPdu4), //10B cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID3, int2bit(8, 8), '0'B), v_EncodedRlcPdu5_Poll), //10B cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID2, int2bit(15, 8), '0'B), v_EncodedRlcPdu3_Poll), //17B cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID, int2bit(15, 8), '0'B), v_EncodedRlcPdu1), //17B cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID, int2bit(15, 8), '0'B), v_EncodedRlcPdu2_Poll) //17B }; //v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, omit, cr_NR_MAC_Padding_SubPDU(''O)) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; //@sic R5-197002 sic@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19"); //@siclog "Step 21-23" siclog@ //@sic R5-201519 sic@ f_NR_RLC_Status_MAC_PDU_Tx(v_LCID3, 2, p_NR_DRB_Id); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID2, 1, p_NR_DRB_Id); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID , 2, p_NR_DRB_Id); //3 B PDCP H + 3 Byte RLC H + 5B PDCP SDU = 8 Byte RLC SDU, 11 B RLC PDU +2 Byte MAC SH =13 Byte //TBD if any padding needed v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_5B))); //3 B PDCP H + 3 Byte RLC H + 4B PDCP SDU = 7 Byte RLC SDU, 10 B RLC PDU +2 Byte MAC SH =12 Byte v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_4B))); //Poll PDU need to be encoded again, as at step 27, 12B & 13B PDU is required //@sic R5s190899 sic@ v_EncodedRlcPdu1_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_5B))); v_EncodedRlcPdu2_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_4B))); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL //@siclog "Step 24" siclog@ // MAC Sub PDU 1:=13 B, MAC Sub PDU 1:=12 B, + 1 B Padding =208 bits an allowed TB size //@sic R5-201519 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1), cs_NR_MAC_SDU_SubPDU_LI8(v_LCID3, v_EncodedRlcPdu2) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL, cs_NR_MAC_Padding_SubPDU) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id,cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); // @siclog "Step 25-26" siclog@ f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation[2].Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation[2].Lrbs)); //256bits // @siclog "Step 27" siclog@ //@sic R5-201519 sic@ v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID3, int2bit(10, 8), '0'B), v_EncodedRlcPdu2_Poll), //12B cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID, int2bit(11, 8), '0'B), v_EncodedRlcPdu1_Poll) //13B }; v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_LongBSR_SubHeader(int2bit(3, 8)),cr_LongBSR_Payload) }; //LCG 2,1 present, v_CE_SubPDUListUL2 := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload(?, ?)) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('?'O)) }; // 2 bye padding v_MAC_PDUListUL2 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, v_CE_SubPDUListUL2, cr_NR_MAC_Padding_SubPDU('????'O)) }; // 5 bye padding DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, (v_MAC_PDUListUL, v_MAC_PDUListUL2))); t_Watchdog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 27"); //@siclog "Step 28-29" siclog@ //@sic R5-201519 sic@ f_NR_RLC_Status_MAC_PDU_Tx(v_LCID3, 3, p_NR_DRB_Id); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 3, p_NR_DRB_Id); };

After Change:

function f_TC_7_1_1_3_4_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { //TB sizes 40, 80, 576, 256, for FR1 var NR_ResourceAllocation_Type v_NR_ResourceAllocation[4]; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var integer v_BSR1; var integer v_BSR2; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_MAC_SDU_Type v_EncodedRlcPdu1_Poll; var NR_MAC_SDU_Type v_EncodedRlcPdu2; var NR_MAC_SDU_Type v_EncodedRlcPdu2_Poll; var NR_MAC_SDU_Type v_EncodedRlcPdu3; var NR_MAC_SDU_Type v_EncodedRlcPdu3_Poll; var NR_MAC_SDU_Type v_EncodedRlcPdu4; var NR_MAC_SDU_Type v_EncodedRlcPdu5; var NR_MAC_SDU_Type v_EncodedRlcPdu5_Poll; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_SubFrameTimingTa; var SubFrameTiming_Type v_SubFrameTimingTb; var integer v_Duration; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL; var template (present) NR_MAC_CE_SubPDU_UL_List_Type v_CE_SubPDUListUL; var template (present) NR_MAC_CE_SubPDU_UL_List_Type v_CE_SubPDUListUL2; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL2; var integer v_SBSR; //@sic R5s200511 sic@ //@sic R5-201519 sic@ var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6);//@sic R5-201519 sic@ var B6_Type v_LCID2 := int2bit(p_NR_DRB_Id+4, 6);//@sic R5-201519 sic@ var B6_Type v_LCID3 := int2bit(p_NR_DRB_Id+5, 6);//@sic R5-201519 sic@ timer t_Watchdog := 5.0; // Stop UL Grant configuration //@siclog "Step 1" siclog@ f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@sic R5s190899 sic@ v_NR_ResourceAllocation[0] := f_NR_ResourceAllocation_UplinkGrant(40, v_NR_UplinkBWP); v_NR_ResourceAllocation[1] := f_NR_ResourceAllocation_UplinkGrant(608, v_NR_UplinkBWP); v_NR_ResourceAllocation[2] := f_NR_ResourceAllocation_UplinkGrant(256, v_NR_UplinkBWP); //@siclog "Step 2" siclog@ //3 B PDCP H + 3 Byte RLC H + 9B PDCP SDU = 12Byte RLC SDU, 15 B RLC PDU +2 Byte MAC SH =17 Byte MAC Sub PDU // 2* 17 =34 B =272 bit an allowed TBsize v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_9B))); v_EncodedRlcPdu1_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_9B))); v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_9B))); v_EncodedRlcPdu2_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_9B))); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1),//@sic R5-201519 sic@ cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu2) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 3" siclog@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20); // 20 ms later f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation[0].Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation[0].Lrbs)); //40 bits // @siclog "Step 4" siclog@ t_Watchdog.start; v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload('010'B, int2bit(4, 5))) }; //LCG 2, value =4 v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('??'O)) }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)) -> value v_NR_DRB_COMMON_IND; // 2byte Padding header and 1 byte padding payload v_SubFrameTimingTa := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5-7" siclog@ //Automatically taken by SS and duration check // @siclog "Step 8" siclog@ v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload('010'B, int2bit(4, 5))) }; //LCG 2, value =4 v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('??'O)) }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)) -> value v_NR_DRB_COMMON_IND; // 1byte Padding header and 2 byte padding payload v_SubFrameTimingTb := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; //@sic R5s200511 sic@ v_SBSR := bit2int(v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.MacPdu[0].UL.CE_SubPDUList[0].ControlElement.ShortBSR.BufferSize); if (v_SBSR >= 4) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } v_Duration := f_SubFrameTiming_Duration(v_SubFrameTimingTa, v_SubFrameTimingTb); if (f_NR_SubFrameTimingCheckDuration(nr_Cell1, l2Timer, v_Duration, 320)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } // @siclog "Step 9" siclog@ //3 B PDCP H + 3 Byte RLC H + 9B PDCP SDU = 12Byte RLC SDU, 15 B RLC PDU +2 Byte MAC SH =17 Byte =136 bit an allowed TB size v_EncodedRlcPdu3 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_9B))); v_EncodedRlcPdu3_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_9B))); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID2, v_EncodedRlcPdu3) };//@sic R5-201519 sic@ v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); // @siclog "Step 10-11" siclog@ //Automatically taken by SS // @siclog "Step 12" siclog@ v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload('010'B, int2bit(5, 5))) }; //LCG 2, value =5 v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('??'O)) }; // 1byte Padding header and 2 byte padding payload DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); //@siclog "Step 13" siclog@ //3 B PDCP H + 3 B RLC H + 2B PDCP SDU = 5Byte RLC SDU, 8 B RLC PDU +2 Byte MAC SH =10 Byte MAC Sub PDU // 2* 10 =20 B =160 bit an allowed TBsize v_EncodedRlcPdu4 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_2B))); v_EncodedRlcPdu5 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_2B))); v_EncodedRlcPdu5_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_2B))); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID3, v_EncodedRlcPdu4),//@sic R5-201519 sic@ cs_NR_MAC_SDU_SubPDU_LI8(v_LCID3, v_EncodedRlcPdu5) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 14-15" siclog@ //WA#WI=962744 - Removed. //v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20); // 20 ms later //f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation[0].Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation[0].Lrbs)); //40 bits //@siclog "Step 16" siclog@ v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_LongBSR_SubHeader(int2bit(3, 8)), cr_LongBSR_Payload) }; //LCG 2,1 present v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL) }; //@sic R5s190552 sic@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)) -> value v_NR_DRB_COMMON_IND; v_BSR1 := oct2int(v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.MacPdu[0].UL.CE_SubPDUList[0].ControlElement.LongBSR.BufferSizeList[0]); v_BSR2 := oct2int(v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.MacPdu[0].UL.CE_SubPDUList[0].ControlElement.LongBSR.BufferSizeList[1]); v_SubFrameTimingTa := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; if ((v_BSR1 >= 1) and (v_BSR2 >= 20)) { //@sic R5-197002 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 16"); } // @siclog "Step 17-19" siclog@ f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation[1].Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation[1].Lrbs)); //576 bits //@siclog "Step 20" siclog@ //@sic R5-201519 sic@ v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID3, int2bit(8, 8), '0'B), v_EncodedRlcPdu4), //10B cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID3, int2bit(8, 8), '0'B), v_EncodedRlcPdu5_Poll), //10B cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID2, int2bit(15, 8), '0'B), v_EncodedRlcPdu3_Poll), //17B cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID, int2bit(15, 8), '0'B), v_EncodedRlcPdu1), //17B cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID, int2bit(15, 8), '0'B), v_EncodedRlcPdu2_Poll) //17B }; //v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, omit, cr_NR_MAC_Padding_SubPDU(''O)) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; //@sic R5-197002 sic@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19"); //@siclog "Step 21-23" siclog@ //@sic R5-201519 sic@ f_NR_RLC_Status_MAC_PDU_Tx(v_LCID3, 2, p_NR_DRB_Id); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID2, 1, p_NR_DRB_Id); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID , 2, p_NR_DRB_Id); //3 B PDCP H + 3 Byte RLC H + 5B PDCP SDU = 8 Byte RLC SDU, 11 B RLC PDU +2 Byte MAC SH =13 Byte //TBD if any padding needed v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_5B))); //3 B PDCP H + 3 Byte RLC H + 4B PDCP SDU = 7 Byte RLC SDU, 10 B RLC PDU +2 Byte MAC SH =12 Byte v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_4B))); //Poll PDU need to be encoded again, as at step 27, 12B & 13B PDU is required //@sic R5s190899 sic@ v_EncodedRlcPdu1_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_5B))); v_EncodedRlcPdu2_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_4B))); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL //@siclog "Step 24" siclog@ // MAC Sub PDU 1:=13 B, MAC Sub PDU 1:=12 B, + 1 B Padding =208 bits an allowed TB size //@sic R5-201519 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1), cs_NR_MAC_SDU_SubPDU_LI8(v_LCID3, v_EncodedRlcPdu2) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL, cs_NR_MAC_Padding_SubPDU) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id,cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); // @siclog "Step 25-26" siclog@ f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation[2].Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation[2].Lrbs)); //256bits // @siclog "Step 27" siclog@ //@sic R5-201519 sic@ v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID3, int2bit(10, 8), '0'B), v_EncodedRlcPdu2_Poll), //12B cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID, int2bit(11, 8), '0'B), v_EncodedRlcPdu1_Poll) //13B }; v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_LongBSR_SubHeader(int2bit(3, 8)),cr_LongBSR_Payload) }; //LCG 2,1 present, v_CE_SubPDUListUL2 := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload(?, ?)) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('?'O)) }; // 2 bye padding v_MAC_PDUListUL2 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, v_CE_SubPDUListUL2, cr_NR_MAC_Padding_SubPDU('????'O)) }; // 5 bye padding DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, (v_MAC_PDUListUL, v_MAC_PDUListUL2))); t_Watchdog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 27"); //@siclog "Step 28-29" siclog@ //@sic R5-201519 sic@ f_NR_RLC_Status_MAC_PDU_Tx(v_LCID3, 3, p_NR_DRB_Id); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 3, p_NR_DRB_Id); };