3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220681

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2537 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 9.1.10.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, eNS-UEConTest |  |  |  |  |  | Date: |  |  | 2022-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation, The UE is switched OFF at Step 24, the NGC Cell A is set to non-suitable cell power at Step 25, and then immediately the UE is Switched ON at Step 26. Sometimes immediately after the Power ON at Step 26, the UE sends arrcSetupREQ on NGC Cell A. This happens because the UE is Switched ON immediately after tsetting NGC Cell A to non-suitable cell, and the cell power may not yet be fully updated. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced a delay of 10s after Step 25 so that the NGC Cell A is definitely set to non-suitable cell power before the UE is Switched ON. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase not reliable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.10.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to fl_TC_9_1_10_6_TestBody	2

# Corrections required

## Correction to fl_TC_9_1_10_6_TestBody

| Function name | fl_TC_9_1_10_6_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation, The UE is switched OFF at Step 24, the NGC Cell A is set to non-suitable cell power at Step 25, and then immediately the UE is Switched ON at Step 26. Sometimes immediately after the Power ON at Step 26, the UE sends arrcSetupREQ on NGC Cell A. This happens because the UE is Switched ON immediately after tsetting NGC Cell A to non-suitable cell, and the cell power may not yet be fully updated. This needs to be addressed. |
| Summary of change | Introduced a delay of 10s after Step 25 so that the NGC Cell A is definitely set to non-suitable cell power before the UE is Switched ON. |
| TTCN module | NetworkSlicingEnh_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_3_4_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { //TB sizes 40, 80, 576, 256, for FR1 var NR_ResourceAllocation_Type v_NR_ResourceAllocation[4]; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var integer v_BSR1; var integer v_BSR2; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_MAC_SDU_Type v_EncodedRlcPdu1_Poll; var NR_MAC_SDU_Type v_EncodedRlcPdu2; var NR_MAC_SDU_Type v_EncodedRlcPdu2_Poll; var NR_MAC_SDU_Type v_EncodedRlcPdu3; var NR_MAC_SDU_Type v_EncodedRlcPdu3_Poll; var NR_MAC_SDU_Type v_EncodedRlcPdu4; var NR_MAC_SDU_Type v_EncodedRlcPdu5; var NR_MAC_SDU_Type v_EncodedRlcPdu5_Poll; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_SubFrameTimingTa; var SubFrameTiming_Type v_SubFrameTimingTb; var integer v_Duration; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL; var template (present) NR_MAC_CE_SubPDU_UL_List_Type v_CE_SubPDUListUL; var template (present) NR_MAC_CE_SubPDU_UL_List_Type v_CE_SubPDUListUL2; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL2; var integer v_SBSR; //@sic R5s200511 sic@ //@sic R5-201519 sic@ var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6);//@sic R5-201519 sic@ var B6_Type v_LCID2 := int2bit(p_NR_DRB_Id+4, 6);//@sic R5-201519 sic@ var B6_Type v_LCID3 := int2bit(p_NR_DRB_Id+5, 6);//@sic R5-201519 sic@ timer t_Watchdog := 5.0; // Stop UL Grant configuration //@siclog "Step 1" siclog@ f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@sic R5s190899 sic@ v_NR_ResourceAllocation[0] := f_NR_ResourceAllocation_UplinkGrant(40, v_NR_UplinkBWP); v_NR_ResourceAllocation[1] := f_NR_ResourceAllocation_UplinkGrant(608, v_NR_UplinkBWP); v_NR_ResourceAllocation[2] := f_NR_ResourceAllocation_UplinkGrant(256, v_NR_UplinkBWP); //@siclog "Step 2" siclog@ //3 B PDCP H + 3 Byte RLC H + 9B PDCP SDU = 12Byte RLC SDU, 15 B RLC PDU +2 Byte MAC SH =17 Byte MAC Sub PDU // 2* 17 =34 B =272 bit an allowed TBsize v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_9B))); v_EncodedRlcPdu1_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_9B))); v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_9B))); v_EncodedRlcPdu2_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_9B))); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1),//@sic R5-201519 sic@ cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu2) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 3" siclog@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20); // 20 ms later f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation[0].Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation[0].Lrbs)); //40 bits // @siclog "Step 4" siclog@ t_Watchdog.start; v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload('010'B, int2bit(4, 5))) }; //LCG 2, value =4 v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('??'O)) }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)) -> value v_NR_DRB_COMMON_IND; // 2byte Padding header and 1 byte padding payload v_SubFrameTimingTa := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5-7" siclog@ //Automatically taken by SS and duration check // @siclog "Step 8" siclog@ v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload('010'B, int2bit(4, 5))) }; //LCG 2, value =4 v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('??'O)) }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)) -> value v_NR_DRB_COMMON_IND; // 1byte Padding header and 2 byte padding payload v_SubFrameTimingTb := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; //@sic R5s200511 sic@ v_SBSR := bit2int(v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.MacPdu[0].UL.CE_SubPDUList[0].ControlElement.ShortBSR.BufferSize); if (v_SBSR >= 4) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } v_Duration := f_SubFrameTiming_Duration(v_SubFrameTimingTa, v_SubFrameTimingTb); if (f_NR_SubFrameTimingCheckDuration(nr_Cell1, l2Timer, v_Duration, 320)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } // @siclog "Step 9" siclog@ //3 B PDCP H + 3 Byte RLC H + 9B PDCP SDU = 12Byte RLC SDU, 15 B RLC PDU +2 Byte MAC SH =17 Byte =136 bit an allowed TB size v_EncodedRlcPdu3 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_9B))); v_EncodedRlcPdu3_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_9B))); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID2, v_EncodedRlcPdu3) };//@sic R5-201519 sic@ v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); // @siclog "Step 10-11" siclog@ //Automatically taken by SS // @siclog "Step 12" siclog@ v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload('010'B, int2bit(5, 5))) }; //LCG 2, value =5 v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('??'O)) }; // 1byte Padding header and 2 byte padding payload DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); //@siclog "Step 13" siclog@ //3 B PDCP H + 3 B RLC H + 2B PDCP SDU = 5Byte RLC SDU, 8 B RLC PDU +2 Byte MAC SH =10 Byte MAC Sub PDU // 2* 10 =20 B =160 bit an allowed TBsize v_EncodedRlcPdu4 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_2B))); v_EncodedRlcPdu5 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_2B))); v_EncodedRlcPdu5_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_2B))); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID3, v_EncodedRlcPdu4),//@sic R5-201519 sic@ cs_NR_MAC_SDU_SubPDU_LI8(v_LCID3, v_EncodedRlcPdu5) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 14-15" siclog@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20); // 20 ms later f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation[0].Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation[0].Lrbs)); //40 bits //@siclog "Step 16" siclog@ v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_LongBSR_SubHeader(int2bit(3, 8)), cr_LongBSR_Payload) }; //LCG 2,1 present v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL) }; //@sic R5s190552 sic@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)) -> value v_NR_DRB_COMMON_IND; v_BSR1 := oct2int(v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.MacPdu[0].UL.CE_SubPDUList[0].ControlElement.LongBSR.BufferSizeList[0]); v_BSR2 := oct2int(v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.MacPdu[0].UL.CE_SubPDUList[0].ControlElement.LongBSR.BufferSizeList[1]); v_SubFrameTimingTa := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; if ((v_BSR1 >= 1) and (v_BSR2 >= 20)) { //@sic R5-197002 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 16"); } // @siclog "Step 17-19" siclog@ f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation[1].Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation[1].Lrbs)); //576 bits //@siclog "Step 20" siclog@ //@sic R5-201519 sic@ v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID3, int2bit(8, 8), '0'B), v_EncodedRlcPdu4), //10B cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID3, int2bit(8, 8), '0'B), v_EncodedRlcPdu5_Poll), //10B cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID2, int2bit(15, 8), '0'B), v_EncodedRlcPdu3_Poll), //17B cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID, int2bit(15, 8), '0'B), v_EncodedRlcPdu1), //17B cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID, int2bit(15, 8), '0'B), v_EncodedRlcPdu2_Poll) //17B }; //v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, omit, cr_NR_MAC_Padding_SubPDU(''O)) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; //@sic R5-197002 sic@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19"); //@siclog "Step 21-23" siclog@ //@sic R5-201519 sic@ f_NR_RLC_Status_MAC_PDU_Tx(v_LCID3, 2, p_NR_DRB_Id); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID2, 1, p_NR_DRB_Id); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID , 2, p_NR_DRB_Id); //3 B PDCP H + 3 Byte RLC H + 5B PDCP SDU = 8 Byte RLC SDU, 11 B RLC PDU +2 Byte MAC SH =13 Byte //TBD if any padding needed v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_5B))); //3 B PDCP H + 3 Byte RLC H + 4B PDCP SDU = 7 Byte RLC SDU, 10 B RLC PDU +2 Byte MAC SH =12 Byte v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_4B))); //Poll PDU need to be encoded again, as at step 27, 12B & 13B PDU is required //@sic R5s190899 sic@ v_EncodedRlcPdu1_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_5B))); v_EncodedRlcPdu2_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_4B))); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL //@siclog "Step 24" siclog@ // MAC Sub PDU 1:=13 B, MAC Sub PDU 1:=12 B, + 1 B Padding =208 bits an allowed TB size //@sic R5-201519 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1), cs_NR_MAC_SDU_SubPDU_LI8(v_LCID3, v_EncodedRlcPdu2) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL, cs_NR_MAC_Padding_SubPDU) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id,cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); // @siclog "Step 25-26" siclog@ f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation[2].Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation[2].Lrbs)); //256bits // @siclog "Step 27" siclog@ //@sic R5-201519 sic@ v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID3, int2bit(10, 8), '0'B), v_EncodedRlcPdu2_Poll), //12B cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID, int2bit(11, 8), '0'B), v_EncodedRlcPdu1_Poll) //13B }; v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_LongBSR_SubHeader(int2bit(3, 8)),cr_LongBSR_Payload) }; //LCG 2,1 present, v_CE_SubPDUListUL2 := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload(?, ?)) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('?'O)) }; // 2 bye padding v_MAC_PDUListUL2 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, v_CE_SubPDUListUL2, cr_NR_MAC_Padding_SubPDU('????'O)) }; // 5 bye padding DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, (v_MAC_PDUListUL, v_MAC_PDUListUL2))); t_Watchdog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 27"); //@siclog "Step 28-29" siclog@ //@sic R5-201519 sic@ f_NR_RLC_Status_MAC_PDU_Tx(v_LCID3, 3, p_NR_DRB_Id); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 3, p_NR_DRB_Id); };

After Change:

function f_TC_7_1_1_3_4_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { //TB sizes 40, 80, 576, 256, for FR1 var NR_ResourceAllocation_Type v_NR_ResourceAllocation[4]; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var integer v_BSR1; var integer v_BSR2; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_MAC_SDU_Type v_EncodedRlcPdu1_Poll; var NR_MAC_SDU_Type v_EncodedRlcPdu2; var NR_MAC_SDU_Type v_EncodedRlcPdu2_Poll; var NR_MAC_SDU_Type v_EncodedRlcPdu3; var NR_MAC_SDU_Type v_EncodedRlcPdu3_Poll; var NR_MAC_SDU_Type v_EncodedRlcPdu4; var NR_MAC_SDU_Type v_EncodedRlcPdu5; var NR_MAC_SDU_Type v_EncodedRlcPdu5_Poll; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_SubFrameTimingTa; var SubFrameTiming_Type v_SubFrameTimingTb; var integer v_Duration; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL; var template (present) NR_MAC_CE_SubPDU_UL_List_Type v_CE_SubPDUListUL; var template (present) NR_MAC_CE_SubPDU_UL_List_Type v_CE_SubPDUListUL2; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL2; var integer v_SBSR; //@sic R5s200511 sic@ //@sic R5-201519 sic@ var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6);//@sic R5-201519 sic@ var B6_Type v_LCID2 := int2bit(p_NR_DRB_Id+4, 6);//@sic R5-201519 sic@ var B6_Type v_LCID3 := int2bit(p_NR_DRB_Id+5, 6);//@sic R5-201519 sic@ timer t_Watchdog := 5.0; // Stop UL Grant configuration //@siclog "Step 1" siclog@ f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@sic R5s190899 sic@ v_NR_ResourceAllocation[0] := f_NR_ResourceAllocation_UplinkGrant(40, v_NR_UplinkBWP); v_NR_ResourceAllocation[1] := f_NR_ResourceAllocation_UplinkGrant(608, v_NR_UplinkBWP); v_NR_ResourceAllocation[2] := f_NR_ResourceAllocation_UplinkGrant(256, v_NR_UplinkBWP); //@siclog "Step 2" siclog@ //3 B PDCP H + 3 Byte RLC H + 9B PDCP SDU = 12Byte RLC SDU, 15 B RLC PDU +2 Byte MAC SH =17 Byte MAC Sub PDU // 2* 17 =34 B =272 bit an allowed TBsize v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_9B))); v_EncodedRlcPdu1_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_9B))); v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_9B))); v_EncodedRlcPdu2_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_9B))); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1),//@sic R5-201519 sic@ cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu2) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 3" siclog@ v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20); // 20 ms later f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation[0].Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation[0].Lrbs)); //40 bits // @siclog "Step 4" siclog@ t_Watchdog.start; v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload('010'B, int2bit(4, 5))) }; //LCG 2, value =4 v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('??'O)) }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)) -> value v_NR_DRB_COMMON_IND; // 2byte Padding header and 1 byte padding payload v_SubFrameTimingTa := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5-7" siclog@ //Automatically taken by SS and duration check // @siclog "Step 8" siclog@ v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload('010'B, int2bit(4, 5))) }; //LCG 2, value =4 v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('??'O)) }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)) -> value v_NR_DRB_COMMON_IND; // 1byte Padding header and 2 byte padding payload v_SubFrameTimingTb := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; //@sic R5s200511 sic@ v_SBSR := bit2int(v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.MacPdu[0].UL.CE_SubPDUList[0].ControlElement.ShortBSR.BufferSize); if (v_SBSR >= 4) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } v_Duration := f_SubFrameTiming_Duration(v_SubFrameTimingTa, v_SubFrameTimingTb); if (f_NR_SubFrameTimingCheckDuration(nr_Cell1, l2Timer, v_Duration, 320)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } // @siclog "Step 9" siclog@ //3 B PDCP H + 3 Byte RLC H + 9B PDCP SDU = 12Byte RLC SDU, 15 B RLC PDU +2 Byte MAC SH =17 Byte =136 bit an allowed TB size v_EncodedRlcPdu3 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_9B))); v_EncodedRlcPdu3_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_9B))); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID2, v_EncodedRlcPdu3) };//@sic R5-201519 sic@ v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); // @siclog "Step 10-11" siclog@ //Automatically taken by SS // @siclog "Step 12" siclog@ v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload('010'B, int2bit(5, 5))) }; //LCG 2, value =5 v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('??'O)) }; // 1byte Padding header and 2 byte padding payload DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); //@siclog "Step 13" siclog@ //3 B PDCP H + 3 B RLC H + 2B PDCP SDU = 5Byte RLC SDU, 8 B RLC PDU +2 Byte MAC SH =10 Byte MAC Sub PDU // 2* 10 =20 B =160 bit an allowed TBsize v_EncodedRlcPdu4 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_2B))); v_EncodedRlcPdu5 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_2B))); v_EncodedRlcPdu5_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_2B))); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID3, v_EncodedRlcPdu4),//@sic R5-201519 sic@ cs_NR_MAC_SDU_SubPDU_LI8(v_LCID3, v_EncodedRlcPdu5) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); //@siclog "Step 14-15" siclog@ //WA#WI=962744 - Removed. //v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20); // 20 ms later //f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation[0].Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation[0].Lrbs)); //40 bits //@siclog "Step 16" siclog@ v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_LongBSR_SubHeader(int2bit(3, 8)), cr_LongBSR_Payload) }; //LCG 2,1 present v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(omit, v_CE_SubPDUListUL) }; //@sic R5s190552 sic@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)) -> value v_NR_DRB_COMMON_IND; v_BSR1 := oct2int(v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.MacPdu[0].UL.CE_SubPDUList[0].ControlElement.LongBSR.BufferSizeList[0]); v_BSR2 := oct2int(v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.MacPdu[0].UL.CE_SubPDUList[0].ControlElement.LongBSR.BufferSizeList[1]); v_SubFrameTimingTa := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame; if ((v_BSR1 >= 1) and (v_BSR2 >= 20)) { //@sic R5-197002 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 16"); } // @siclog "Step 17-19" siclog@ f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation[1].Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation[1].Lrbs)); //576 bits //@siclog "Step 20" siclog@ //@sic R5-201519 sic@ v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID3, int2bit(8, 8), '0'B), v_EncodedRlcPdu4), //10B cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID3, int2bit(8, 8), '0'B), v_EncodedRlcPdu5_Poll), //10B cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID2, int2bit(15, 8), '0'B), v_EncodedRlcPdu3_Poll), //17B cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID, int2bit(15, 8), '0'B), v_EncodedRlcPdu1), //17B cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID, int2bit(15, 8), '0'B), v_EncodedRlcPdu2_Poll) //17B }; //v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, omit, cr_NR_MAC_Padding_SubPDU(''O)) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, *, *) }; //@sic R5-197002 sic@ DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19"); //@siclog "Step 21-23" siclog@ //@sic R5-201519 sic@ f_NR_RLC_Status_MAC_PDU_Tx(v_LCID3, 2, p_NR_DRB_Id); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID2, 1, p_NR_DRB_Id); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID , 2, p_NR_DRB_Id); //3 B PDCP H + 3 Byte RLC H + 5B PDCP SDU = 8 Byte RLC SDU, 11 B RLC PDU +2 Byte MAC SH =13 Byte //TBD if any padding needed v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_5B))); //3 B PDCP H + 3 Byte RLC H + 4B PDCP SDU = 7 Byte RLC SDU, 10 B RLC PDU +2 Byte MAC SH =12 Byte v_EncodedRlcPdu2 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_4B))); //Poll PDU need to be encoded again, as at step 27, 12B & 13B PDU is required //@sic R5s190899 sic@ v_EncodedRlcPdu1_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_5B))); v_EncodedRlcPdu2_Poll := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_4B))); v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL //@siclog "Step 24" siclog@ // MAC Sub PDU 1:=13 B, MAC Sub PDU 1:=12 B, + 1 B Padding =208 bits an allowed TB size //@sic R5-201519 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1), cs_NR_MAC_SDU_SubPDU_LI8(v_LCID3, v_EncodedRlcPdu2) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL, cs_NR_MAC_Padding_SubPDU) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id,cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); // @siclog "Step 25-26" siclog@ f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation[2].Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation[2].Lrbs)); //256bits // @siclog "Step 27" siclog@ //@sic R5-201519 sic@ v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID3, int2bit(10, 8), '0'B), v_EncodedRlcPdu2_Poll), //12B cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(v_LCID, int2bit(11, 8), '0'B), v_EncodedRlcPdu1_Poll) //13B }; v_CE_SubPDUListUL := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_LongBSR_SubHeader(int2bit(3, 8)),cr_LongBSR_Payload) }; //LCG 2,1 present, v_CE_SubPDUListUL2 := { cr_NR_MAC_CE_SubPDU_UL(cr_NR_MAC_ShortBSR_SubHeader, cr_ShortBSR_Payload(?, ?)) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, v_CE_SubPDUListUL, cr_NR_MAC_Padding_SubPDU('?'O)) }; // 2 bye padding v_MAC_PDUListUL2 := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL, v_CE_SubPDUListUL2, cr_NR_MAC_Padding_SubPDU('????'O)) }; // 5 bye padding DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, (v_MAC_PDUListUL, v_MAC_PDUListUL2))); t_Watchdog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 27"); //@siclog "Step 28-29" siclog@ //@sic R5-201519 sic@ f_NR_RLC_Status_MAC_PDU_Tx(v_LCID3, 3, p_NR_DRB_Id); f_NR_RLC_Status_MAC_PDU_Tx(v_LCID, 3, p_NR_DRB_Id); };