3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220685

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2540 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 7.1.1.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-05-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per prose Step 9 and Step 11, UE will transmit the RLC PDU transmitted at Step 7 but at Step 11 checking the complete UL PDU(v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.MacPdu[0]) is inappropriate as NR_MAC_PDU_UL_Type will include MAC CE and padding which might change in retransmission by UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For step 11 retransmission just check the transmitted RLC PDU corresponds to Step 7. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.1.NR5GC. 7.1.1.3.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to f_TC_7_1_1_3_1_NR_TestBody1()	2

# Corrections required

## Correction to f_TC_7_1_1_3_1_NR_TestBody1()

| Function name | f_TC_7_1_1_3_1_NR_TestBody1 |
| --- | --- |
| Reason for change | As per prose Step 9 and Step 11, UE will transmit the RLC PDU transmitted at Step 7 but at Step 11 checking the complete UL PDU(v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.MacPdu[0]) is inappropriate as NR_MAC_PDU_UL_Type will include MAC CE and padding which might change in retransmission by UE. |
| Summary of change | For step 11 retransmission just check the transmitted RLC PDU corresponds to Step 7. |
| TTCN module | \ NR_TC_Common\7_1_1\ MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_3_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { const integer const_UL_TBS := 384; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var RNTI_Value_Type v_C_RNTI; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingUL1; //var SubFrameTiming_Type v_TimingUL2; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL; timer t_Watchdog := 0.6; // @siclog "Step 1" siclog@ // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); // store cell info parameters for C-RNTI v_C_RNTI := f_NR_CellInfo_GetRNTI(nr_Cell1); v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP); // @siclog "Step 2" siclog@ // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); //@sic R5s190362 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); // Timing of DL v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20); // 20 ms later v_TimingUL1:= f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 120); // 120 ms later //Timing when the UL is expected based on UL Grant on C-RNTI. The last grant with valid C-RNTI =v_TimingUL + ((cycles-1) *period)) //v_TimingUL2 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL, 400 + tsc_NR_SlotOffsetK2); t_Watchdog.start; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); // @siclog "Step 3 & 5" siclog@ // Configure SS with a different RNTI than allocated to UE f_NR_SS_C_RNTI_Config(nr_Cell1, tsc_RNTI_Value_Diff, cs_TimingInfo_NR(v_TimingUL)); // Start periodic cyclic grant f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 10, 11); // Configure SS with a same RNTI allocated to UE f_NR_SS_C_RNTI_Config(nr_Cell1, v_C_RNTI, cs_TimingInfo_NR(v_TimingUL1)); // @siclog "Step 4 & 6" siclog@ v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(?, ?) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL) }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)); //@sic R5s190362 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); t_Watchdog.stop; f_NR_RLC_Status_MAC_PDU_Tx(tsc_LCID_SCG_Bearer, 1, p_NR_DRB_Id); // Configure SS to Enforce UL retransmissions // @siclog "Steps 8,10" siclog@ f_NR_ULGrantConfiguration_Start_71131( nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation.Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs),-,-,cs_HarqProcessConfig(1)); //v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL // @siclog "Step 7" siclog@ // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B))); t_Watchdog.start; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); // @siclog "Step 9" siclog@ v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(?, ?) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL) }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)) -> value v_NR_DRB_COMMON_IND; // @siclog "Step 11" siclog@ v_MAC_PDUListUL := { v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.MacPdu[0] }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)); f_NR_RLC_Status_MAC_PDU_Tx(tsc_LCID_SCG_Bearer, 2, p_NR_DRB_Id); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); // @siclog "Step 12" siclog@ f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation,cs_HarqProcessConfig(1)); // @siclog "Step 13" siclog@ v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU_SDU_NoSubPDU }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(6144, v_NR_UplinkBWP); f_NR_ULGrantConfiguration_Start( nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation.Imcs,cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs),-,-,cs_NR_HarqProcessConfig_AnyProcess); //@sic R5s190362 R5s200010 sic@ // @siclog "Step 14" siclog@ // PDCP SDU =122B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =130B=1040 bits + 3B Padding 1064 (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_122B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL, cs_NR_MAC_Padding_SubPDU(tsc_NR_MAC_Padding_2B)) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); // @siclog "Step 15" siclog@ taken care by SS automatically // @siclog "Step 16" siclog@ v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, '????????'B, '0'B), ?) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL) }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)); f_NR_RLC_Status_MAC_PDU_Tx(tsc_LCID_SCG_Bearer, 3, p_NR_DRB_Id); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); // @siclog "Step 17" siclog@ // PDCP SDU =506B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =514B=4112 bits +14B Padding =4224 bits(an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(3, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(3, crs_NR_PDCP_SDU_506B))); //@sic R5s190889 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI16(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; //@sic R5s190362 sic@ v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL, cs_NR_MAC_Padding_SubPDU(tsc_NR_MAC_Padding_13B)) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); // @siclog "Step 18" siclog@ taken care by SS automatically // @siclog "Step 19" siclog@ v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, '????????????????'B, '1'B), ?) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL) }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19"); f_NR_RLC_Status_MAC_PDU_Tx(tsc_LCID_SCG_Bearer, 4, p_NR_DRB_Id); }

After Change:

function f_TC_7_1_1_3_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { const integer const_UL_TBS := 384; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var RNTI_Value_Type v_C_RNTI; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingUL1; //var SubFrameTiming_Type v_TimingUL2; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (present) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListUL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL; timer t_Watchdog := 0.6; // @siclog "Step 1" siclog@ // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); // store cell info parameters for C-RNTI v_C_RNTI := f_NR_CellInfo_GetRNTI(nr_Cell1); v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(const_UL_TBS, v_NR_UplinkBWP); // @siclog "Step 2" siclog@ // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); //@sic R5s190362 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); // Timing of DL v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20); // 20 ms later v_TimingUL1:= f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 120); // 120 ms later //Timing when the UL is expected based on UL Grant on C-RNTI. The last grant with valid C-RNTI =v_TimingUL + ((cycles-1) *period)) //v_TimingUL2 := f_SubFrameTiming_AddMilliSeconds(v_TimingUL, 400 + tsc_NR_SlotOffsetK2); t_Watchdog.start; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), v_MAC_PDUListDL)); // @siclog "Step 3 & 5" siclog@ // Configure SS with a different RNTI than allocated to UE f_NR_SS_C_RNTI_Config(nr_Cell1, tsc_RNTI_Value_Diff, cs_TimingInfo_NR(v_TimingUL)); // Start periodic cyclic grant f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 10, 11); // Configure SS with a same RNTI allocated to UE f_NR_SS_C_RNTI_Config(nr_Cell1, v_C_RNTI, cs_TimingInfo_NR(v_TimingUL1)); // @siclog "Step 4 & 6" siclog@ v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(?, ?) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL) }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)); //@sic R5s190362 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); t_Watchdog.stop; f_NR_RLC_Status_MAC_PDU_Tx(tsc_LCID_SCG_Bearer, 1, p_NR_DRB_Id); // Configure SS to Enforce UL retransmissions // @siclog "Steps 8,10" siclog@ f_NR_ULGrantConfiguration_Start_71131( nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation.Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs),-,-,cs_HarqProcessConfig(1)); //v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL // @siclog "Step 7" siclog@ // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B))); t_Watchdog.start; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); // @siclog "Step 9" siclog@ v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(?, ?) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL) }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)) -> value v_NR_DRB_COMMON_IND; // @siclog "Step 11" siclog@ //WA#WI=965945 -Removed it. //v_MAC_PDUListUL := { v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.MacPdu[0] }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)); f_NR_RLC_Status_MAC_PDU_Tx(tsc_LCID_SCG_Bearer, 2, p_NR_DRB_Id); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); // @siclog "Step 12" siclog@ f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation,cs_HarqProcessConfig(1)); // @siclog "Step 13" siclog@ v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU_SDU_NoSubPDU }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13"); v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(6144, v_NR_UplinkBWP); f_NR_ULGrantConfiguration_Start( nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation.Imcs,cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs),-,-,cs_NR_HarqProcessConfig_AnyProcess); //@sic R5s190362 R5s200010 sic@ // @siclog "Step 14" siclog@ // PDCP SDU =122B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =130B=1040 bits + 3B Padding 1064 (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(2, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(2, crs_NR_PDCP_SDU_122B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL, cs_NR_MAC_Padding_SubPDU(tsc_NR_MAC_Padding_2B)) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); // @siclog "Step 15" siclog@ taken care by SS automatically // @siclog "Step 16" siclog@ v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, '????????'B, '0'B), ?) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL) }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)); f_NR_RLC_Status_MAC_PDU_Tx(tsc_LCID_SCG_Bearer, 3, p_NR_DRB_Id); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); // @siclog "Step 17" siclog@ // PDCP SDU =506B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =514B=4112 bits +14B Padding =4224 bits(an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(3, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(3, crs_NR_PDCP_SDU_506B))); //@sic R5s190889 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI16(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; //@sic R5s190362 sic@ v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL, cs_NR_MAC_Padding_SubPDU(tsc_NR_MAC_Padding_13B)) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); // @siclog "Step 18" siclog@ taken care by SS automatically // @siclog "Step 19" siclog@ v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(cr_NR_MAC_SubHeaderWithLI(tsc_LCID_SCG_Bearer, '????????????????'B, '1'B), ?) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL) }; DRB.receive(car_NR_DRB_COMMON_IND_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListUL)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19"); f_NR_RLC_Status_MAC_PDU_Tx(tsc_LCID_SCG_Bearer, 4, p_NR_DRB_Id); }