3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220709

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0778 | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS Call Control test case 7.24a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To verify IMS 5GS test case 7.24a using IWD 21wk49 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS test case 7.24a required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.24a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

3	Branches executed	5

4	Execution Log Files	5

HiSilicon	5

5	References	6

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of IMS test case 7.24a which is part of the iwd-TTCN3-B2020-09_D21wk49.zip test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case:	7.24a.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D21wk49.zip

System Simulator used:	Rohde & Schwarz 5G Protocol Conformance Test platform

UE used:	Hisilicon Balong 5000 UE

Verification Status:	PASS

# Corrections required

  - Correction to function f_TC_7_24a_NR5GC_IMS1

| Function name | f_TC_7_24a_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Change 1: UE will deliver SDP Offer within PRACK after 183 Session in progress for secondDialog, code needs to check SDP message of PRACK at step 23. Change 2: SS sends 199 Early Dialog to firstDialog instead to secondDialog at step 27. Change 3: SS sends 200 OK for INVITE to secondDialog instead of firstDialog at step 28. |
| Summary of change | Change 1: Added SDP message check to received PRACK at step 22. Change 2: Activated first dialog at step 27. Change 3: Activated second dialog at step 28. |
| TTCN module | IMS\NR5GC_7\IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

function f_TC_7_24a_NR5GC_IMS1() runs on IMS_PTC

{ // MTSI MO Voice Call / Forking / UE receives two preliminary responses and one early dialog termination / 5GS

var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp;

var template (value) MessageHeader v_MessageHeader;

var template (value) SDP_Message v_SdpAnswer;

var SipUrl v_ContactUrlUE;

var charstring v_ToTag2 := f_IMS_GenerateTag("TC_7_24a-Dialog-2");

timer t_Timer;

f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED);

f_IMS_TestBody_Set(true);

// @siclog "Step 1 - 9" siclog@

f_IMS_CC_StartCall(IPCAN_MO_SpeechCall);

v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1_Step1();

if (not match(v_InviteRequestWithSdp.Invite.msgHeader.supported, cr_Supported( { "199" } ))) {

f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Supported header field does not contain '199' option tag");

}

// @siclog "Step 9 - 20" siclog@

f_IMS_MOCallSetup_NR5GC_A_4_1_Step2_10(v_InviteRequestWithSdp);

f_IMS_PTC_ImsInfo_ActivateDialog(secondDialog);

f_IMS_PTC_ImsInfo_DialogInit(dialogRemote, v_InviteRequestWithSdp.Invite.msgHeader);  // second dialog

f_IMS_PTC_ImsInfo_DialogSetLocalTag(v_ToTag2);                                        // second dialog: different To-tag

// @siclog "Step 21" siclog@                183 Session progress (dialog 2)

v_MessageHeader := f_IMS_InviteResponse_183_MessageHeaderTX(v_InviteRequestWithSdp.Invite, px_IMS_CalleeContactUri2);

v_SdpAnswer := f_IMS_BuildSDP_InitialAnswerAudio_TX(v_InviteRequestWithSdp.SdpOffer, -, EVS_16000, 65);

IMS_Server.send(cas_IMS_DATA_RSP(v_InviteRequestWithSdp.RoutingInfo, cs_Response(c_statusLine183, v_MessageHeader, cs_MessageBody_SDP(v_SdpAnswer))));

// @siclog "Step 22 - 23" siclog@

/* v_SdpOffer := f_IMS_BuildSDP_UpdatedOfferAudio_RX(v_InviteRequestWithSdp.SdpOffer, -, EVS_16000);

f_IMS_MOCallSetup_ReceivePRACK_Send200OK(v_InviteRequestWithSdp, v_MessageHeader, cr_ContentTypeSDP, v_SdpOffer);

!!!! According to the prose there is no updated SDP offer even though the 2nd remote UE does not the preconditions updated (INVITE: a=curr:qos local/remote none, 183 reponse from the 2nd UE: a=curr:qos local/remote none) !!!! */

f_IMS_MOCallSetup_ReceivePRACK_Send200OK(v_InviteRequestWithSdp, v_MessageHeader, cr_ContentTypeSDP);

// @siclog "Step 24 - 26" siclog@

f_IMS_MOCallSetup_NR5GC_A_4_1_Step8_10(v_InviteRequestWithSdp, px_IMS_CalleeContactUri2);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 25");

// @siclog "Step 27" siclog@                199 Early Dialog Terminated

v_MessageHeader := f_IMS_BaseResponse_MessageHeaderTX(v_InviteRequestWithSdp.Invite.msgHeader);

IMS_Server.send(cas_IMS_DATA_RSP(v_InviteRequestWithSdp.RoutingInfo, cs_Response(c_statusLine199, v_MessageHeader)));

// @siclog "Step 28 - 29" siclog@

f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 29");

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_TC_7_24a_NR5GC_IMS1() runs on IMS_PTC

{ // MTSI MO Voice Call / Forking / UE receives two preliminary responses and one early dialog termination / 5GS

var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp;

var template (value) MessageHeader v_MessageHeader;

var template (value) SDP_Message v_SdpAnswer;

var template (present) SDP_Message v_SdpOffer;

var SipUrl v_ContactUrlUE;

var charstring v_ToTag2 := f_IMS_GenerateTag("TC_7_24a-Dialog-2");

timer t_Timer;

f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED);

f_IMS_TestBody_Set(true);

// @siclog "Step 1 - 9" siclog@

f_IMS_CC_StartCall(IPCAN_MO_SpeechCall);

v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1_Step1();

if (not match(v_InviteRequestWithSdp.Invite.msgHeader.supported, cr_Supported( { "199" } ))) {

f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Supported header field does not contain '199' option tag");

}

// @siclog "Step 9 - 20" siclog@

f_IMS_MOCallSetup_NR5GC_A_4_1_Step2_10(v_InviteRequestWithSdp);

f_IMS_PTC_ImsInfo_ActivateDialog(secondDialog);

f_IMS_PTC_ImsInfo_DialogInit(dialogRemote, v_InviteRequestWithSdp.Invite.msgHeader);  // second dialog

f_IMS_PTC_ImsInfo_DialogSetLocalTag(v_ToTag2);                                        // second dialog: different To-tag

// @siclog "Step 21" siclog@                183 Session progress (dialog 2)

v_MessageHeader := f_IMS_InviteResponse_183_MessageHeaderTX(v_InviteRequestWithSdp.Invite, px_IMS_CalleeContactUri2);

v_SdpAnswer := f_IMS_BuildSDP_InitialAnswerAudio_TX(v_InviteRequestWithSdp.SdpOffer, -, EVS_16000, 65);

IMS_Server.send(cas_IMS_DATA_RSP(v_InviteRequestWithSdp.RoutingInfo, cs_Response(c_statusLine183, v_MessageHeader, cs_MessageBody_SDP(v_SdpAnswer))));

// @siclog "Step 22 - 23" siclog@

/* v_SdpOffer := f_IMS_BuildSDP_UpdatedOfferAudio_RX(v_InviteRequestWithSdp.SdpOffer, -, EVS_16000);

f_IMS_MOCallSetup_ReceivePRACK_Send200OK(v_InviteRequestWithSdp, v_MessageHeader, cr_ContentTypeSDP, v_SdpOffer);

!!!! According to the prose there is no updated SDP offer even though the 2nd remote UE does not the preconditions updated (INVITE: a=curr:qos local/remote none, 183 reponse from the 2nd UE: a=curr:qos local/remote none) !!!! */

v_SdpOffer := f_IMS_BuildSDP_UpdatedOfferAudio_RX(v_InviteRequestWithSdp.SdpOffer, -, EVS_16000);

f_IMS_MOCallSetup_ReceivePRACK_Send200OK(v_InviteRequestWithSdp, v_MessageHeader, cr_ContentTypeSDP,v_SdpOffer);

// @siclog "Step 24 - 26" siclog@

f_IMS_MOCallSetup_NR5GC_A_4_1_Step8_10(v_InviteRequestWithSdp, px_IMS_CalleeContactUri2);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 25");

// @siclog "Step 27" siclog@                199 Early Dialog Terminated

f_IMS_PTC_ImsInfo_ActivateDialog(firstDialog);

v_MessageHeader := f_IMS_BaseResponse_MessageHeaderTX(v_InviteRequestWithSdp.Invite.msgHeader );

IMS_Server.send(cas_IMS_DATA_RSP(v_InviteRequestWithSdp.RoutingInfo, cs_Response(c_statusLine199, v_MessageHeader)));

// @siclog "Step 28 - 29" siclog@

f_IMS_PTC_ImsInfo_ActivateDialog(secondDialog);

f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 29");

<<SKIPPED CODE>>

# Branches executed

The test case was executed on NR band n78 using NR ciphering algorithm nea1 and integrity algorithm nia1 on Rohde & Schwarz 5G Protocol Conformance Test platform with Hisilicon Balong 5000 UE

# Execution Log Files

## HiSilicon

The HiSilicon Balong 5000 UE passed this test case on Rohde & Schwarz 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7.24a_Hisilicon.log

In the log file (in .html format) the complete test case execution can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s220238: Supporting information for addition of IMS 5GS Call Control test case 7.24a |
| --- | --- |