3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220727

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2562 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_UT_ReadAllowedNSSAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In this function, the call to function f_StringSplitLines first removes the empty lines. As a result the trimmed list will now be stored at index 0, i.,e in v_ListOfLines[0]. Similarly, the call to function f_CharStringList_Remove(v_ListOfLines, ""); will also remove the empty linnes and store the trimmed list to location v_ListOfLines[0]. However the current TTCN tries to access v_ListOfLines[0] : v_CharStringList := f_StringSplit(v_ListOfLines[1], {","}); This causes a runtime error as we try to access a field that does not exist. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of calling v_CharStringList := f_StringSplit(v_ListOfLines[1], {","});, called v_CharStringList := f_StringSplit(v_ListOfLines[0], {","}); |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail this test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	Change 1	3

# Corrections required

## Change 1

| Function name | f_UT_ReadAllowedNSSAI () |
| --- | --- |
| Reason for change | In this function, the call to function f_StringSplitLines first removes the empty lines. As a result the trimmed list will now be stored at index 0, i.,e in v_ListOfLines[0]. Similarly, the call to function f_CharStringList_Remove(v_ListOfLines, ""); will also remove the empty linnes and store the trimmed list to location v_ListOfLines[0]. However the current TTCN tries to access v_ListOfLines[0] : v_CharStringList := f_StringSplit(v_ListOfLines[1], {","}); This causes a runtime error as we try to access a field that does not exist. This needs to be addressed. |
| Summary of change | Instead of calling v_CharStringList := f_StringSplit(v_ListOfLines[1], {","});, called v_CharStringList := f_StringSplit(v_ListOfLines[0], {","}); |
| TTCN module | UpperTesterFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_UT_ReadAllowedNSSAI(UT_PTC_MTC_PORT p_Port, template (omit) charstring p_PLMN_ID := omit, boolean p_Access3GPP := true) return charstring { var charstring v_ResultString := f_UT_ReadNSSAI_DynamicParams(p_Port, Default_Rejected_Configured_Allowed, p_PLMN_ID); var CharStringList_Type v_ListOfLines := f_StringSplitLines(v_ResultString, "<CR><LF>"); // a list charstrings separated by CR LF @sic R5s210431 sic@ var charstring v_Allowed := ""; var CharStringList_Type v_CharStringList; var integer i; v_ListOfLines := f_CharStringList_Remove(v_ListOfLines, ""); // remove empty lines from the list @sic R5s210431 sic@ v_CharStringList := f_StringSplit(v_ListOfLines[1], {","}); // @sic R5s210431 sic@ //WA#9.1.4.1 if (p_Access3GPP) { i := 4; } else { i := 6; } if (isvalue(v_CharStringList[i])) { // @sic R5s210431 sic@ v_Allowed := regexp(v_CharStringList[i], """(*)""", 0); } return v_Allowed; }

TAfter Change:

function f_UT_ReadAllowedNSSAI(UT_PTC_MTC_PORT p_Port, template (omit) charstring p_PLMN_ID := omit, boolean p_Access3GPP := true) return charstring { var charstring v_ResultString := f_UT_ReadNSSAI_DynamicParams(p_Port, Default_Rejected_Configured_Allowed, p_PLMN_ID); var CharStringList_Type v_ListOfLines := f_StringSplitLines(v_ResultString, "<CR><LF>"); // a list charstrings separated by CR LF @sic R5s210431 sic@ var charstring v_Allowed := ""; var CharStringList_Type v_CharStringList; var integer i; v_ListOfLines := f_CharStringList_Remove(v_ListOfLines, ""); // remove empty lines from the list @sic R5s210431 sic@ v_CharStringList := f_StringSplit(v_ListOfLines[0], {","}); // @sic R5s210431 sic@ //WA#9.1.4.1 if (p_Access3GPP) { i := 4; } else { i := 6; } if (isvalue(v_CharStringList[i])) { // @sic R5s210431 sic@ v_Allowed := regexp(v_CharStringList[i], """(*)""", 0); } return v_Allowed; }