3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220774

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2591 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for NR5GC MAC DRX test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For 7.1.1.5.3, When UL grant is restarted at the end of the test case, the SS may transmit a MAC TA PDU which can result in HARQ indication being delivered to TTCN before HARQ reporing can be disabled. - The last slot returned from the function f_NR_LastDL_Slot() is not always correct in the case of FR2. According to the current configuration, valid DL slots are 0,1,5,6,10,11...,75,76 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Disable HARQ reporting before restarting UL grant - Correct v_MaxSlot to 1 in case of kHz120 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.3.NR5GC, 7.1.1.3.4.NR5GC, 7.1.1.3.5.NR5GC, 7.1.1.3.3.ENDC, 7.1.1.3.4.ENDC, 7.1.1.3.5.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk23 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_5_3_NR5GC |
| --- | --- |
| Reason for change | When UL grant is restarted at the end of the test case, the SS may transmit a MAC TA PDU which can result in HARQ indication being delivered to TTCN before HARQ reporing can be disabled. |
| Summary of change | Disable HARQ reporting before restarting UL grant |
| TTCN module | DRX_MAC_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_5_3_NR5GC() runs on NR5GC_PTC { // DRX operation / Short cycle not configured / Parameters configured by RRC const integer tsc_LongDRXCycle := 640; const integer tsc_DrxStartOffset := 7; // in ms @sic R5-202609 sic@ var DRB_Identity v_DRBId; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var template (value) CellGroupConfig v_CellGroupConfig; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var SubFrameTiming_Type v_Timing1; //@sic R5s210594 sic@ var SubcarrierSpacing v_SCS; var SubFrameNumber_Type v_SF := 0; //Init Cell parameters f_NR5GC_Init(NR_1); ………………………………………….. f_TC_7_1_1_5_3_NR_TestBody(v_DRBId); f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config // configure SS Mac IN NORMAL TEST MODE. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); // to configure SS to stop report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); // leaving UE test state 3 v_Timing1 := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, tsc_LongDRXCycle, tsc_DrxStartOffset); f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease(tsc_NR_RRC_TI_Def), cs_TimingInfo_NR(v_Timing1)); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); };

After Change

function f_TC_7_1_1_5_3_NR5GC() runs on NR5GC_PTC { // DRX operation / Short cycle not configured / Parameters configured by RRC const integer tsc_LongDRXCycle := 640; const integer tsc_DrxStartOffset := 7; // in ms @sic R5-202609 sic@ var DRB_Identity v_DRBId; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var template (value) CellGroupConfig v_CellGroupConfig; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var SubFrameTiming_Type v_Timing1; //@sic R5s210594 sic@ var SubcarrierSpacing v_SCS; var SubFrameNumber_Type v_SF := 0; //Init Cell parameters f_NR5GC_Init(NR_1); ………………………………………………………………. f_TC_7_1_1_5_3_NR_TestBody(v_DRBId); f_NR_TestBody_Set(false); //Postamble // to configure SS to stop report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config // configure SS Mac IN NORMAL TEST MODE. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); // to configure SS to stop report reception of HARQ ACK or NACK //MOVED f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); // leaving UE test state 3 v_Timing1 := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, tsc_LongDRXCycle, tsc_DrxStartOffset); f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease(tsc_NR_RRC_TI_Def), cs_TimingInfo_NR(v_Timing1)); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); };

## Change 2

| Function name | f_NR_LastDL_Slot() |
| --- | --- |
| Reason for change | The last slot returned from this function is not always correct in the case of FR2. According to the current configuration, valid DL slots are 0,1,5,6,10,11...,75,76 |
| Summary of change | Correct v_MaxSlot to 1 in case of kHz120 |
| TTCN module | DRX_MAC_TC_Common_NR |
| MCC160 Comment |  |

Before Change

function f_NR_LastDL_Slot( NR_CellId_Type p_NR_CellId, SubFrameTiming_Type p_TimingInfo) runs on NR_BASE_PTC return SubFrameTiming_Type { // as per 38.508-1 Table 4.6.3-103: PDSCH-TimeDomainResourceAllocationList K0 =0 (Slot offset between DCI and its scheduled PDSCH)is assumed // K1 = 7,5,8 for TDD SCS15, SCS30, SCS120 var integer v_LastSlot:= 0; var NR_DuplexMode_Type v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(p_NR_CellId); var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var integer v_SlotsInSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(nr_Cell1) ; var integer v_SlotsInframe := v_SlotsInSubframe * 10; var integer v_Dl_UL_TransPeriodicity :=5; var integer v_MinSlot := 0; var integer v_MaxSlot := 5; var integer v_Slot := 0; var SubFrameTiming_Type v_TimingInfo := p_TimingInfo; if (ischosen(v_NR_DuplexMode.TDD)) { select (v_SCS) { case (kHz15) { // K1 = 7, DDDFU-DDDFU, valid DL slot 1,2,6,7 v_Dl_UL_TransPeriodicity := 5; v_Slot := v_TimingInfo.Subframe.Number; v_TimingInfo.Subframe.Number := 0; v_MinSlot := 1; v_MaxSlot := 2; } case (kHz30) { // K1 = 5, DD, DD,DD,DF,UU-DD,DD,IDD,DF,UU, valid DL slot 2,3,4,12,13,14 v_Dl_UL_TransPeriodicity := 10; v_Slot := v_TimingInfo.Subframe.Number * v_SlotsInSubframe + v_TimingInfo.Slot.SlotOffset.Numerology1; v_TimingInfo.Subframe.Number := 0; v_TimingInfo.Slot.SlotOffset.Numerology1 := 0; v_MinSlot := 2; v_MaxSlot := 4; } case (kHz120) { // K1 = 8, DDDFU DDDFU, ...., valid DL slot 0,5,10,...,75 v_Dl_UL_TransPeriodicity := 5; v_Slot := v_TimingInfo.Subframe.Number * v_SlotsInSubframe + v_TimingInfo.Slot.SlotOffset.Numerology3; v_TimingInfo.Subframe.Number := 0; v_TimingInfo.Slot.SlotOffset.Numerology3 := 0; v_MinSlot := 0; v_MaxSlot := 0; } case else {FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing");} } …………………….. }

After Change

function f_NR_LastDL_Slot( NR_CellId_Type p_NR_CellId, SubFrameTiming_Type p_TimingInfo) runs on NR_BASE_PTC return SubFrameTiming_Type { // as per 38.508-1 Table 4.6.3-103: PDSCH-TimeDomainResourceAllocationList K0 =0 (Slot offset between DCI and its scheduled PDSCH)is assumed // K1 = 7,5,8 for TDD SCS15, SCS30, SCS120 var integer v_LastSlot:= 0; var NR_DuplexMode_Type v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(p_NR_CellId); var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); var integer v_SlotsInSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(nr_Cell1) ; var integer v_SlotsInframe := v_SlotsInSubframe * 10; var integer v_Dl_UL_TransPeriodicity :=5; var integer v_MinSlot := 0; var integer v_MaxSlot := 5; var integer v_Slot := 0; var SubFrameTiming_Type v_TimingInfo := p_TimingInfo; if (ischosen(v_NR_DuplexMode.TDD)) { select (v_SCS) { case (kHz15) { // K1 = 7, DDDFU-DDDFU, valid DL slot 1,2,6,7 v_Dl_UL_TransPeriodicity := 5; v_Slot := v_TimingInfo.Subframe.Number; v_TimingInfo.Subframe.Number := 0; v_MinSlot := 1; v_MaxSlot := 2; } case (kHz30) { // K1 = 5, DD, DD,DD,DF,UU-DD,DD,IDD,DF,UU, valid DL slot 2,3,4,12,13,14 v_Dl_UL_TransPeriodicity := 10; v_Slot := v_TimingInfo.Subframe.Number * v_SlotsInSubframe + v_TimingInfo.Slot.SlotOffset.Numerology1; v_TimingInfo.Subframe.Number := 0; v_TimingInfo.Slot.SlotOffset.Numerology1 := 0; v_MinSlot := 2; v_MaxSlot := 4; } case (kHz120) { // K1 = 8, DDDFU DDDFU, ...., valid DL slot 0,1,5,6,10,11...,75,76 v_Dl_UL_TransPeriodicity := 5; v_Slot := v_TimingInfo.Subframe.Number * v_SlotsInSubframe + v_TimingInfo.Slot.SlotOffset.Numerology3; v_TimingInfo.Subframe.Number := 0; v_TimingInfo.Slot.SlotOffset.Numerology3 := 0; v_MinSlot := 0; v_MaxSlot := 1; } case else {FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing");} } ……………………………. }