3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220790

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2604 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech function activates dedicated EPS bearer yet it returns identity of the default bearer. Test bodies of the concerned tests, however, rely on the identity being the one of dedicated bearer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech now returns identity of the dedicated instead of default bearer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Affected tests attempt to deactivate wrong bearer identity and conformant UEs behave unexpectedly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1.NR5GC through 11.1.9.NR5GC, 11.4.11.NR5GC and 11.4.12.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech |
| --- | --- |
| Reason for change | The f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech function activates dedicated EPS bearer yet it returns identity of the default bearer. Test bodies of the concerned tests, however, rely on the identity being the one of dedicated bearer. |
| Summary of change | f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech now returns identity of the dedicated instead of default bearer. |
| TTCN module | EUTRA38_NR5GC_InitialRegistration.ttcn |
| MCC160 Comment |  |

Before Change

function f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(EUTRA_CellId_Type p_CellId, PDU_PDN_DNN_Type p_DNNType := IMS_DNN, boolean p_IsSRVCC := false) runs on EUTRA_Multi5GS_PTC return integer { var integer v_DefaultBearerId := valueof(f_Get_EPSBearerIdForAPNType(p_DNNType)); var PDN_Index_Type v_PdnIndex := f_GetMultiPdnIndex (p_DNNType); f_EUTRA38_MultiPDN_ActivateDedicated(p_CellId, tsc_RRC_TI_Def, int2hex(v_DefaultBearerId, 1), 0, 1, IPCAN_SpeechCall, v_PdnIndex, p_IsSRVCC); return v_DefaultBearerId; // @sic R5s220448 sic@ }

After Change

function f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(EUTRA_CellId_Type p_CellId, PDU_PDN_DNN_Type p_DNNType := IMS_DNN, boolean p_IsSRVCC := false) runs on EUTRA_Multi5GS_PTC return integer { var integer v_DefaultBearerId := valueof(f_Get_EPSBearerIdForAPNType(p_DNNType)); var PDN_Index_Type v_PdnIndex := f_GetMultiPdnIndex (p_DNNType); f_EUTRA38_MultiPDN_ActivateDedicated(p_CellId, tsc_RRC_TI_Def, int2hex(v_DefaultBearerId, 1), 0, 1, IPCAN_SpeechCall, v_PdnIndex, p_IsSRVCC); return v_DefaultBearerId+1; // @sic R5s220448 sic@ }