3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220836

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0844 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_IMS_Route_ReverseOrderOfRecordRouteRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | With the implementation of R5-223453 a new condition for A.2.3 has been introduced: A15 - 183 sent for re-INVITE within an established dialog. This affects Record-Route which will not not be present in case A1 and A15 conditions are met. This implies that a PRACK response to a 183 Session in Progress will not include “Route” header as it should populate field with reverse Record-Route from 183 Session in progress. This can make provoke runtime error while calling following function and condition A1 and A15 are met. In this case, v_RouteBodyList being omit will provoke a runtime error at following statement return cr_Route(v_RouteBodyList); as v_RouteBodyList is a mandatory field and cannot resolve to omit. In this case Route should resolve to omit as a whole not just the contents. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected returned value for Route in case that v_RouteBodyList is omit. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Test case meeting conditions A1 and A15 in Annex A.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	3

2.1	Correction to function fl_IMS_Route_ReverseOrderOfRecordRouteRX	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to function fl_IMS_Route_ReverseOrderOfRecordRouteRX

| Function name | fl_IMS_Route_ReverseOrderOfRecordRouteRX |
| --- | --- |
| Reason for change | v_RouteBodyList being omit will provoke a runtime error at following statement return cr_Route(v_RouteBodyList); as v_RouteBodyList is a mandatory field and cannot resolve to omit. |
| Summary of change | Corrected returned value for Route in case that v_RouteBodyList is omit. |
| TTCN module | Common\IMS\IMS_Procedures_CallControlCommon.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function fl_IMS_Route_ReverseOrderOfRecordRouteRX(template (omit) RecordRoute p_RecordRoute) return template (present) Route { var template RouteBody_List v_RouteBodyList := *; if (isvalue(p_RecordRoute)) { v_RouteBodyList := f_IMS_RouteSet_ReverseOrder(valueof(p_RecordRoute.routeBody)); } return cr_Route(v_RouteBodyList); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function fl_IMS_Route_ReverseOrderOfRecordRouteRX(template (omit) RecordRoute p_RecordRoute) return template Route { var template RouteBody_List v_RouteBodyList := *; if (isvalue(p_RecordRoute)) { v_RouteBodyList := f_IMS_RouteSet_ReverseOrder(valueof(p_RecordRoute.routeBody)); return cr_Route(v_RouteBodyList); } return omit; } <<SKIPPED CODE>>