3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220862

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0850 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS Emergency Call test case 10.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To verify IMS 5GS test case 10.13 using IWD 22wk23 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS test case 10.13 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.13.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 34.229-5 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | A prose CR will be submitted to make SS release the call at step 16-17 as the rest of IMS emergency test cases. In current ttcn-3 implementation SS releases the call. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Correction to function f_TC_10_13_NR5GC_IMS2	3

3	Branches executed	5

4	Execution Log Files	5

5	References	5

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of IMS test case 10.13 which is part of the iwd-TTCN3-B2020-09_D22wk23.zip test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case:	10.13.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D22wk23.zip

System Simulator used:	Rohde & Schwarz 5G Protocol Conformance Test platform

UE used:	Samsung SM-G996U

Verification Status:	PASS

# Corrections required

## Correction to function f_TC_10_13_NR5GC_IMS2

| Function name | f_TC_10_13_NR5GC_IMS2 |
| --- | --- |
| Reason for change | Coordination messages required to reserve resources to establish an IMS call as well to inform IMS2 component that PDU session modification has been completed are missing. |
| Summary of change | Added coordination messages required for a successful IMS emergency call. |
| TTCN module | IMS\NR5GC_10\IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> // Test Case Body f_IMS_TestBody_Set(true); f_IMS_IMS_WaitForTrigger(OtherPDN); //@siclog "Step 1" siclog@ //UE is made to initiate an emergency call f_IMS_CC_StartCall(IPCAN_EmergencyCall_NoRegistration); //@siclog "Steps 2-5" siclog@ //Check: Does the UE correctly initiate emergency registration ? (Steps 1-3 of Annex A.3) //SS responds 200 OK to finish the registration. (Note: the SS sets the expiration time to 30 seconds.) (Step 4 of Annex A.3) f_IMS_EmergencyRegistration(-, int2str(v_Expiry)); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); t_RegExpireMax.start; //@siclog "Step 6-8" siclog@ <<SKIPPED CODE>> //@siclog "Step 15" siclog@ // Response from UE to confirm the dialog (Step 5 of annex A.6) v_MessageHeader_Ack := f_IMS_AckRequest_MessageHeaderRX(v_InviteRequest, c_statusLine200, v_EmergencyType); // @sic R5-145732 sic@ @sic R5s141335 sic@ @sic R5s150692 change 25 sic@ v_IMS_DATA_REQ := f_IMS_ReceiveRequest(car_IMS_Ack_Request(cr_ACK_Request(v_CalleeContactSipUri, v_MessageHeader_Ack))); // @sic R5s150195 sic@ //@siclog "Step 16" siclog@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> // Test Case Body f_IMS_TestBody_Set(true); f_IMS_IMS_WaitForTrigger(OtherPDN); //@siclog "Step 1" siclog@ //UE is made to initiate an emergency call f_IMS_CC_StartCall(IPCAN_EmergencyCall_NoRegistration); //@siclog "Steps 2-5" siclog@ //Check: Does the UE correctly initiate emergency registration ? (Steps 1-3 of Annex A.3) //SS responds 200 OK to finish the registration. (Note: the SS sets the expiration time to 30 seconds.) (Step 4 of Annex A.3) f_IMS_EmergencyRegistration(-, int2str(v_Expiry)); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); t_RegExpireMax.start; //Session modification start f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //@siclog "Step 6-8" siclog@ <<SKIPPED CODE>> //@siclog "Step 15" siclog@ // Response from UE to confirm the dialog (Step 5 of annex A.6) v_MessageHeader_Ack := f_IMS_AckRequest_MessageHeaderRX(v_InviteRequest, c_statusLine200, v_EmergencyType); // @sic R5-145732 sic@ @sic R5s141335 sic@ @sic R5s150692 change 25 sic@ v_IMS_DATA_REQ := f_IMS_ReceiveRequest(car_IMS_Ack_Request(cr_ACK_Request(v_CalleeContactSipUri, v_MessageHeader_Ack))); // @sic R5s150195 sic@ //Session modification complete f_IMS_IPCAN_WaitForTrigger(IPCAN); //@siclog "Step 16" siclog@ <<SKIPPED CODE>>

# Branches executed

The test case was executed on NR5GC band n41 with NEA1/NIA1.

# Execution Log Files

Samsung

The Samsung SM-G996U UE passed this test case on Rohde & Schwarz 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_10.13_Samsung.log

In the log file (in .html format) the complete test case execution can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

- PICS/PIXIT parameter file:

TTCNParameter.rspp

# References

| [1] | R5s220863: Supporting information for Addition of IMS 5GS Emergency Call test case 10.13 |
| --- | --- |