3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220876

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0852 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for EUTRA IMS emergency call test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2022-08-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The condition added by the MCC implementation of TTCN CR R5s220531 is bracketed incorrectly. With the current implementation, if the test procedure is IPCAN_EmergencyCall_NormalService, then the additional trigger will be sent even if f_IMS_PTC_RanTypeIsNR5GC() retuns false. This affects EUTRA IMS emergency call test cases in normal service. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the bracketing such that the condition will only be fulfilled if _IMS_PTC_RanTypeIsNR5GC() retuns true. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.3.3, 19.3.4, 19.5.6, 19.5.7, 19.5.9 and others |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	Change 1	3

Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_IMS_CC_Postamble() |
| --- | --- |
| Reason for change | The condition added by the MCC implementation of TTCN CR R5s220531 is bracketed incorrectly. With the current implementation, if the test procedure is IPCAN_EmergencyCall_NormalService, then the additional trigger will be sent even if f_IMS_PTC_RanTypeIsNR5GC() retuns false. This affects EUTRA IMS emergency call test cases in normal service. |
| Summary of change | Correct the bracketing such that the condition will only be fulfilled if _IMS_PTC_RanTypeIsNR5GC() retuns true. |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment |  |

Before Change

function f_IMS_CC_Postamble(template (omit) IMS_TestProcedure_Type p_TestProcedure := omit, template (omit) IMS_TCP_CloseHandling_Type p_TcpCloseHandling := omit, float p_DelayForDL_SIP := 2.0) runs on IMS_PTC { /* @sic R5s140801: new parameter p_TcpCloseHandling, R5s170028: removed parameter p_WaitForDeRegistration sic@ */ /* @sic R5s190785: new parameter p_DelayForDL_SIP sic@ */ var boolean v_AllowRetransmission := f_IMS_PTC_RanTypeIsUTRAN(); var float v_DelayForDL_SIP := p_DelayForDL_SIP; if (isvalue(p_TestProcedure)) { if (valueof(p_TestProcedure) == IPCAN_EmergencyCall_NormalService or valueof(p_TestProcedure) == IPCAN_EmergencyCall_NoRegistration and f_IMS_PTC_RanTypeIsNR5GC()) { // @sic R5s220531 sic@ // the IMS messages have already been sent, so tell IPCAN not to wait for it IPCAN.send(cms_IMS_IPCAN_Trigger); } f_IMS_CC_ReleaseConnection(valueof(p_TestProcedure), p_TcpCloseHandling, v_DelayForDL_SIP); /* @sic R5s140801: p_TcpCloseHandling, R5s190785 ch. 2.2 sic@ */ IPCAN.send(cms_IMS_IPCAN_ReleaseReq); if (valueof(p_TestProcedure) != IPCAN_Deregistration) { // @sic R5s211495 sic@ f_IMS_IpcanReleaseWithOptionalDeregistration(v_AllowRetransmission); /* @sic R5s140346 - MCC160 implementation: new function f_IMS_CC_IpcanReleaseWithOptionalDeregistration, R5s170541 sic@ */ } } else { // @sic R5s211495 sic@ IPCAN.send(cms_IMS_IPCAN_ReleaseReq); f_IMS_IpcanReleaseWithOptionalDeregistration(v_AllowRetransmission); /* @sic R5s140346 - MCC160 implementation: new function f_IMS_CC_IpcanReleaseWithOptionalDeregistration, R5s170541 sic@ */ } }

After Change

function f_IMS_CC_Postamble(template (omit) IMS_TestProcedure_Type p_TestProcedure := omit, template (omit) IMS_TCP_CloseHandling_Type p_TcpCloseHandling := omit, float p_DelayForDL_SIP := 2.0) runs on IMS_PTC { /* @sic R5s140801: new parameter p_TcpCloseHandling, R5s170028: removed parameter p_WaitForDeRegistration sic@ */ /* @sic R5s190785: new parameter p_DelayForDL_SIP sic@ */ var boolean v_AllowRetransmission := f_IMS_PTC_RanTypeIsUTRAN(); var float v_DelayForDL_SIP := p_DelayForDL_SIP; if (isvalue(p_TestProcedure)) { if ((valueof(p_TestProcedure) == IPCAN_EmergencyCall_NormalService or valueof(p_TestProcedure) == IPCAN_EmergencyCall_NoRegistration) and f_IMS_PTC_RanTypeIsNR5GC()) { // @sic R5s220531 sic@ // the IMS messages have already been sent, so tell IPCAN not to wait for it IPCAN.send(cms_IMS_IPCAN_Trigger); } f_IMS_CC_ReleaseConnection(valueof(p_TestProcedure), p_TcpCloseHandling, v_DelayForDL_SIP); /* @sic R5s140801: p_TcpCloseHandling, R5s190785 ch. 2.2 sic@ */ IPCAN.send(cms_IMS_IPCAN_ReleaseReq); if (valueof(p_TestProcedure) != IPCAN_Deregistration) { // @sic R5s211495 sic@ f_IMS_IpcanReleaseWithOptionalDeregistration(v_AllowRetransmission); /* @sic R5s140346 - MCC160 implementation: new function f_IMS_CC_IpcanReleaseWithOptionalDeregistration, R5s170541 sic@ */ } } else { // @sic R5s211495 sic@ IPCAN.send(cms_IMS_IPCAN_ReleaseReq); f_IMS_IpcanReleaseWithOptionalDeregistration(v_AllowRetransmission); /* @sic R5s140346 - MCC160 implementation: new function f_IMS_CC_IpcanReleaseWithOptionalDeregistration, R5s170541 sic@ */ } }