3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220909

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2559 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC PS data off test case 11.6.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-05-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NR5GC FR1 : Addition of NR5GC PS data off test case 11.6.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 11.6.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.6.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1 Change 1	3

2.1 Change 2	4

2.1 Change 3	6

3	Branches executed	7

4	Execution Log Files	7

4.1	Qualcomm SM 8450+SDX65	7

5	References	8

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.6.1 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	11.6.1

ATS Version:	iwd-TTCN3-B2020-09_D22wk12

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm SM 8450+SDX65

Verification Status:	PASS

# Corrections required

## 2.1 Change 1

| Function name | f_TC_11_6_1_NR5GC_IMS1 |
| --- | --- |
| Reason for change | A coordination trigger is missing from the IMS PTC to the NR5GC PTC to trigger the PDU SESSION MODIFICATION procedure to add voice to the call |
| Summary of change | Added a coordination send trigger |
| TTCN module | PS_Data_OFF_NR5GC_IMS.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_11_6_1_NR5GC_IMS1()runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; fl_TC_11_6_x_Steps5and6_NR5GC_IMS1(); //@siclog "Steps 8-25" siclog@ v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1a_Step1(); //100 Trying, 183 Session Progress, PRACK, 200 OK f_IMS_MOCallSetup_NR5GC_A_4_1a_Step2_5(v_InviteRequestWithSdp); … … … }

After Change

function f_TC_11_6_1_NR5GC_IMS1()runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; fl_TC_11_6_x_Steps5and6_NR5GC_IMS1(); //@siclog "Steps 8-25" siclog@ v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1a_Step1(); //100 Trying, 183 Session Progress, PRACK, 200 OK f_IMS_MOCallSetup_NR5GC_A_4_1a_Step2_5(v_InviteRequestWithSdp); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //UPDATE, 200 OK f_IMS_MOCallSetup_NR5GC_A_4_1a_Step6_7(v_InviteRequestWithSdp); … … … }

## 2.1 Change 2

| Function name | fl_TC_11_6_1_Body |
| --- | --- |
| Reason for change | - The UT trigger to dial the call, and the steps to perform the PDU SESSION MODIFICATION procedure to add voide to the call are missing - The function fl_TC_11_6_1_Body is used for TC 11.6.2 as well, where it is required to wait for the coordination trigger before performing the Postamble. In testcase 11.6.1, however, there are additional steps to add voice to the call need to be performed, and hece the coordination message here will fail the testcase before the the NAS procedure is complete. |
| Summary of change | - Added the UT trigger to dial the call, and added the steps for PDU SESSION MODIFCATION procedure - Added a Boolean flag in the function signature based on which the coordination trigger will be handled. |
| TTCN module | PS_Data_OFF_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_11_6_1_Body() runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; //Main test body fl_TC_11_6_x_Body(); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8 - 25"); //Now release the call f_NR5GC_CallRelease(nr_Cell1, IPCAN_SpeechCall); } //End of test body of TC_11_6_1

After Change

function fl_TC_11_6_1_Body() runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; //Main test body fl_TC_11_6_x_Body(false); f_UT_RequestIMSCall(UT); v_GSM_MobilityInfo:= f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_NR5GC_ModifyPDUSession_AddVoice(nr_Cell1, valueof(v_GSM_MobilityInfo)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Established"); // Call now established f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8 - 25"); //Now release the call f_NR5GC_CallRelease(nr_Cell1, IPCAN_SpeechCall); } //End of test body of TC_11_6_1

## 2.1 Change 3

| Function name | fl_TC_11_6_x_Body |
| --- | --- |
| Reason for change | The function fl_TC_11_6_1_Body is used for TC 11.6.2 as well, where it is required to wait for the coordination trigger before performing the Postamble. In testcase 11.6.1, however, there are additional steps to add voice to the call need to be performed, and hece the coordination message here will fail the testcase before the the NAS procedure is complete. |
| Summary of change | Add parameter to control additional coordination required for 11.6.2 only |
| TTCN module | PS_Data_OFF_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_11_6_x_Body() runs on NR5GC_PTC { … … … //@siclog "Steps 4" siclog@ //receive PDU Session Modification Complete SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToModify)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } //Postamble after receiving coordination from IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); } //End of test body of TC_11_6_x

After Change

function fl_TC_11_6_x_Body(boolean p_WaitForTrigger := true) runs on NR5GC_PTC { … … … //@siclog "Steps 4" siclog@ //receive PDU Session Modification Complete SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToModify)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } if(p_WaitForTrigger){ //Postamble after receiving coordination from IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); } } //End of test body of TC_11_6_x

# Branches executed

The test case was executed on NR band n41 using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## Qualcomm SM 8450+SDX65

The Qualcomm SM 8450+SDX65 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_11_6_1_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_11_6_1_PIXIT.xml

# References

| [1] | R5s220700 : NR5GC FR1 : Supporting information for addition of NR5GC PS data off test case 11.6.1 |
| --- | --- |