3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220912

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2567 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GMM test case 9.1.6.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For a UE operating with IMS enabled, when switched on at step 13a1 the current TTCN implementation may not correctly handle the IMS registraion signalling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use function f_NR5GC_SwitchOnAndResetIMS() instead of f_UT_SwitchOnUE() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.6.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_9_1_6_1_1_TestBody() |
| --- | --- |
| Reason for change | For a UE operating with IMS enabled, when switched on at step 13a1 the current TTCN implementation may not correctly handle the IMS registraion signalling. |
| Summary of change | Use function f_NR5GC_SwitchOnAndResetIMS() instead of f_UT_SwitchOnUE() |
| TTCN module | Deregistration_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_9_1_6_1_1_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SRB_COMMON_IND v_ReceivedAsp; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; //@sic R5-192397 sic@ timer t_T3522 := f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 6.0); timer t_T3570 := f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 6.0); /**** Skiping code ***********/ //EXCEPTION: Steps 13a1 to 13a4 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if the UE supports remove USIM without power down: pc_USIM_Removal = TRUE [29] if (pc_USIM_Removal) { // The UE is switched on. //@siclog "Step 13a1" siclog@ f_UT_SwitchOnUE(UT); //The UE performs Registration procedure as specified in TS 38.508-1 [4] subclause 4.5.2 with 'connected without release'. //@siglog "step13a2" siclog@ f_NR5GC_RRC_IdleState1N_Def(v_NGC_NASCellA, -, noRrcConnectionRelease); //Cause removal of USIM from the UE without powering down. //@siglog "step13a3" siclog@ f_UT_USIM_Remove (UT); /**** Skiping code ***********/ }

After Change:

function fl_TC_9_1_6_1_1_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SRB_COMMON_IND v_ReceivedAsp; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; //@sic R5-192397 sic@ timer t_T3522 := f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 6.0); timer t_T3570 := f_NR_SetTimerToleranceMin(v_NGC_NASCellA, nasTimer, 6.0); ………………………………… //EXCEPTION: Steps 13a1 to 13a4 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that take place if the UE supports remove USIM without power down: pc_USIM_Removal = TRUE [29] if (pc_USIM_Removal) { // The UE is switched on. //@siclog "Step 13a1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //The UE performs Registration procedure as specified in TS 38.508-1 [4] subclause 4.5.2 with 'connected without release'. //@siglog "step13a2" siclog@ f_NR5GC_RRC_IdleState1N_Def(v_NGC_NASCellA, -, noRrcConnectionRelease); //Cause removal of USIM from the UE without powering down. //@siglog "step13a3" siclog@ f_UT_USIM_Remove (UT); ……………………………. }