3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220925

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2663 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR test case 7.1.1.1.1a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per Table 7.1.1.1.1a.3.3-6: RACH-ConfigGeneric, preambleTransMax is set to n4 and as per TP2 test Step 2 to 5 expects four Rach attempts from UE. Here, if SS receiver miss any initial Rach attempt from UE due to the rach power falling outside its reception window which in turn also depends upon path loss( as a result of cable loss variation depending upon different band cabling connections requirement etc.), then it has been observed that current preambleReceivedTargetPower -118 dBm(TS 38.508-1, Table 4.6.3-130) is not very appropriate for this case to be used further and UE sends scgFailure(randomAccessProblem) once its 4 rach attempt gets exhausted. Therefore, in order to increase the test case relialibilty a high preambleReceivedTargetPower of -104 dBm is more suitable here. Simliar power is being used in 7.1.1.1.2 test prose as a reference. NOTE: A Prose CR shall be raised at the next RAN5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set preambleReceivedTargetPower as -104 dBm |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.1a.NR5GC, 7.1.1.1.1a.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to f_TC_7_1_1_1_1a_NR5GC	2

2.2	Correction to f_TC_7_1_1_1_1a_ENDC_NR	10

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to f_TC_7_1_1_1_1a_NR5GC

| Function name | f_TC_7_1_1_1_1a_NR5GC |
| --- | --- |
| Reason for change | As per Table 7.1.1.1.1a.3.3-6: RACH-ConfigGeneric, preambleTransMax is set to n4 and as per TP2 test Step 2 to 5 expects four Rach attempts from UE. Here, if SS receiver miss any initial Rach attempt from UE due to the rach power falling outside its reception window which in turn also depends upon path loss( as a result of cable loss variation depending upon different band cabling connections requirement etc.), then it has been observed that current preambleReceivedTargetPower -118 dBm(TS 38.508-1, Table 4.6.3-130) is not very appropriate for this case to be used further and UE sends scgFailure(randomAccessProblem) once its 4 rach attempt gets exhausted. Therefore, in order to increase the test case relialibilty a high preambleReceivedTargetPower of -104 dBm is more suitable here. Simliar power is being used in 7.1.1.1.2 test prose as a reference. NOTE: A Prose CR shall be raised at the next RAN5. |
| Summary of change | Set preambleReceivedTargetPower as -104 dBm |
| TTCN module | \NR5GC\7_1_1\MAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_1_1a_NR5GC() runs on NR5GC_PTC { //Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by PDCCH Order / contention free random access procedure var template (value) CellGroupConfig v_CellGroupConfig; var template(value) SpCellConfig v_SpCellConfig ; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; //@sic R5-198888 sic@ var NR_DuplexMode_Type v_NR_DuplexMode ; var template (value) TDD_UL_DL_ConfigCommon v_TDD_UL_DL_ConfigCommon; var NR_Prach_RootSequenceIndex_Type v_Prach_RootSequenceIndex; var boolean v_IsFR1 := false; var SubcarrierSpacing v_SubcarrierSpacing; //@sic R5s200526 sic@ var NR_UplinkBWP_Type v_NR_UplinkBWP; //@sic R5s211670 sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters; var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure; //Init Cell parameters f_NR5GC_Init(NR_1); //@sic R5s190762 sic@ f_NR_CellInfo_SetRACH_ConfigCommon_71111a(nr_Cell1); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); //Bring UE to Stat 3A f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, -, -, -, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, - ); f_NR_TestBody_Set(true); //@siclog "Steps 0A-0B" siclog@ //@sic R5-198888 R5s200891 sic@ v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); if (v_IsFR1) //FR1 { v_Prach_RootSequenceIndex := {l839 := 0}; f_NR_CellInfo_SetSIB1_Prach_RootSequenceIndex(nr_Cell1, v_Prach_RootSequenceIndex); f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell1,14); f_NR_CellInfo_SetZeroCorrelationZoneConfig(nr_Cell1,12); } //@sic R5-198888 R5s200526 sic@ if (ischosen(v_NR_DuplexMode.TDD)) { if(v_IsFR1) { if(v_SubcarrierSpacing == kHz30){ v_TDD_UL_DL_ConfigCommon :=cs_TDD_UL_DL_ConfigCommonPattern2(v_SubcarrierSpacing,cs_TDD_UL_DL_Pattern1_FR1,cs_TDD_UL_DL_Pattern2_FR1); }else if(v_SubcarrierSpacing == kHz15){ v_TDD_UL_DL_ConfigCommon :=cs_TDD_UL_DL_ConfigCommonPattern2(v_SubcarrierSpacing,cs_TDD_UL_DL_Pattern1_FR1_SCS15, cs_TDD_UL_DL_Pattern2_FR1_SCS15); }else{ FatalError(__FILE__, __LINE__, "SCS value not handled in TTCN"); } } else { v_TDD_UL_DL_ConfigCommon :=cs_TDD_UL_DL_ConfigCommonPattern2(v_SubcarrierSpacing,cs_TDD_UL_DL_Pattern1_FR2,cs_TDD_UL_DL_Pattern2_FR2); } f_NR_CellInfo_SetTDD_UL_DL_ConfigCommon(nr_Cell1,v_TDD_UL_DL_ConfigCommon); }; //@sic R5s210088 sic@ v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); v_NR_UplinkBWP.Dedicated.R15.pucch_Config.setup.schedulingRequestResourceToAddModList[0] := valueof(cs_SchedulingRequestResourceConfig_Mac_71111a(v_SubcarrierSpacing)); f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP, tsc_NR_BWP_Id); //@sic r5s201469 sic@ f_NR_SS_ConfigureSysinfo(nr_Cell1, -, f_NR_InitialiseBcchToPdschConfig(nr_Cell1)); //f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, {v_NR_UplinkBWP})); v_SpCellConfig := f_NR_GetReconfigurationWithSyncDef_MCG(nr_Cell1); //@sic R5s190511 sic@ v_SpCellConfig.reconfigurationWithSync.t304 := ms2000; v_SpCellConfig.reconfigurationWithSync.rach_ConfigDedicated := omit; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric.preambleTransMax := n4; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); //Configure SS CFRA @sic R5s200526 sic@ //v_RACH_ConfigDedicated := cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(f_NR_CellInfo_GetRACH_ConfigGeneric(nr_Cell1,tsc_NR_BWP_Id), tsc_PreambleIndex,f_NR_CellInfo_GetSSB_Index(nr_Cell1))); /*v_RACH_ConfigDedicated := cs_NR_RACH_ConfigDedicated_Uplink(f_NR_GetRACHConfigDedicated_Def( nr_Cell1, tsc_PreambleIndex)); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell1, cs_TimingInfo_Now, cs_NR_CellConfigPhysicalLayerUplink_Def(f_ConvertRACH_ConfigDedicated(v_RACH_ConfigDedicated.uplink)))); */ //@sic R5s211670 sic@ v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); // Send message to UE f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,-,false); //@sic R5s200891 sic@ if (ischosen(v_NR_DuplexMode.TDD)) {//@sic R5s211670 sic@ f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_Common_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, {v_NR_UplinkBWP},v_TDD_UL_DL_ConfigCommon)); }else { f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, {v_NR_UplinkBWP})); }; SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)); f_TC_7_1_1_1_1a_NR_TestBody(); f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_1_1a_NR5GC() runs on NR5GC_PTC { //Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by PDCCH Order / contention free random access procedure var template (value) CellGroupConfig v_CellGroupConfig; var template(value) SpCellConfig v_SpCellConfig ; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; //@sic R5-198888 sic@ var NR_DuplexMode_Type v_NR_DuplexMode ; var template (value) TDD_UL_DL_ConfigCommon v_TDD_UL_DL_ConfigCommon; var NR_Prach_RootSequenceIndex_Type v_Prach_RootSequenceIndex; var boolean v_IsFR1 := false; var SubcarrierSpacing v_SubcarrierSpacing; //@sic R5s200526 sic@ var NR_UplinkBWP_Type v_NR_UplinkBWP; //@sic R5s211670 sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters; var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure; //Init Cell parameters f_NR5GC_Init(NR_1); //@sic R5s190762 sic@ f_NR_CellInfo_SetRACH_ConfigCommon_71111a(nr_Cell1); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); //Bring UE to Stat 3A f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, -, -, -, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, - ); f_NR_TestBody_Set(true); //@siclog "Steps 0A-0B" siclog@ //@sic R5-198888 R5s200891 sic@ v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); if (v_IsFR1) //FR1 { v_Prach_RootSequenceIndex := {l839 := 0}; f_NR_CellInfo_SetSIB1_Prach_RootSequenceIndex(nr_Cell1, v_Prach_RootSequenceIndex); f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell1,14); f_NR_CellInfo_SetZeroCorrelationZoneConfig(nr_Cell1,12); } //@sic R5-198888 R5s200526 sic@ if (ischosen(v_NR_DuplexMode.TDD)) { if(v_IsFR1) { if(v_SubcarrierSpacing == kHz30){ v_TDD_UL_DL_ConfigCommon :=cs_TDD_UL_DL_ConfigCommonPattern2(v_SubcarrierSpacing,cs_TDD_UL_DL_Pattern1_FR1,cs_TDD_UL_DL_Pattern2_FR1); }else if(v_SubcarrierSpacing == kHz15){ v_TDD_UL_DL_ConfigCommon :=cs_TDD_UL_DL_ConfigCommonPattern2(v_SubcarrierSpacing,cs_TDD_UL_DL_Pattern1_FR1_SCS15, cs_TDD_UL_DL_Pattern2_FR1_SCS15); }else{ FatalError(__FILE__, __LINE__, "SCS value not handled in TTCN"); } } else { v_TDD_UL_DL_ConfigCommon :=cs_TDD_UL_DL_ConfigCommonPattern2(v_SubcarrierSpacing,cs_TDD_UL_DL_Pattern1_FR2,cs_TDD_UL_DL_Pattern2_FR2); } f_NR_CellInfo_SetTDD_UL_DL_ConfigCommon(nr_Cell1,v_TDD_UL_DL_ConfigCommon); }; //@sic R5s210088 sic@ v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); v_NR_UplinkBWP.Dedicated.R15.pucch_Config.setup.schedulingRequestResourceToAddModList[0] := valueof(cs_SchedulingRequestResourceConfig_Mac_71111a(v_SubcarrierSpacing)); f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP, tsc_NR_BWP_Id); //@sic r5s201469 sic@ f_NR_SS_ConfigureSysinfo(nr_Cell1, -, f_NR_InitialiseBcchToPdschConfig(nr_Cell1)); //f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, {v_NR_UplinkBWP})); v_SpCellConfig := f_NR_GetReconfigurationWithSyncDef_MCG(nr_Cell1); //@sic R5s190511 sic@ v_SpCellConfig.reconfigurationWithSync.t304 := ms2000; v_SpCellConfig.reconfigurationWithSync.rach_ConfigDedicated := omit; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric.preambleTransMax := n4; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric.preambleReceivedTargetPower := -104; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); //Configure SS CFRA @sic R5s200526 sic@ //v_RACH_ConfigDedicated := cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(f_NR_CellInfo_GetRACH_ConfigGeneric(nr_Cell1,tsc_NR_BWP_Id), tsc_PreambleIndex,f_NR_CellInfo_GetSSB_Index(nr_Cell1))); /*v_RACH_ConfigDedicated := cs_NR_RACH_ConfigDedicated_Uplink(f_NR_GetRACHConfigDedicated_Def( nr_Cell1, tsc_PreambleIndex)); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell1, cs_TimingInfo_Now, cs_NR_CellConfigPhysicalLayerUplink_Def(f_ConvertRACH_ConfigDedicated(v_RACH_ConfigDedicated.uplink)))); */ //@sic R5s211670 sic@ v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters); f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); // Send message to UE f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,-,false); //@sic R5s200891 sic@ if (ischosen(v_NR_DuplexMode.TDD)) {//@sic R5s211670 sic@ f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_Common_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, {v_NR_UplinkBWP},v_TDD_UL_DL_ConfigCommon)); }else { f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, {v_NR_UplinkBWP})); }; SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)); f_TC_7_1_1_1_1a_NR_TestBody(); f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

## Correction to f_TC_7_1_1_1_1a_ENDC_NR

| Function name | f_TC_7_1_1_1_1a_ENDC_NR |
| --- | --- |
| Reason for change | As per Table 7.1.1.1.1a.3.3-6: RACH-ConfigGeneric, preambleTransMax is set to n4 and as per TP2 test Step 2 to 5 expects four Rach attempts from UE. Here, if SS receiver miss any initial Rach attempt from UE due to the rach power falling outside its reception window which in turn also depends upon path loss( as a result of cable loss variation depending upon different band cabling connections requirement etc.), then it has been observed that current preambleReceivedTargetPower -118 dBm(TS 38.508-1, Table 4.6.3-130) is not very appropriate for this case to be used further and UE sends scgFailure(randomAccessProblem) once its 4 rach attempt gets exhausted. Therefore, in order to increase the test case relialibilty a high preambleReceivedTargetPower of -104 dBm is more suitable here. Simliar power is being used in 7.1.1.1.2 test prose as a reference. NOTE: A Prose CR shall be raised at the next RAN5. |
| Summary of change | Set preambleReceivedTargetPower as -104 dBm |
| TTCN module | \ENDC\7_1_1\MAC_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_1_1a_ENDC_NR() runs on ENDC_NR_PTC { //Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by PDCCH Order / //contention free random access procedure var template (value) CellGroupConfig v_SCGConfig; var template(value) SpCellConfig v_SpCellConfig ; //@sic R5-198888 sic@ var NR_DuplexMode_Type v_NR_DuplexMode ; var template (value) TDD_UL_DL_ConfigCommon v_TDD_UL_DL_ConfigCommon; var NR_Prach_RootSequenceIndex_Type v_Prach_RootSequenceIndex; var boolean v_IsFR1 := false; var SubcarrierSpacing v_SubcarrierSpacing;//@sic R5s201022 sic@ //@sic R5s211670 sic@ var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List; //Init Cell parameters f_ENDC_NR_Init(); //@sic R5s190762 sic@ f_NR_CellInfo_SetRACH_ConfigCommon_71111a(nr_Cell1); //@sic R5-198888 sic@ v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); if (v_IsFR1) //FR1 { v_Prach_RootSequenceIndex := {l839 := 0}; f_NR_CellInfo_SetSIB1_Prach_RootSequenceIndex(nr_Cell1, v_Prach_RootSequenceIndex); f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell1,14); f_NR_CellInfo_SetZeroCorrelationZoneConfig(nr_Cell1,12); } //@sic R5-198888 sic@ if (ischosen(v_NR_DuplexMode.TDD)) { if(v_IsFR1) { v_TDD_UL_DL_ConfigCommon :=cs_TDD_UL_DL_ConfigCommonPattern2(v_SubcarrierSpacing,cs_TDD_UL_DL_Pattern1_FR1,cs_TDD_UL_DL_Pattern2_FR1); } else { v_TDD_UL_DL_ConfigCommon :=cs_TDD_UL_DL_ConfigCommonPattern2(v_SubcarrierSpacing,cs_TDD_UL_DL_Pattern1_FR2,cs_TDD_UL_DL_Pattern2_FR2); } f_NR_CellInfo_SetTDD_UL_DL_ConfigCommon(nr_Cell1,v_TDD_UL_DL_ConfigCommon); }; //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the SCG DRB in test mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0, -, -, -, -, -, -); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); //@sic R5s211670 sic@ f_NR_CellInfo_SetSIB1_PreambleTransMax(nr_Cell1, n4); v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_NR_UplinkBWP_List[0].Dedicated := omit; f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); //@siclog "Steps 0A-0B" siclog@ v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1); v_SpCellConfig.reconfigurationWithSync.t304 := ms2000; v_SpCellConfig.reconfigurationWithSync.rach_ConfigDedicated := omit; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric.preambleTransMax := n4; v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG, omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig,-,-,-,reconfigurationWithSync_CheckRACH); //@sic R5s190541 sic@ f_TC_7_1_1_1_1a_NR_TestBody(); f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

After Change:

function f_TC_7_1_1_1_1a_ENDC_NR() runs on ENDC_NR_PTC { //Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by PDCCH Order / //contention free random access procedure var template (value) CellGroupConfig v_SCGConfig; var template(value) SpCellConfig v_SpCellConfig ; //@sic R5-198888 sic@ var NR_DuplexMode_Type v_NR_DuplexMode ; var template (value) TDD_UL_DL_ConfigCommon v_TDD_UL_DL_ConfigCommon; var NR_Prach_RootSequenceIndex_Type v_Prach_RootSequenceIndex; var boolean v_IsFR1 := false; var SubcarrierSpacing v_SubcarrierSpacing;//@sic R5s201022 sic@ //@sic R5s211670 sic@ var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List; //Init Cell parameters f_ENDC_NR_Init(); //@sic R5s190762 sic@ f_NR_CellInfo_SetRACH_ConfigCommon_71111a(nr_Cell1); //@sic R5-198888 sic@ v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); if (v_IsFR1) //FR1 { v_Prach_RootSequenceIndex := {l839 := 0}; f_NR_CellInfo_SetSIB1_Prach_RootSequenceIndex(nr_Cell1, v_Prach_RootSequenceIndex); f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell1,14); f_NR_CellInfo_SetZeroCorrelationZoneConfig(nr_Cell1,12); } //@sic R5-198888 sic@ if (ischosen(v_NR_DuplexMode.TDD)) { if(v_IsFR1) { v_TDD_UL_DL_ConfigCommon :=cs_TDD_UL_DL_ConfigCommonPattern2(v_SubcarrierSpacing,cs_TDD_UL_DL_Pattern1_FR1,cs_TDD_UL_DL_Pattern2_FR1); } else { v_TDD_UL_DL_ConfigCommon :=cs_TDD_UL_DL_ConfigCommonPattern2(v_SubcarrierSpacing,cs_TDD_UL_DL_Pattern1_FR2,cs_TDD_UL_DL_Pattern2_FR2); } f_NR_CellInfo_SetTDD_UL_DL_ConfigCommon(nr_Cell1,v_TDD_UL_DL_ConfigCommon); }; //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the SCG DRB in test mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0, -, -, -, -, -, -); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); //@sic R5s211670 sic@ f_NR_CellInfo_SetSIB1_PreambleTransMax(nr_Cell1, n4); v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_NR_UplinkBWP_List[0].Dedicated := omit; f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); //@siclog "Steps 0A-0B" siclog@ v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1); v_SpCellConfig.reconfigurationWithSync.t304 := ms2000; v_SpCellConfig.reconfigurationWithSync.rach_ConfigDedicated := omit; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric.preambleTransMax := n4; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric.preambleReceivedTargetPower := -104; v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG, omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig,-,-,-,reconfigurationWithSync_CheckRACH); //@sic R5s190541 sic@ f_TC_7_1_1_1_1a_NR_TestBody(); f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }