3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221101

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2753 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcases 8.1.5.7.1.1, 8.1.5.7.1.2 and 8.1.5.7.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current implementation of the function f_TC_8_1_5_7_1_X_NR5GC_TestBody - At step 8 , Periodic UL grant has to be provided on Scell .Currently it is implemented to transmit the periodic UL grant on Pcell - At Step 10, Failure Information is looking for the v_DRBj_PCell presence in logical channel . As per the reconfiguration Peer message sent to the UE, maxRetxThreshold = t1 (0) is set for logical channel id ( 6 ) and maxRetxThreshold = t32 (7) is set for logical channel id ( 5 ). - Due to this configuration, UE will trigger failure information with logical channel id ( 6 ) . hence it is proposed to set the Failure information with v_DRBj_SCell presence in logical channel - Updated Prose is also stating that logicalChannelIdentity should be LogicalChannelIdentity with condition DRB j+1 ( which is for Scell )Table 8.1.5.7.1.1.3.3-8: FailureInformation ( Please refer R5-224196 ) |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the function f_TC_8_1_5_7_1_X_NR5GC_TestBody , - At step 8, UL grant has to be provided on Scell - At step 10 , Failure information is corrected to check for the logical channel containing v_DRBj_SCell presence |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.7.1.1.NR5GC, 8.1.5.7.1.2.NR5GC, 8.1.5.7.1.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the function f_TC_8_1_5_7_1_X_NR5GC_TestBody	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the function f_TC_8_1_5_7_1_X_NR5GC_TestBody

| Template name | f_TC_8_1_5_7_1_X_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current implementation of the function f_TC_8_1_5_7_1_X_NR5GC_TestBody - At step 8 , Periodic UL grant has to be provided on Scell .Currently it is implemented to transmit the periodic UL grant on Pcell - At Step 10, Failure Information is looking for the v_DRBj_PCell presence in logical channel . but as per the reconfiguration Peer message sent to the UE , maxRetxThreshold = t1 (0) is set for logical channel id ( 6 ) and maxRetxThreshold = t32 (7) is set for logical channel id ( 5 ). - Due to this configuration , UE will trigger failure information with logical channel id ( 6 ) . hence it is proposed to set the Failure information with v_DRBj_SCell presence in logical channel |
| Summary of change | In the function f_TC_8_1_5_7_1_X_NR5GC_TestBody , - At step 8, an UL grant has to be provided on Scell - At step 10, Failure information is corrected to check for the logical channel containing v_DRBj_SCell presence |
| TTCN module | RRC_Others_CA_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_5_7_1_X_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { var template(value) CellGroupConfig v_MCG_Config; var template(value) UL_AM_RLC v_UL_AM_RLC; var template(value) RadioBearerConfig v_RadioBearerConfig; var template(value) RRCReconfiguration_v1530_IEs v_RRCReconfiguration_v1530; var template(omit) NR_RLC_BearerToAddModList_Type v_RLC_Bearer_Config; var IntegerList_Type v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(1); var octetstring v_PDCP_Data := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_NW()); var template (value) NR_RLC_TestModeConfig_Type v_NR_RLC_TestModeConfig := {Info := {AckProhibit := Prohibit}}; //@sic R5s220944 sic@ var template (value) NR_RLC_TestModeConfig_Type v_NR_RLC_TestModeConfig_continue := {Info := {AckProhibit := Continue}}; //@sic R5s220944 sic@ var integer v_DRBj_PCell := f_NR_GetDefaultDRB_ForFirstPDUSession(); var integer v_DRBj_SCell := v_ListOfDRBs[0]; //DRB is used ONLY for SS configuration! @sic R5-223444 sic@ var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_SCell, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList_NotAck := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_PCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig), f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig)}; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList_Ack := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_PCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig_continue), f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig_continue)}; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode, -, cs_SDAP_Configuration_None )}; //@sic R5s220944 sic@ var SubFrameTiming_Type v_TimingUL; //@siclog Steps 1-2 siclog@ //The SS transmits an RRCReconfiguration message to add SCell for the UE f_NR5GC_AddMod_1SCell( p_PCell, p_SCell,-, true, -,false); //CA UL enabled //@sic R5-223424 sic@ //Reconfigure SCell in SS //The UE transmits an RRCReconfigurationComplete message //@siclog Steps 2A siclog@ //The SS transmits a SCell Activation/Deactivation MAC-CE to activate SCell (NR Cell 3). f_SS_CellConfig_ActivateSCell_CA(p_PCell); // @sic R5s220669 R5s220818 sic@ // Dummy DRB configured on SS f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigList, cs_TimingInfo_Now); //@siclog Steps 3-4 siclog@ //The SS transmits an RRCReconfiguration message to establish DRB #j for the UE v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-,{cs_38508_NRDRB_ToAddMod( //@sic R5s220944 sic@ f_NR_GetSDAP_InDRBList(v_DRBj_PCell), v_DRBj_PCell, cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, f_NR_LogicalChannelId_DRB(v_DRBj_PCell), tsc_NR_CellGroupId_MCG, true))}, -, -); //Configure RLC-BearerConfig-1 v_UL_AM_RLC := f_NR_UL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell)); v_UL_AM_RLC.maxRetxThreshold := t32; v_RLC_Bearer_Config[0] := cs_38508_RLC_BearerConfig_DRB_LCID( f_NR_LogicalChannelId_DRB(v_DRBj_PCell), v_DRBj_PCell, cs_38508_RLC_Config_AM_DRB(v_UL_AM_RLC, f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell))), cs_38508_LogicalChannelConfig_ASC(f_NR_LogicalChannelId_DRB(v_DRBj_PCell),-,-,-,-,-,{0})) ; //Configure RLC-BearerConfig21 v_UL_AM_RLC.maxRetxThreshold := t1; v_RLC_Bearer_Config[1] := cs_38508_RLC_BearerConfig_DRB_LCID( f_NR_LogicalChannelId_DRB(v_DRBj_SCell), v_DRBj_PCell, cs_38508_RLC_Config_AM_DRB(v_UL_AM_RLC, f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell))), cs_38508_LogicalChannelConfig_ASC(f_NR_LogicalChannelId_DRB(v_DRBj_SCell),-,-,-,-,-,{1})) ; v_MCG_Config := cs_NR_CellGroupConfigDef(tsc_NR_CellGroupId_MCG, v_RLC_Bearer_Config,omit, omit, omit, omit,omit, omit); v_RRCReconfiguration_v1530 := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_MCG_Config))); f_NR_SendRRCReconfiguration(p_PCell,-,v_RadioBearerConfig,v_RRCReconfiguration_v1530, -, -, -, -, -, crs_MacBearerRouting(p_PCell)); //@sic R5-213516 sic@ //@siclog "Step 5-6" siclog@ // void @sic R5s220944 sic@ //@siclog "Step 7" siclog@ //The SS stops sending MCG RLC acknowledgments on PCell (NR Cell 1) and SCell (NR Cell 3) f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigList_NotAck); //@sic R5s220944 sic@ //@siclog "Step 8" siclog@ //@sic R5-223424 sic@ //SS transmits a periodic UL Grant on SCell (as PUCCH is not configured in the SCell), allowing the UE to loop back the PDCP PDU on SCell. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(4096, v_NR_UplinkBWP); //@sic R5s220458 sic@ v_TimingUL := f_NR_GetNextSendOccasion_UL(p_SCell); //@sic R5s220944 sic@ f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(p_PCell, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, 6); //@siclog "Step 8A" siclog@ //The SS transmits one PDCP PDU to the UE on DRB #j. DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_PDUList(p_PCell, v_DRBj_PCell, cs_TimingInfo_Now, {cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, -, -, crs_MacBearerRouting(p_PCell))); interleave{ //@siclog "Step 9" siclog@ //The UE loops back the PDCP PDU on PCell on DRB #j [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PCell,v_DRBj_PCell,cs_RlcBearerRouting_NR(p_PCell), -, {cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, crs_MacBearerRouting(p_PCell))){} //@siclog "Step 9A" siclog@ //The UE loops back the PDCP PDU on SCell on DRB #j [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PCell, v_DRBj_SCell, cs_RlcBearerRouting_NR(p_PCell), -,{cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, crs_MacBearerRouting(p_SCell))){} //@sic R5s220944 sic@ } //@siclog "Step 10" siclog@ //Check: Does the UE transmit a FailureInformation message with failureType set to rlc-failure? /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ SRB.receive(car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_FailureInformation(tsc_NR_CellGroupId_MCG, f_NR_LogicalChannelId_DRB(v_DRBj_PCell),rlc_failure), -, crs_MacBearerRouting(p_PCell))); //@sic R5-225281 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); //@siclog "Step 10A" siclog@ f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigList_Ack); //@sic R5s220944 sic@ //@siclog "Steps 11 - 12" siclog@ //The SS transmits an RRCReconfigurationmessage containing an sCellToReleaseList with SCell NR Cell 3. //Check: Does the UE transmit an RRCReconfigurationComplete message? f_NR5GC_Rel_1SCell(p_PCell, p_SCell); //@sic R5-214617 sic@ }

After Change:

function f_TC_8_1_5_7_1_X_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { var template(value) CellGroupConfig v_MCG_Config; var template(value) UL_AM_RLC v_UL_AM_RLC; var template(value) RadioBearerConfig v_RadioBearerConfig; var template(value) RRCReconfiguration_v1530_IEs v_RRCReconfiguration_v1530; var template(omit) NR_RLC_BearerToAddModList_Type v_RLC_Bearer_Config; var IntegerList_Type v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(1); var octetstring v_PDCP_Data := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_NW()); var template (value) NR_RLC_TestModeConfig_Type v_NR_RLC_TestModeConfig := {Info := {AckProhibit := Prohibit}}; //@sic R5s220944 sic@ var template (value) NR_RLC_TestModeConfig_Type v_NR_RLC_TestModeConfig_continue := {Info := {AckProhibit := Continue}}; //@sic R5s220944 sic@ var integer v_DRBj_PCell := f_NR_GetDefaultDRB_ForFirstPDUSession(); var integer v_DRBj_SCell := v_ListOfDRBs[0]; //DRB is used ONLY for SS configuration! @sic R5-223444 sic@ var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_SCell, tsc_NR_BWP_Id); var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList_NotAck := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_PCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig), f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig)}; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList_Ack := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_PCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig_continue), f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig_continue)}; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := { f_NR_GetRadioBearerConfig_FullAM(p_PCell, v_DRBj_SCell, -, cs_NR_PDCP_RbConfig_TransparentMode, -, cs_SDAP_Configuration_None )}; //@sic R5s220944 sic@ var SubFrameTiming_Type v_TimingUL; //@siclog Steps 1-2 siclog@ //The SS transmits an RRCReconfiguration message to add SCell for the UE f_NR5GC_AddMod_1SCell( p_PCell, p_SCell,-, true, -,false); //CA UL enabled //@sic R5-223424 sic@ //Reconfigure SCell in SS //The UE transmits an RRCReconfigurationComplete message //@siclog Steps 2A siclog@ //The SS transmits a SCell Activation/Deactivation MAC-CE to activate SCell (NR Cell 3). f_SS_CellConfig_ActivateSCell_CA(p_PCell); // @sic R5s220669 R5s220818 sic@ // Dummy DRB configured on SS f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigList, cs_TimingInfo_Now); //@siclog Steps 3-4 siclog@ //The SS transmits an RRCReconfiguration message to establish DRB #j for the UE v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-,{cs_38508_NRDRB_ToAddMod( //@sic R5s220944 sic@ f_NR_GetSDAP_InDRBList(v_DRBj_PCell), v_DRBj_PCell, cs_NR_PDCP_Config_Split(cs_38508_PDCP_Config_DRB, f_NR_LogicalChannelId_DRB(v_DRBj_PCell), tsc_NR_CellGroupId_MCG, true))}, -, -); //Configure RLC-BearerConfig-1 v_UL_AM_RLC := f_NR_UL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell)); v_UL_AM_RLC.maxRetxThreshold := t32; v_RLC_Bearer_Config[0] := cs_38508_RLC_BearerConfig_DRB_LCID( f_NR_LogicalChannelId_DRB(v_DRBj_PCell), v_DRBj_PCell, cs_38508_RLC_Config_AM_DRB(v_UL_AM_RLC, f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell))), cs_38508_LogicalChannelConfig_ASC(f_NR_LogicalChannelId_DRB(v_DRBj_PCell),-,-,-,-,-,{0})) ; //Configure RLC-BearerConfig21 v_UL_AM_RLC.maxRetxThreshold := t1; v_RLC_Bearer_Config[1] := cs_38508_RLC_BearerConfig_DRB_LCID( f_NR_LogicalChannelId_DRB(v_DRBj_SCell), v_DRBj_PCell, cs_38508_RLC_Config_AM_DRB(v_UL_AM_RLC, f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(p_PCell))), cs_38508_LogicalChannelConfig_ASC(f_NR_LogicalChannelId_DRB(v_DRBj_SCell),-,-,-,-,-,{1})) ; v_MCG_Config := cs_NR_CellGroupConfigDef(tsc_NR_CellGroupId_MCG, v_RLC_Bearer_Config,omit, omit, omit, omit,omit, omit); v_RRCReconfiguration_v1530 := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_MCG_Config))); f_NR_SendRRCReconfiguration(p_PCell,-,v_RadioBearerConfig,v_RRCReconfiguration_v1530, -, -, -, -, -, crs_MacBearerRouting(p_PCell)); //@sic R5-213516 sic@ //@siclog "Step 5-6" siclog@ // void @sic R5s220944 sic@ //@siclog "Step 7" siclog@ //The SS stops sending MCG RLC acknowledgments on PCell (NR Cell 1) and SCell (NR Cell 3) f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigList_NotAck); //@sic R5s220944 sic@ //@siclog "Step 8" siclog@ //@sic R5-223424 sic@ //SS transmits a periodic UL Grant on SCell (as PUCCH is not configured in the SCell), allowing the UE to loop back the PDCP PDU on SCell. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(4096, v_NR_UplinkBWP); //@sic R5s220458 sic@ v_TimingUL := f_NR_GetNextSendOccasion_UL(p_SCell); //@sic R5s220944 sic@ f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(p_SCell, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, 6); //WA#WI=1007803 //@siclog "Step 8A" siclog@ //The SS transmits one PDCP PDU to the UE on DRB #j. DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_PDUList(p_PCell, v_DRBj_PCell, cs_TimingInfo_Now, {cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, -, -, crs_MacBearerRouting(p_PCell))); interleave{ //@siclog "Step 9" siclog@ //The UE loops back the PDCP PDU on PCell on DRB #j [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PCell,v_DRBj_PCell,cs_RlcBearerRouting_NR(p_PCell), -, {cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, crs_MacBearerRouting(p_PCell))){} //@siclog "Step 9A" siclog@ //The UE loops back the PDCP PDU on SCell on DRB #j [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(p_PCell, v_DRBj_SCell, cs_RlcBearerRouting_NR(p_PCell), -,{cs_NR_PDCP_PDU_18B(0, v_PDCP_Data)}, crs_MacBearerRouting(p_SCell))){} //@sic R5s220944 sic@ } //@siclog "Step 10" siclog@ //Check: Does the UE transmit a FailureInformation message with failureType set to rlc-failure? /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ SRB.receive(car_NR_SRB_RrcPdu_IND(p_PCell, tsc_NR_RbId_SRB1, cr_FailureInformation(tsc_NR_CellGroupId_MCG, f_NR_LogicalChannelId_DRB(v_DRBj_SCell),rlc_failure), -, crs_MacBearerRouting(p_PCell))); //@sic R5-225281 sic@ //WA#WI=1007803 f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); //@siclog "Step 10A" siclog@ f_NR_SS_CommonRadioBearerConfig(p_PCell, v_SS_Drb_ConfigList_Ack); //@sic R5s220944 sic@ //@siclog "Steps 11 - 12" siclog@ //The SS transmits an RRCReconfigurationmessage containing an sCellToReleaseList with SCell NR Cell 3. //Check: Does the UE transmit an RRCReconfigurationComplete message? f_NR5GC_Rel_1SCell(p_PCell, p_SCell); //@sic R5-214617 sic@ }