3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221102

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2754 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 8.1.1.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current implementation of the function fl_TC_8_1_1_2_3_TestBody - At step 15a9, rrc release is sent and brings the UE back to Idle state This means , Postamble has to be done in IDLE state |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the function f_TC_8_1_1_2_3_NR5GC, Postamble has to be done in STATE_IDLE_1A instead of STATE_CONNECTED_3A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to the function f_TC_8_1_1_2_3_NR5GC	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the function f_TC_8_1_1_2_3_NR5GC

| Template name | f_TC_8_1_1_2_3_NR5GC |
| --- | --- |
| Reason for change | In the current implementation of the function fl_TC_8_1_1_2_3_TestBody - At step 15a9, rrc release is sent and brings the UE back to Idle state - This means , Postamble has to be done in IDLE state |
| Summary of change | In the function f_TC_8_1_1_2_3_NR5GC , - Postamble has to be done in STATE_IDLE_1A instead of STATE_CONNECTED_3A |
| TTCN module | RRC_ConnEst_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_1_2_3_NR5GC() runs on NR5GC_PTC { // RRC connection establishment / RRC Reject with wait time var Octet_Type v_IP_PDU_delay := '05'O; f_NR5GC_Init(NR_1); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, TEST_LOOPModeB_ON); f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeB_ON, -, v_IP_PDU_delay); f_NR_TestBody_Set(true); fl_TC_8_1_1_2_3_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

After Change:

function f_TC_8_1_1_2_3_NR5GC() runs on NR5GC_PTC { // RRC connection establishment / RRC Reject with wait time var Octet_Type v_IP_PDU_delay := '05'O; f_NR5GC_Init(NR_1); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, TEST_LOOPModeB_ON); f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeB_ON, -, v_IP_PDU_delay); f_NR_TestBody_Set(true); fl_TC_8_1_1_2_3_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); //WA#WI=1008005 }