3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221103

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2755 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC emergency call test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The logic for checking p_IMSMsgsNotYetSent parameter in function f_NR5GC_EmergencyCallReleaseMT() is incorrect – this parameter will be set “true” if a trigger has not already been received from IMS PTC in the calling function, so this is the case where we should wait for the outstanding trigger in this function. This causes the “IMS Call Release Msgs Sent” trigger to block other coordination messages later in the test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove “not” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.x.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR5GC_EmergencyCallReleaseMT() |
| --- | --- |
| Reason for change | The logic for checking p_IMSMsgsNotYetSent parameter is incorrect – this parameter will be set “true” if a trigger has not already been received from IMS PTC in the calling function, so this is the case where we should wait for the outstanding trigger in this function. This causes the “IMS Call Release Msgs Sent” trigger to block other coordination messages later in the test cases. |
| Summary of change | Remove “not” |
| TTCN module | NR5GC_IMS_Procedures.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_EmergencyCallReleaseMT(NR_CellId_Type p_NR_CellId, boolean p_IMSMsgsNotYetSent:= true, EmergencyCallRelease_PDUSession p_Release_Keep := Release_PDUSession) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // @sic R5s210363 sic@ var GSM_MobilityInfo_Type v_PDUSession := valueof(v_GSM_MobilityInfo); // Must be there as previously established it var O1_Type v_PDUSessionId := v_PDUSession.SessionId; var PDN_Index_Type v_PdnIndex := v_PDUSession.PdnIndex; // @sic R5s220829 sic@ if (not p_IMSMsgsNotYetSent) { // @sic R5s220789 sic@ IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger ("IMS Call Release Msgs Sent")); // @sic R5s220039, R5-225362 sic@ sic@ } if (p_Release_Keep == Release_PDUSession) { // @sic R5-225362 sic@ f_NR5GC_PDUSessionRelease (p_NR_CellId, v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); f_IP_RtpRtcpLoopback(IP, v_PdnIndex, tsc_IMS_MediaPort_M1, omit); // @sic R5s210363, R5s220829 sic@ } else { f_NR5GC_ModifyPDUSession_RemoveEmergency(p_NR_CellId); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release"); }

After Change:

function f_NR5GC_EmergencyCallReleaseMT(NR_CellId_Type p_NR_CellId, boolean p_IMSMsgsNotYetSent:= true, EmergencyCallRelease_PDUSession p_Release_Keep := Release_PDUSession) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // @sic R5s210363 sic@ var GSM_MobilityInfo_Type v_PDUSession := valueof(v_GSM_MobilityInfo); // Must be there as previously established it var O1_Type v_PDUSessionId := v_PDUSession.SessionId; var PDN_Index_Type v_PdnIndex := v_PDUSession.PdnIndex; // @sic R5s220829 sic@ if (p_IMSMsgsNotYetSent) { // @sic R5s220789 sic@ IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger ("IMS Call Release Msgs Sent")); // @sic R5s220039, R5-225362 sic@ sic@ } if (p_Release_Keep == Release_PDUSession) { // @sic R5-225362 sic@ f_NR5GC_PDUSessionRelease (p_NR_CellId, v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); f_IP_RtpRtcpLoopback(IP, v_PdnIndex, tsc_IMS_MediaPort_M1, omit); // @sic R5s210363, R5s220829 sic@ } else { f_NR5GC_ModifyPDUSession_RemoveEmergency(p_NR_CellId); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release"); }