3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221127

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0865 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Re-verification of IMS 5GS call control test case 7.21 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To re-verify IMS 5GS test case 7.21 to the IMS/NR5GC ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS test case 7.21 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.21.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 34.229-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	5

4	Execution Log Files	6

4.1	Qualcomm SM8350 + SDX60	6

5	References	6

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 7.21 which is part of the IMS NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

Verification Test Summary

Test Case: 	7.21.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D22wk37

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm SM8350 + SDX60

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_IMS_BuildSDP_InitialOfferAddVideo_TX() |
| --- | --- |
| Reason for change | This function is initalising the audio stream parameters in the SDP offer using the function f_SDP_MediaDescriptionSubsequentOfferAudioTX(), with the parameter p_PrevSdpFromUE defaulting to omit. In this case the function always throws a fatal error (for codec EVS_1600) because the variable v_MediaDesc is never bound. In the context of test case 7.21, the audio stream parameters are updated in the calling function to reflect the previous SDP recieved from the UE. So this function f_IMS_BuildSDP_InitialOfferAddVideo_TX()can use f_SDP_MediaDescriptionInitialOfferAudioTX() instead of f_SDP_MediaDescriptionSubsequentOfferAudioTX() to initialise the audio stream parameters. |
| Summary of change | Use function f_SDP_MediaDescriptionInitialOfferAudioTX() instead of f_SDP_MediaDescriptionSubsequentOfferAudioTX() |
| TTCN module | IMS_SDP_Templates.ttcn |
| MCC160 Comment |  |

Before Change

function f_IMS_BuildSDP_InitialOfferAddVideo_TX(IMS_SDP_AudioCodec_Type p_AudioCodec := AMR_16000, IMS_SDP_VideoCodecList_Type p_VideoCodecList := tsc_IMS_SDP_VideoCodecList_H264_only, boolean p_WithPreconditions := true) runs on IMS_PTC return template (value) SDP_Message { /* @sic R5-213524, R5-213538, R5-213539, R5-213541: new common function replacing f_IMS_BuildSDP_MTCallAudioVideo_TX sic@ */ /* @sic R5s220939, R5-225423: p_PrevSdpFromUE removed sic@ */ var template (value) IMS_SDP_MediaDescriptionRtpList_Type v_MediaDescriptionRtpList_Audio; var template (value) IMS_SDP_MediaDescriptionRtpList_Type v_MediaDescriptionRtpList_Video; var template (omit) SDP_attribute_list v_PreconditionAttributes_Audio := omit; var template (omit) SDP_attribute_list v_PreconditionAttributes_Video := omit; if (p_WithPreconditions) { v_PreconditionAttributes_Audio := cs_SDP_PreconditionAttributes(c_sendrecv, c_sendrecv, c_mandatory, c_optional); v_PreconditionAttributes_Video := cs_SDP_PreconditionAttributes(c_none, c_none, c_mandatory, c_optional); } // ***** Media description audio ***** v_MediaDescriptionRtpList_Audio := f_SDP_MediaDescriptionSubsequentOfferAudioTX(p_AudioCodec); /* @sic R5-172930 sic@ @sic R5-213524, R5-213538, R5-213539, R5-213541: f_SDP_MediaDescriptionSubsequentOfferAudioTX sic@ */ // ***** Media description video ***** v_MediaDescriptionRtpList_Video := fl_SDP_MediaAttributesVideoCodecTX(p_VideoCodecList[0]); /* @sic R5-213524, R5-213538, R5-213539, R5-213541: fl_SDP_MediaAttributesVideoCodecTX sic@ so far there is only one video codec used in MT video call test cases */ return fl_IMS_BuildSDP_CommonOfferAudioVideo_TX(AddVideo, v_MediaDescriptionRtpList_Audio, v_MediaDescriptionRtpList_Video, v_PreconditionAttributes_Audio, v_PreconditionAttributes_Video); }

After Change

function f_IMS_BuildSDP_InitialOfferAddVideo_TX(IMS_SDP_AudioCodec_Type p_AudioCodec := AMR_16000, IMS_SDP_VideoCodecList_Type p_VideoCodecList := tsc_IMS_SDP_VideoCodecList_H264_only, boolean p_WithPreconditions := true) runs on IMS_PTC return template (value) SDP_Message { /* @sic R5-213524, R5-213538, R5-213539, R5-213541: new common function replacing f_IMS_BuildSDP_MTCallAudioVideo_TX sic@ */ /* @sic R5s220939, R5-225423: p_PrevSdpFromUE removed sic@ */ var template (value) IMS_SDP_MediaDescriptionRtpList_Type v_MediaDescriptionRtpList_Audio; var template (value) IMS_SDP_MediaDescriptionRtpList_Type v_MediaDescriptionRtpList_Video; var template (omit) SDP_attribute_list v_PreconditionAttributes_Audio := omit; var template (omit) SDP_attribute_list v_PreconditionAttributes_Video := omit; if (p_WithPreconditions) { v_PreconditionAttributes_Audio := cs_SDP_PreconditionAttributes(c_sendrecv, c_sendrecv, c_mandatory, c_optional); v_PreconditionAttributes_Video := cs_SDP_PreconditionAttributes(c_none, c_none, c_mandatory, c_optional); } // ***** Media description audio ***** v_MediaDescriptionRtpList_Audio := f_SDP_MediaDescriptionInitialOfferAudioTX ({p_AudioCodec}); /* @sic R5-172930 sic@ @sic R5-213524, R5-213538, R5-213539, R5-213541: f_SDP_MediaDescriptionSubsequentOfferAudioTX sic@ */ // ***** Media description video ***** v_MediaDescriptionRtpList_Video := fl_SDP_MediaAttributesVideoCodecTX(p_VideoCodecList[0]); /* @sic R5-213524, R5-213538, R5-213539, R5-213541: fl_SDP_MediaAttributesVideoCodecTX sic@ so far there is only one video codec used in MT video call test cases */ return fl_IMS_BuildSDP_CommonOfferAudioVideo_TX(AddVideo, v_MediaDescriptionRtpList_Audio, v_MediaDescriptionRtpList_Video, v_PreconditionAttributes_Audio, v_PreconditionAttributes_Video); }

# Branches executed

The test case was executed on NR band n79 with 5G encryption algorithm nea1 and integrity algorithm nia1.

# Execution Log Files

## Qualcomm SM8350 + SDX60

The Qualcomm SM8350 + SDX60 UE passed this test case on the Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_7_21_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_7_21_PIXIT.html

# References

| [1] | R5s221129 : Additional supporting information for addition of IMS 5GS call control test case 7.21 |
| --- | --- |