3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221147

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2788 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the NR capability test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN Implementation of the function fl_Check_Rf_Parameters_NRBand_PICS_Supp () - Before checking for “Acc. to TS38.306 cl. 4.2.7.4 A UE supporting scellDormancyOutsideActiveTime_r16 shall also indicate support of power saving DRX adaptation using drx-Adaptation-r16 and shall also support one dormant BWP and at least one non-dormant BWP per carrier” , there has to be check involving pc_scellDormancyOutsideActiveTime_r16 presence In the current TTCN Implementation of the function fl_Check_featureSets () - Before checking for featureSetsDownlink_v1610 sub IEs , check is missing for confirming the presence of featureSetsDownlink_v1610 In the current TTCN Implementation of the function fl_Check_NR_Capability_1540() - Before checking the sub IEs for fr1_fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff , check for the if(ispresent(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff)) is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the TTCN function fl_Check_Rf_Parameters_NRBand_PICS_Supp () Before checking for “Acc. to TS38.306 cl. 4.2.7.4 A UE supporting scellDormancyOutsideActiveTime_r16 shall also indicate support of power saving DRX adaptation using drx-Adaptation-r16 and shall also support one dormant BWP and at least one non-dormant BWP per carrier” , check is introduced for the presence of pc_scellDormancyOutsideActiveTime_r16 In the TTCN function fl_Check_featureSets () ispresent(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlink_v1610) check is introduced before checking the sub IEs of featureSetsDownlink_v1610 In the TTCN function fl_Check_NR_Capability_1540() if(ispresent(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff)) check is introduced before checking for the Sub IEs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1.1.ENDC , 8.1.5.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the function fl_Check_Rf_Parameters_NRBand_PICS_Supp	3

2.2	Correction to the function fl_Check_featureSets	5

2.3	Correction to the function fl_Check_NR_Capability_1540	8

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the function fl_Check_Rf_Parameters_NRBand_PICS_Supp

| Function name | fl_Check_Rf_Parameters_NRBand_PICS_Supp |
| --- | --- |
| Reason for change | In the current TTCN Implementation of the function fl_Check_Rf_Parameters_NRBand_PICS_Supp () - Before checking for “Acc. to TS38.306 cl. 4.2.7.4 A UE supporting scellDormancyOutsideActiveTime_r16 shall also indicate support of power saving DRX adaptation using drx-Adaptation-r16 and shall also support one dormant BWP and at least one non-dormant BWP per carrier” , there has to be check involving pc_scellDormancyOutsideActiveTime_r16 presence |
| Summary of change | Before checking for “Acc. to TS38.306 cl. 4.2.7.4 A UE supporting scellDormancyOutsideActiveTime_r16 shall also indicate support of power saving DRX adaptation using drx-Adaptation-r16 and shall also support one dormant BWP and at least one non-dormant BWP per carrier” , check is introduced for the presence of pc_scellDormancyOutsideActiveTime_r16 |
| TTCN module | NR_CapabilityFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_Check_Rf_Parameters_NRBand_PICS_Supp (UE_NR_Capability p_NRCapability) { << SKIPPED CODE >> if (not f_NR_CompareCapability_vs_PICS(pc_scellDormancyOutsideActiveTime_r16, v_Found_scellDormancyOutsideActiveTime_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_scellDormancyOutsideActiveTime_r16 not correctly set"); } else { if (not pc_DRX_Adaptation or (not (pc_bwp_SameNumerology_upto2 or pc_bwp_SameNumerology_upto4))) { f_SetVerdict(fail,__FILE__, __LINE__, "Acc. to TS38.306 cl. 4.2.7.4 A UE supporting scellDormancyOutsideActiveTime_r16 shall also indicate support of power saving DRX adaptation using drx-Adaptation-r16 and shall also support one dormant BWP and at least one non-dormant BWP per carrier"); } } if (not f_NR_CompareCapability_vs_PICS(pc_interFreqDAPS_r16, v_Found_interFreqDAPS_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_interFreqDAPS_r16 not correctly set"); } }; //end of fl_Check_Rf_Parameters_NRBand_PICS_Supp

After Change:

function fl_Check_Rf_Parameters_NRBand_PICS_Supp (UE_NR_Capability p_NRCapability) { << SKIPPED CODE >> if (not f_NR_CompareCapability_vs_PICS(pc_scellDormancyOutsideActiveTime_r16, v_Found_scellDormancyOutsideActiveTime_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_scellDormancyOutsideActiveTime_r16 not correctly set"); } else { if (pc_scellDormancyOutsideActiveTime_r16 and (not pc_DRX_Adaptation or (not (pc_bwp_SameNumerology_upto2 or pc_bwp_SameNumerology_upto4)))) { //WA#WI=1009382 f_SetVerdict(fail,__FILE__, __LINE__, "Acc. to TS38.306 cl. 4.2.7.4 A UE supporting scellDormancyOutsideActiveTime_r16 shall also indicate support of power saving DRX adaptation using drx-Adaptation-r16 and shall also support one dormant BWP and at least one non-dormant BWP per carrier"); } } if (not f_NR_CompareCapability_vs_PICS(pc_interFreqDAPS_r16, v_Found_interFreqDAPS_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_interFreqDAPS_r16 not correctly set"); } }; //end of fl_Check_Rf_Parameters_NRBand_PICS_Supp

## Correction to the function fl_Check_featureSets

| Function name | fl_Check_featureSets |
| --- | --- |
| Reason for change | In the current TTCN Implementation of the function fl_Check_featureSets () - Before checking for featureSetsDownlink_v1610 sub IEs , check is missing for confirming the presence of featureSetsDownlink_v1610 |
| Summary of change | ispresent(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlink_v1610) check is introduced before checking the sub IEs of featureSetsDownlink_v1610 |
| TTCN module | NR_CapabilityFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_Check_featureSets (UE_NR_Capability p_NRCapability) { var UE_NR_Capability v_Received_NRCapabilityMsg; var integer v_Index := 0; var boolean v_Found_dynamicSwitchSUL := false; var boolean v_Found_intraFreqDAPS_r16 := false; var boolean v_Found_pusch_RepetitionTypeB_r16 := false; v_Received_NRCapabilityMsg := p_NRCapability; while ((v_Index < lengthof(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink)) and (not v_Found_dynamicSwitchSUL)) { // check featureSetsUplink if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink[v_Index].dynamicSwitchSUL)) { v_Found_dynamicSwitchSUL := true; } v_Index := v_Index + 1; //WA#R5s221100 } if (not f_NR_CompareCapability_vs_PICS(pc_dynamicSwitch_SUL, v_Found_dynamicSwitchSUL)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_dynamicSwitch_SUL not correctly set"); } v_Index := 0; while ((v_Index < lengthof(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlink_v1610)) and (not v_Found_intraFreqDAPS_r16)) { // check featureSetsDownlink_v1610 if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlink_v1610[v_Index].intraFreqDAPS_r16)) { v_Found_intraFreqDAPS_r16 := true; } v_Index := v_Index + 1; //WA#R5s221100 } if (not f_NR_CompareCapability_vs_PICS(pc_intraFreqDAPS_r16, v_Found_intraFreqDAPS_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_intraFreqDAPS_r16 not correctly set"); } v_Index := 0; while ((v_Index < lengthof(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink_v1610)) and (not v_Found_pusch_RepetitionTypeB_r16)) { // check featureSetsUplink_v1610 if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink_v1610[v_Index].pusch_RepetitionTypeB_r16)) { v_Found_pusch_RepetitionTypeB_r16 := true; } v_Index := v_Index + 1; //WA#R5s221100 } if (not f_NR_CompareCapability_vs_PICS(pc_pusch_RepetitionTypeB_r16, v_Found_pusch_RepetitionTypeB_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_pusch_RepetitionTypeB_r16 not correctly set"); } }

After Change:

function fl_Check_featureSets (UE_NR_Capability p_NRCapability) { var UE_NR_Capability v_Received_NRCapabilityMsg; var integer v_Index := 0; var boolean v_Found_dynamicSwitchSUL := false; var boolean v_Found_intraFreqDAPS_r16 := false; var boolean v_Found_pusch_RepetitionTypeB_r16 := false; v_Received_NRCapabilityMsg := p_NRCapability; while ((v_Index < lengthof(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink)) and (not v_Found_dynamicSwitchSUL)) { // check featureSetsUplink if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink[v_Index].dynamicSwitchSUL)) { v_Found_dynamicSwitchSUL := true; } v_Index := v_Index + 1; //WA#R5s221100 } if (not f_NR_CompareCapability_vs_PICS(pc_dynamicSwitch_SUL, v_Found_dynamicSwitchSUL)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_dynamicSwitch_SUL not correctly set"); } v_Index := 0; if(ispresent(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlink_v1610)) //WA#WI=1009382 { while ((v_Index < lengthof(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlink_v1610)) and (not v_Found_intraFreqDAPS_r16)) { // check featureSetsDownlink_v1610 if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsDownlink_v1610[v_Index].intraFreqDAPS_r16)) { v_Found_intraFreqDAPS_r16 := true; } v_Index := v_Index + 1; //WA#R5s221100 } } //WA#WI=1009382 if (not f_NR_CompareCapability_vs_PICS(pc_intraFreqDAPS_r16, v_Found_intraFreqDAPS_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_intraFreqDAPS_r16 not correctly set"); } v_Index := 0; if(ispresent(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink_v1610)) //WA#WI=1009382 { while ((v_Index < lengthof(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink_v1610)) and (not v_Found_pusch_RepetitionTypeB_r16)) { // check featureSetsUplink_v1610 if (isvalue(v_Received_NRCapabilityMsg.featureSets.featureSetsUplink_v1610[v_Index].pusch_RepetitionTypeB_r16)) { v_Found_pusch_RepetitionTypeB_r16 := true; } v_Index := v_Index + 1; //WA#R5s221100 } }//WA#WI=1009382 if (not f_NR_CompareCapability_vs_PICS(pc_pusch_RepetitionTypeB_r16, v_Found_pusch_RepetitionTypeB_r16)) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_pusch_RepetitionTypeB_r16 not correctly set"); } }

## Correction to the function fl_Check_NR_Capability_1540

| Function name | fl_Check_NR_Capability_1540 |
| --- | --- |
| Reason for change | In the current TTCN Implementation of the function fl_Check_NR_Capability_1540() Before checking the sub IEs for fr1_fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff , check for the if(ispresent(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff)) is missing |
| Summary of change | if(ispresent(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff)) check is introduced before checking for the Sub IEs |
| TTCN module | NR_CapabilityFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_Check_NR_Capability_1540 (UE_NR_Capability p_NRCapability) { var UE_NR_Capability v_Received_NRCapabilityMsg; v_Received_NRCapabilityMsg := p_NRCapability; if (isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension )) { if (not f_NR_CompareCapability_vs_PICS(pc_as_ReflectiveQoS, isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.sdap_Parameters.as_ReflectiveQoS ))) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_as_ReflectiveQoS not correctly set"); } // Checks on fr1_fr2_Add_UE_NR_Capabilities if (not f_NR_CompareCapability_vs_PICS(pc_ss_SINR_Meas, isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff.ss_SINR_Meas ))) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_ss_SINR_Meas not correctly set"); // @sic R5-225384 sic@ } if (not f_NR_CompareCapability_vs_PICS(pc_csi_RSRP_AndRSRQ_MeasWithSSB, isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff.csi_RSRP_AndRSRQ_MeasWithSSB ))) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_csi_RSRP_AndRSRQ_MeasWithSSB not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_csi_RSRP_AndRSRQ_MeasWithoutSSB, isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff.csi_RSRP_AndRSRQ_MeasWithoutSSB ))) { // @sic R5-225384 sic@ f_SetVerdict(inconc,__FILE__, __LINE__, "pc_csi_RSRP_AndRSRQ_MeasWithoutSSB not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_handoverInterF, isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff.handoverInterF ))) { // @sic R5-225384 sic@ f_SetVerdict(inconc,__FILE__, __LINE__, "pc_handoverInterF not correctly set"); } } // nonCriticalExtension is OPTIONAL else { if (pc_as_ReflectiveQoS or pc_csi_RSRP_AndRSRQ_MeasWithSSB or pc_ss_SINR_Meas or pc_csi_RSRP_AndRSRQ_MeasWithoutSSB or pc_handoverInterF) { f_SetVerdict(inconc,__FILE__, __LINE__, "Setting of UE-NR-Capability-1540 PICS is inconsistent with UE reported capabilities"); } } }; //end of fl_Check_NR_Capability_1540

After Change:

function fl_Check_NR_Capability_1540 (UE_NR_Capability p_NRCapability) { var UE_NR_Capability v_Received_NRCapabilityMsg; v_Received_NRCapabilityMsg := p_NRCapability; if (isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension )) { if (not f_NR_CompareCapability_vs_PICS(pc_as_ReflectiveQoS, isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.sdap_Parameters.as_ReflectiveQoS ))) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_as_ReflectiveQoS not correctly set"); } // Checks on fr1_fr2_Add_UE_NR_Capabilities if(ispresent(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff)) //WA#WI=1009382 { if (not f_NR_CompareCapability_vs_PICS(pc_ss_SINR_Meas, isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff.ss_SINR_Meas ))) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_ss_SINR_Meas not correctly set"); // @sic R5-225384 sic@ } if (not f_NR_CompareCapability_vs_PICS(pc_csi_RSRP_AndRSRQ_MeasWithSSB, isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff.csi_RSRP_AndRSRQ_MeasWithSSB ))) { f_SetVerdict(inconc,__FILE__, __LINE__, "pc_csi_RSRP_AndRSRQ_MeasWithSSB not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_csi_RSRP_AndRSRQ_MeasWithoutSSB, isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff.csi_RSRP_AndRSRQ_MeasWithoutSSB ))) { // @sic R5-225384 sic@ f_SetVerdict(inconc,__FILE__, __LINE__, "pc_csi_RSRP_AndRSRQ_MeasWithoutSSB not correctly set"); } if (not f_NR_CompareCapability_vs_PICS(pc_handoverInterF, isvalue(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.fr1_fr2_Add_UE_NR_Capabilities.measAndMobParametersFRX_Diff.handoverInterF ))) { // @sic R5-225384 sic@ f_SetVerdict(inconc,__FILE__, __LINE__, "pc_handoverInterF not correctly set"); } //WA#WI=1009382 } } // nonCriticalExtension is OPTIONAL else { if (pc_as_ReflectiveQoS or pc_csi_RSRP_AndRSRQ_MeasWithSSB or pc_ss_SINR_Meas or pc_csi_RSRP_AndRSRQ_MeasWithoutSSB or pc_handoverInterF) { f_SetVerdict(inconc,__FILE__, __LINE__, "Setting of UE-NR-Capability-1540 PICS is inconsistent with UE reported capabilities"); } } }; //end of fl_Check_NR_Capability_1540