3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221153

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2790 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 8.1.4.2.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2022-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the PDU SESSION ESTABLISHMENT procedure, there has one EPS bearer with bearer Identity 10 configured in the Mapped EPS bearer context to UE; in the PDU SESSION MODIFICATION COMMAND message there again has one EPS bearer with bearer Identity 7 configured in the Mapped EPS bearer context to UE. Then in function fl_TC_8_1_4_2_1_2_NR5GC_EUTRA_TestBody () at step 3 the template cr_508_EPS_BearerContextStatusTAUReq expects epsBearerContextStatus.ebi8_15 to be '00000000'B , which is incorrect as the UE would report EPS Bearer ID 10 and 7 as active.. This needs to be addressed. Since this testcase has no dependency on content of EPS Bearer Context Status in the TAU REQ, it is proposed to relax the check for this IE. A Prose CR will be raised for this |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of using template cr_508_EPS_BearerContextStatusTAUReq, used template cr_EPS_BearerContextStatus_Any if present |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail this test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.2.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | fl_TC_8_1_4_2_1_2_NR5GC_EUTRA_TestBody() |
| --- | --- |
| Reason for change | In the PDU SESSION ESTABLISHMENT procedure, there has one EPS bearer with bearer Identity 10 configured in the Mapped EPS bearer context to UE; in the PDU SESSION MODIFICATION COMMAND message there again has one EPS bearer with bearer Identity 7 configured in the Mapped EPS bearer context to UE. Then in function fl_TC_8_1_4_2_1_2_NR5GC_EUTRA_TestBody () at step 3 the template cr_508_EPS_BearerContextStatusTAUReq expects epsBearerContextStatus.ebi8_15 to be '00000000'B , which is incorrect as the UE would report EPS Bearer ID 10 and 7 as active.. This needs to be addressed. Since this testcase has no dependency on content of EPS Bearer Context Status in the TAU REQ, it is proposed to relax the check for this IE. A Prose CR will be raised for this |
| Summary of change | Instead of using template cr_508_EPS_BearerContextStatusTAUReq, used template cr_EPS_BearerContextStatus_Any if present |
| TTCN module | 8_1_4\IRAT_Handover_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_4_2_1_2_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { ….. // @siclog "Step 3" siclog@ SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_TAU_Request_N1ToS1((tsc_EpsUpdate_TaUpdate, tsc_EpsUpdate_Combined_IMSIAttach, tsc_EpsUpdate_Combined_TaLaUpdate), v_SecurityRx.KSI, cr_DRXparameter_Any ('5C'O) ifpresent, v_AdditionalUpdateType, v_MappedGUTI, v_TAI ifpresent, -, tsc_NasKsi_MappedSecurityContext, cdr_NAS_KeySetIdentifier_tlv (?, ?) ifpresent, -, -, cr_508_EPS_BearerContextStatusTAUReq ifpresent))))-> value v_ReceivedMsg; …… }

After Change:

function fl_TC_8_1_4_2_1_2_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { ….. // @siclog "Step 3" siclog@ SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_TAU_Request_N1ToS1((tsc_EpsUpdate_TaUpdate, tsc_EpsUpdate_Combined_IMSIAttach, tsc_EpsUpdate_Combined_TaLaUpdate), v_SecurityRx.KSI, cr_DRXparameter_Any ('5C'O) ifpresent, v_AdditionalUpdateType, v_MappedGUTI, v_TAI ifpresent, -, tsc_NasKsi_MappedSecurityContext, cdr_NAS_KeySetIdentifier_tlv (?, ?) ifpresent, -, -, cr_EPS_BearerContextStatus_Any ifpresent))))-> value v_ReceivedMsg; …… }