3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221156

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2792 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC SNPN test case 6.5.1.2 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD, Rohde & Schwarz and Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR5GC SNPN test case 6.5.1.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC SNPN test case 6.5.1.2 for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	4

4	Execution Log Files	4

4.1	Qualcomm Snapdragon X70 5G Modem-RF System	4

5	References	5

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.5.1.2 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	6.5.1.2

ATS Version:	iwd-TTCN3-B2020-09_D22wk37

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

R&S 5G Conformance Platform,

Keysight S8704A Protocol Conformance Toolset solution

UE used:	Qualcomm Snapdragon X70 5G Modem-RF System 5G UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | TC_6_5_1_2_NR5GC |
| --- | --- |
| Reason for change | Current guard timer of 300s may not be sufficient for execution of this test case, following the addition of manual SNPN search steps at RAN5#96. |
| Summary of change | Increased the guard timer for the tesc case to 600s. |
| TTCN module | NR5GC_Testsuite |
| MCC160 Comment |  |

Before Change:

testcase TC_6_5_1_2_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // SNPN Selection in Automatic Mode var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_6_5_1_2_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

After Change:

testcase TC_6_5_1_2_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // SNPN Selection in Automatic Mode var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(600); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_6_5_1_2_NR5GC()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

# Branches executed

On Anritsu, the test case was executed on NR band n41 using NR ciphering nea2 and integrity algorithm nia2.

On RnS, the test case was executed on NR band n41 using NR ciphering nea1 and integrity algorithm nia1.

On Keysight, the test case was executed on NR band n41 using NR ciphering nea1 and integrity algorithm nia1.

# Execution Log Files

## Qualcomm Snapdragon X70 5G Modem-RF System

The Snapdragon X70 5G Modem-RF System, Qualcomm Technologies’ 5th generation modem-to-antenna UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR, R&S 5G Conformance Platform, and Keysight S8704A Protocol Conformance Toolset solution. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\ TC_6_5_1_2_LOG.xml

RnS\ TC_6_5_1_2_LOG.log

Keysight\ TC_6_5_1_2_LOG.xml

- PICS/PIXIT parameter file:

Anritsu\ TC_6_5_1_2_PIXIT.txt

RnS\ TC_6_5_1_2_LOG.log

Keysight\ TC_6_5_1_2_PIXIT.xml

# References

| [1] | R5s221157 - Supporting information for addition of NR5GC SNPN test case 6.5.1.2 in FR1 |
| --- | --- |