3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221166

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2797 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC MDT-SON test case 8.1.6.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following the change of R5-225386, the IEs timeUntilReconnection-16 and reconnectCellId-r16 in UEInformationResponse message at step 14 have been changed to “Not Checked” and “Not Present/Not Checked” respectively. Current TTCN is still expecting & checking these. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the expectation for IEs timeUntilReconnection-16 and reconnectCellId-r16to AnyOrOmit |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_8_1_6_2_3_TestBody |
| --- | --- |
| Reason for change | Following the change of R5-225386, the IEs timeUntilReconnection-16 and reconnectCellId-r16 in UEInformationResponse message at step 14 have been changed to “Not Checked” and “Not Present/Not Checked” respectively. Current TTCN is still expecting & checking these. |
| Summary of change | Changed the expectation for IEs timeUntilReconnection-16 and reconnectCellId-r16to AnyOrOmit |
| TTCN module | RRC_MDT_InterRAT_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_1_6_2_3_TestBody() runs on NR5GC_PTC { … … … } //@siclog "Step 12" siclog@ //Void.// @sic R5s220831 R5-225386 sic@ deactivate(v_MyDefaultVar); //@siclog "Step 13" siclog@ //The SS transmits a UEInformationRequest message. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, -, -, -, true_))); //@siclog "Step 14" siclog@ //Check: Does the UE transmit a UEInformationResponse message on NR Cell 1? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def,-,-,-,-,-,?))) -> value v_ReceivedAsp; v_Nr_Rlf_Report_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16; v_MeasResultNeighCells_r16:=cr_NR_RLF_Report_measResultNeighCells_Eutra_r16(cr_MeasResultNeighCellListEUTRA_1Entry(v_ARFCN_ValueEUTRA,v_EUTRA_CellId,// @sic R5s220831 sic@ f_EUTRA_Get_MeasQuantityResults())); // @sic R5s220831 R5-225386 sic@ if (match(v_Nr_Rlf_Report_r16, cr_NR_Rlf_Report(v_C_RNTI_beforeRLF, -, //SSB omit incorrect ?, ?, v_MeasResultNeighCells_r16, omit, cr_NR_RLF_Report_failedPCellId_cellGlobalId_r16(cr_CGI_Info_Logging_r16(v_PLMN_Identity_Cell1, v_CellIdentity_Cell1, v_TAC_Cell1)), cr_EutraReconnectCellId_r16, ?, omit, -, ?, rlf, t310_Expiry, -, omit))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); } else if (match(v_Nr_Rlf_Report_r16, cr_NR_Rlf_Report(v_C_RNTI_beforeRLF, -, //SSB omit incorrect ?, ?, v_MeasResultNeighCells_r16, omit, cr_NR_RLF_Report_failedPCellId_PciArfcn_r16(v_PhysCellId_Cell1, v_SSB_Freq_Cell1), cr_EutraReconnectCellId_r16, ?, omit, -, ?, rlf, t310_Expiry, -, omit))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14"); } }//End of f_TC_8_1_6_2_3_TestBody()

After Change:

function fl_TC_8_1_6_2_3_TestBody() runs on NR5GC_PTC { … … … } //@siclog "Step 12" siclog@ //Void.// @sic R5s220831 R5-225386 sic@ deactivate(v_MyDefaultVar); //@siclog "Step 13" siclog@ //The SS transmits a UEInformationRequest message. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, -, -, -, true_))); //@siclog "Step 14" siclog@ //Check: Does the UE transmit a UEInformationResponse message on NR Cell 1? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def,-,-,-,-,-,?))) -> value v_ReceivedAsp; v_Nr_Rlf_Report_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16; v_MeasResultNeighCells_r16:=cr_NR_RLF_Report_measResultNeighCells_Eutra_r16(cr_MeasResultNeighCellListEUTRA_1Entry(v_ARFCN_ValueEUTRA,v_EUTRA_CellId,// @sic R5s220831 sic@ f_EUTRA_Get_MeasQuantityResults())); // @sic R5s220831 R5-225386 sic@ if (match(v_Nr_Rlf_Report_r16, cr_NR_Rlf_Report(v_C_RNTI_beforeRLF, -, //SSB omit incorrect ?, ?, v_MeasResultNeighCells_r16, omit, cr_NR_RLF_Report_failedPCellId_cellGlobalId_r16(cr_CGI_Info_Logging_r16(v_PLMN_Identity_Cell1, v_CellIdentity_Cell1, v_TAC_Cell1)), *, *, omit, -, ?, rlf, t310_Expiry, -, omit))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); } else if (match(v_Nr_Rlf_Report_r16, cr_NR_Rlf_Report(v_C_RNTI_beforeRLF, -, //SSB omit incorrect ?, ?, v_MeasResultNeighCells_r16, omit, cr_NR_RLF_Report_failedPCellId_PciArfcn_r16(v_PhysCellId_Cell1, v_SSB_Freq_Cell1), *, *, omit, -, ?, rlf, t310_Expiry, -, omit))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14"); } }//End of f_TC_8_1_6_2_3_TestBody()