3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221179

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2806 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 Sep'22 baseline upgrade for 5GS Test Suites |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2022-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to develop the Rel-17 test cases in TTCN-3, the current baseline of type definitions shall be upgraded. RAN5 agreed to upgrade the baseline to September 2022 Rel-17 to all RAN5 TTCN-3 test suites. This CR is implementing this decision on the 38.523-3 Test Suites. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR lists all changes affecting the messages and IEs type definitions caused by the baseline upgrade to September 2022 Rel-17. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-17 test cases cannot be implemented and verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All 5GS Test Suites |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

Overview	3

1	Baseline Moving – 38.331 ASN.1	4

1.1	NR_RRC_ASN1_Definitions	4

1.1.1	Type change of nonCriticalExtension	4

1.1.2	Name changes	5

1.1.3	Fields added with extension marker (ellipsis)	8

1.1.4	Further ASN.1 changes	28

1.2	NR_SideLink_Preconf	28

1.2.1	Fields added with extension marker (ellipsis)	28

1.3	NR_PC5_RRC_Definitions	29

1.3.1	Type change of nonCriticalExtension	29

1.3.2	Name changes	29

1.3.3	Fields added with extension marker (ellipsis)	29

1.3.4	Further ASN.1 changes	30

2	Baseline Moving – 38.321	30

3	Baseline Moving – 38.322	30

4	Baseline Moving – 38.323	30

5	Baseline Moving – 38.212 38.214	31

6	Baseline Moving – 37.324	32

7	Baseline Moving – 24.501 (NAS)	33

7.1	Modified Message Definitions	33

7.2	Modified Information Elements	35

7.3	New Information Elements	36

8	Baseline Moving – 24.587 (V2X)	37

8.1	Modified Information Elements	37

9	Baseline Moving – 24.588 (V2X)	38

9.1	Modified Information Elements	38

9.2	New Information Elements	38

10	Other changes	39

10.1	ASP type SPS_Config_R16	39

10.2	f_NR5GC_RRC_ProcedureDelayReestablishment	39

10.3	Changes for Audit test cases	40

10.4	NAS Changes	48

10.4.1	Modified the definition of the CAG_Info NSSAI and NG_TrackingAreaUpdateList Types	48

10.4.2	Changes related to new DNNs	50

# Overview

This document lists all the changes needed for baseline moving based on iwd-TTCN3-B2020-09_D22wk37.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Baseline Moving – 38.331 ASN.1

## NR_RRC_ASN1_Definitions

### Type change of nonCriticalExtension

| ASN.1 type affected | New nonCriticalExtension type |
| --- | --- |
| DLInformationTransfer-v1610-IEs | DLInformationTransfer-v1700-IEs |
| LoggedMeasurementConfiguration-r16-IEs | LoggedMeasurementConfiguration-v1700-IEs |
| Paging | Paging-v1700-IEs |
| RRCReestablishment-IEs | RRCReestablishment-v1700-IEs |
| RRCReconfiguration-v1610-IEs | RRCReconfiguration-v1700-IEs |
| RRCReconfigurationComplete-v1640-IEs | RRCReconfigurationComplete-v1700-IEs |
| RRCRelease-v1650-IEs | RRCRelease-v1710-IEs |
| RRCResume-v1700-IEs | RRCResume-v1700-IEs |
| RRCResumeComplete-v1640-IEs | RRCResumeComplete-v1700-IEs |
| RRCSetup-IEs | RRCSetup-v1700-IEs |
| RRCSetupComplete-v1610-IEs | RRCSetupComplete-v1690-IEs |
| SIB1-v1630-IEs | SIB1-v1700-IEs |
| SidelinkUEInformationNR-r16-IEs | SidelinkUEInformationNR-v1700-IEs |
| UEAssistanceInformation-v1610-IEs | UEAssistanceInformation-v1700-IEs |
| UEInformationRequest-r16-IEs | UEInformationRequest-v1700-IEs |
| UEInformationResponse-r16-IEs | UEInformationResponse-v1700-IEs |
| ULInformationTransfer-IEs | ULInformationTransfer-v1700-IEs |
| UE-CapabilityRequestFilterNR-v1540 | UE-CapabilityRequestFilterNR-v1710 |
| UE-MRDC-Capability | OCTET STRING (CONTAINING UE-MRDC-Capability-v15g0) |
| UE-MRDC-Capability-v1610 | UE-MRDC-Capability-v1700 |
| UE-NR-Capability-v15c0 | UE-NR-Capability-v15g0 |
| UE-NR-Capability-v1650 | UE-NR-Capability-v1690 |

no changes in TTCN

### Name changes

| ASN.1 type affected | Renaming | Impact on TTCN |
| --- | --- | --- |
| SIB-ReqInfo-r16 | Enumerated values spare5, spare4 Renamed respectively to sib20-v1700, sib21-v1700 | No impact |
| FailureReportSCG | Enumerated value spare4 renamed to beamFailure-r17 | No impact |
| SL-Failure-r16 | Enumerated value spare6 renamed to drxReject-v1710 | No impact |
| RA-Report-r16 | Enumerated value spare9 renamed to msg3RequestForOtherSI-r17 | No impact |
| PerRAInfoListExt-v1660 | Type renamed to PerRAInfoList-v1660 | No impact |
| RA-InformationCommon-r16 | Field perRAInfoListExt-v1660 renamed to perRAInfoList-v1660 | cr_RA_InformationCommon_r16 |
| PerRACSI-RSInfoExt-v1660 | Type renamed to PerRACSI-RSInfo-v1660 | No impact |
| RLF-Report-r16 | Enumerated value spare2 renamed to t312-expiry-r17 | No impact |
| SIB3 | Field intraFreqBlackCellList renamed to intraFreqExcludedCellList Field intraFreqWhiteCellList-r16 renamed to intraFreqAllowedCellList-r16 | cs_38508_SIB3_Def |
| IntraFreqBlackCellList | Type renamed to IntraFreqExcludedCellList | cs_38508_SIB3_Def f_NR5GC_CellInfo_SetSIB3_IntraFreqBlackCellList f_TC_6_1_2_17_NR5GC |
| IntraFreqWhiteCellList-r16 | Type renamed to IntraFreqAllowedCellList-r16 | No impact |
| InterFreqCarrierFreqInfo | Field interFreqBlackCellList renamed to interFreqExcludedCellList | cs_38508_InterFreqCarrierFreqInfo |
| InterFreqCarrierFreqInfo-v1610 | Field interFreqWhiteCellList-r16 renamed to interFreqAllowedCellList-r16 | No impact |
| InterFreqWhiteCellList-r16 | Type renamed to InterFreqAllowedCellList-r16 | No impact |
| CarrierFreqEUTRA | Field eutra-BlackCellList renamed to eutra-ExcludedCellList | No impact |
| EUTRA-FreqBlackCellList | Type renamed to EUTRA-FreqExcludedCellList | No impact |
| BeamFailureRecoverySCellConfig-r16 | Type renamed to BeamFailureRecoveryRSConfig-r16 | No impact |
| CSI-ReportConfig | Enumerated spare1 renamed to table4-r17 | No impact |
| PCCH-Config | Fields sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT sCS120KHZoneEighthT-SCS60KHZoneSixteenthT sCS120KHZoneSixteenthT renamed respectively to sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT sCS480KHZquarterT-SCS120KHZoneSixteenthT | No impact |
| DRX-ConfigSecondaryGroup | Type changed to DRX-ConfigSecondaryGroup-r16 Fields renamed from drx-onDurationTimer drx-InactivityTimer to drx-onDurationTimer-r16 drx-InactivityTimer-r16 | No impact |
| LogicalChannelConfig | Enumerated spare2 to ms0p01-v1700 | No impact |
| MeasObjectEUTRA | blackCellsToRemoveListEUTRAN renamed to excludedCellsToRemoveListEUTRAN blackCellsToAddModListEUTRAN renamed to excludedCellsToAddModListEUTRAN | cs_MeasObjectEUTRA_Common |
| EUTRA-BlackCell | Type renamed to EUTRA-ExcludedCell | No impact |
| MeasObjectNR | Fields blackCellsToRemoveList blackCellsToAddModList whiteCellsToRemoveList whiteCellsToAddModList renamed to excludedCellsToRemoveList excludedCellsToAddModList allowedCellsToRemoveList allowedCellsToAddModList | f_TC_8_1_3_1_16_TestBody |
| NeedForGapsIntraFreqlist-r16 | Type renamed to NeedForGapsIntraFreqList-r16 | No impact |
| NeedForGapsBandlistNR-r16 | Type renamed to NeedForGapsBandListNR-r16 | No impact |
| PUCCH-Config | Field resourceToAddModListExt-r16 renamed to resourceToAddModListExt-v1610 | cs_38508_PUCCH_Config |
| PUCCH-ResourceExt-r16 | Type renamed to PUCCH-ResourceExt-v1610 And fields formatExt-v1610 renamed to format-v1610 rb-SetIndex renamed to rb-SetIndex-r16 interlace0 renamed to interlace0-r16 | No impact |
| EventTriggerConfig | useWhiteCellList renamed to useAllowedCellList | cs_NR_ReportConfigEventA3 cs_38508_EventA3 cs_38508_EventA3_RSRQ reorganised to use cs_38508_EventA3 cs_38508_EventA4 cs_38508_EventA5 cs_NR_EventA5_SINR |
| PeriodicalReportConfig | useWhiteCellList renamed to useAllowedCellList | cs_38508_ReportConfigNR_Periodical cs_ReportConfigNR_Periodical_UlDelayValueConfig_r16_release cs_ReportConfigNR_Periodical_UlDelayValueConfig_r16_setup |
| SubcarrierSpacing | Enumerated values spare3 spare2 renamed respectively to kHz480-v1700, kHz960-v1700 | No impact |
| UL-DelayValueConfig-r16 | delay-DRBlist renamed to delay-DRBlist-r16 | cs_ReportConfigNR_Periodical_UlDelayValueConfig_r16_setup |
| AccessStratumRelease | Spare6 renamed to rel17 | Added rel17 branch in f_NR_ConvertAccessStratumReleaseToInteger f_NR_NRFullCapabilityCheck |
| CA-ParametersNR-v1630 | cbm renamed to dummy | No impact |
| MIMO-ParametersPerBand | Enumerated n0, n2, n3 renamed respectively to supported1, supported2, supported3 | No impact |
| Phy-ParametersCommon | Fields seperateSMTC-InterIAB-Support-r16 and seperateRACH-IAB-Support-r1 renamed respectively to separateSMTC-InterIAB-Support-r16 separateRACH-IAB-Support-r16 | cr_Phy_ParametersCommon |
| InterFreqTargetInfo-r16 | Fields dl-CarrierFreq and cellList renamed respectively to dl-CarrierFreq-r16 and cellList-r16 | No impact |
| RA-InformationCommon-r16 | perRAInfoListExt-v1660 renamed to perRAInfoList-v1660 | cr_RA_InformationCommon_r16 |

### Fields added with extension marker (ellipsis)

| ASN.1 type affected | New fields | Impact on TTCN |
| --- | --- | --- |
| PosSIB-ReqInfo-r16 | Enumerated values added | No impact |
| SuspendConfig | sl-UEIdentityRemote-r17 sdt-Config-r17 srs-PosRRC-Inactive-r17 ran-ExtendedPagingCycle-r17 | Fields added as omit in cs_SuspendConfig_Params Template renamed to cs_SuspendConfigDef |
| CellReselectionPriorities | freqPriorityListDedicatedSlicing-r17 | Fields added as omit in cs_CellReselectionPriorities |
| FailureReportSCG | previousPSCellId-r17 failedPSCellId-r17 timeSCGFailure-r17 perRAInfoList-r17 | Fields added as * in cr_FailureReportSCG_NR_r15 |
| LogMeasInfo-r16 | inDeviceCoexDetected-r17 | Field added as * in cr_LogMeasInfo_r16 cr_LogMeasInfo_r16_LocationInfo_Motion Templates reorganised to use cr_LogMeasInfo_r16_Def |
| RA-Report-r16 | spCellID-r17 | Field added as * in cr_NR_RA_Report_r16 |
| RA-InformationCommon-r16 | msg1-SCS-From-prach-ConfigurationIndex-r16 msg1-SCS-From-prach-ConfigurationIndexCFRA-r16 msgA-RO-FrequencyStart-r17 msgA-RO-FrequencyStartCFRA-r17 msgA-SubcarrierSpacing-r17 msgA-RO-FDM-r17 msgA-RO-FDMCFRA-r17 msgA-SCS-From-prach-ConfigurationIndex-r17 msgA-TransMax-r17 msgA-MCS-r17 nrofPRBs-PerMsgA-PO-r17 msgA-PUSCH-TimeDomainAllocation-r17 frequencyStartMsgA-PUSCH-r17 nrofMsgA-PO-FDM-r17 dlPathlossRSRP-r17 intendedSIBs-r17 ssbsForSI-Acquisition-r17 msgA-PUSCH-PayloadSize-r17 onDemandSISuccess-r17 | Fields added as * in cr_RA_InformationCommon_r16 |
| PerRAAttemptInfo-r16 | fallbackToFourStepRA-r17 | Field added as * in cr_PerRAInfoList_r16_dlRSRPAboveThreshold cr_PerRAAttemptInfo_r16 cr_PerRAInfoList_r16_contentionDetected_true cr_PerRAInfoList_r16_contentionDetected_false Templates reorganised to use cr_PerRAInfoList_r16_Def |
| RLF-Report-r16 | lastHO-Type-r17 timeConnSourceDAPS-Failure-r17 timeSinceCHO-Reconfig-r17 choCellId-r17 choCandidateCellList-r17 measResult-RLF-Report-EUTRA-v1690 | Field added as * in cr_NR_Rlf_Report cr_NR_RLF_Report_r16 |
| SIB2 | ssb-PositionQCL-Common-r17 smtc4list-r17 cellEquivalentSize-r17 relaxedMeasurement-r17 | Added as omit in cs_38508_SIB2_Def |
| SIB3 | intraFreqNeighHSDN-CellList-r17 intraFreqNeighCellList-v1710 | Added as omit in cs_38508_SIB3_Def |
| SIB4 | interFreqCarrierFreqList-v1700 interFreqCarrierFreqList-v1720 | Added as omit in cs_38508_SIB4_Def |
| SIB5 | carrierFreqListEUTRA-v1700 idleModeMeasVoiceFallback-r17 | Added as omit in cs_38508_SIB5_Def |
| SIB12-IEs-r16 | sl-DRX-ConfigCommonGC-BC-r17 sl-DiscConfigCommon-r17 sl-L2U2N-Relay-r17 sl-NonRelayDiscovery-r17 sl-L3U2N-RelayDiscovery-r17 sl-TimersAndConstantsRemoteUE-r17 | Added as omit in cs_38508_SIB12_IEs |
| PosSystemInformation-r16-IEs | Choice values added | No impact |
| PosSI-SchedulingInfo-r16 | posSI-RequestConfigRedCap-r17 | Added as omit in cs_PosSI_SchedulingInfo |
| AvailabilityCombinationsPerCell-r16 | availabilityCombinationsRB-Groups-r17 positionInDCI-AI-RBGroups-v1720 | No impact |
| BeamFailureRecoverySCellConfig-r16 | candidateBeamRS-List2-r17 | No impact |
| BWP-DownlinkDedicated | preConfGapStatus-r17 beamFailureRecoverySpCellConfig-r17 harq-FeedbackEnablingforSPSactive-r17 cfr-ConfigMulticast-r17 dl-PPW-PreConfigToAddModList-r17 dl-PPW-PreConfigToReleaseList-r17 nonCellDefiningSSB-r17 servingCellMO-r17 | Added as omit in cs_BWP_DownlinkDedicated_Def Template cs_38508_BWP_DownlinkDedicated redefined using cs_BWP_DownlinkDedicated_Def |
| BWP-UplinkCommon | enableRA-PrioritizationForSlicing-r17 additionalRACH-ConfigList-r17 rsrp-ThresholdMsg3-r17 numberOfMsg3-RepetitionsList-r17 mcs-Msg3-Repetitions-r17 | Added as omit in cs_BWP_UplinkCommon_Def |
| BWP-UplinkDedicated | ul-TCI-StateList-r17 ul-powerControl-r17 pucch-ConfigurationListMulticast1-r17 pucch-ConfigurationListMulticast2-r17 pucch-ConfigMulticast1-r17 pucch-ConfigMulticast2-r17 | Added as omit in cs_BWP_UplinkDedicated_Def Template cs_38508_BWP_UplinkDedicated reoganised |
| CellAccessRelatedInfo | snpn-AccessInfoList-r17 | Added as omit in cs_38508_CellAccessRelatedInfo |
| CellGroupConfig | f1c-TransferPathNRDC-r17 uplinkTxSwitching-2T-Mode-r17 uplinkTxSwitching-DualUL-TxState-r17 uu-RelayRLC-ChannelToAddModList-r17 uu-RelayRLC-ChannelToReleaseList-r17 simultaneousU-TCI-UpdateList1-r17 simultaneousU-TCI-UpdateList2-r17 simultaneousU-TCI-UpdateList3-r17 simultaneousU-TCI-UpdateList4-r17 rlc-BearerToReleaseListExt-r17 iab-ResourceConfigToAddModList-r17 iab-ResourceConfigToReleaseList-r17 reportUplinkTxDirectCurrentMoreCarrier-r17 | Added as omit in cs_NR_CellGroupConfigDef |
| SpCellConfig | lowMobilityEvaluationConnected-r17 goodServingCellEvaluationRLM-r17 goodServingCellEvaluationBFD-r17 deactivatedSCG-Config-r17 | Added as omit in cs_NR_SpCellConfigDef |
| ReconfigurationWithSync | sl-PathSwitchConfig-r17 | Added as omit in cs_NR_ReconfigurationWithSyncDef |
| SCellConfig | preConfGapStatus-r17 goodServingCellEvaluationBFD-r17 sCellSIB20-r17 | Added as omit in cs_SCellConfig_Def |
| CondReconfigToAddMod-r16 | condExecutionCondSCG-r17 | Added as omit in new template cs_CondReconfigToAddMod_Def cs_CondReconfigToAddModList_r16_1EntryCR_1EntryMI and cs_CondReconfigToAddModList_r16_2EntryCR_1EntryMI and cs_CondReconfigToAddModList_r16_2EntryCR_2EntryMI_First_1Entry_Second reorganised using cs_CondReconfigToAddMod_Def |
| ConfiguredGrantConfig | pathlossReferenceIndex2-r17 srs-ResourceIndicator2-r17 precodingAndNumberOfLayers2-r17 timeDomainAllocation-v1710 timeDomainOffset-r17 cg-SDT-Configuration-r17 and cg-betaOffsetsCrossPri0-r17 cg-betaOffsetsCrossPri1-r17 mappingPattern-r17 sequenceOffsetForRV-r17 p0-PUSCH-Alpha2-r17 powerControlLoopToUse2-r17 cg-COT-SharingList-r17 periodicityExt-r17 repK-v1710 nrofHARQ-Processes-v1700 harq-ProcID-Offset2-v1700 configuredGrantTimer-v1700 cg-minDFI-Delay-v1710 | Added as omit in cs_RRC_ConfiguredUplinkGrant and cs_ConfiguredGrantConfig_Def |
| ControlResourceSet | followUnifiedTCIstate-r17 | Added as omit in cs_38508_ControlResourceSet |
| CrossCarrierSchedulingConfig | ccs-BlindDetectionSplit-r17 | No impact |
| CSI-AperiodicTriggerState | ap-CSI-MultiplexingMode-r17 | No impact |
| CSI-AssociatedReportConfigInfo | resourcesForChannel2-r17 csi-SSB-ResourceSetExt | No impact |
| CSI-MeasConfig | sCellActivationRS-ConfigToAddModList-r17 sCellActivationRS-ConfigToReleaseList-r17 | Added as omit in cs_CSI_MeasConfig cs_TC_7_1_1_8_1_CSI_MeasConfig_FR2 cs_TC_7_1_1_8_1_CSI_MeasConfig_FR1 Templates reorganised using cs_CSI_MeasConfig_Def |
| CSI-ReportConfig | cqi-BitsPerSubband-r17 groupBasedBeamReporting-v1710 codebookConfig-r17 sharedCMR-r17 csi-ReportMode-r17 numberOfSingleTRP-CSI-Mode1-r17 reportQuantity-r17 semiPersistentOnPUSCH-v1720 aperiodic-v1720 | No impact |
| CSI-ResourceConfig | csi-SSB-ResourceSetListExt-r17 | Added as omit in cs_CSI_ResourceConfig |
| CSI-RS-Resource-Mobility | slotConfig-r17 | Added as omit in cs_NR_CSI_RS_Resource_Mobility |
| CSI-SemiPersistentOnPUSCH-TriggerState | sp-CSI-MultiplexingMode-r17 | No impact |
| CSI-SSB-ResourceSet | servingAdditionalPCIList-r17 | No impact |
| DownlinkConfigCommon | initialDownlinkBWP-RedCap-r17 | Added as omit in cs_38508_DownlinkConfigCommon |
| DownlinkConfigCommonSIB | pei-Config-r17 initialDownlinkBWP-RedCap-r17 | Added as omit in cs_38508_DownlinkConfigCommonSIB |
| PCCH-Config | ranPagingInIdlePO-r17 firstPDCCH-MonitoringOccasionOfPO-v1710 | Added as omit in cs_38508_PCCH_Config |
| LogicalChannelConfig | logicalChannelGroupIAB-Ext-r17 allowedHARQ-mode-r17 | Added as omit in cs_NR_LogicalChannelConfigDef cs_38508_LogicalChannelConfig_ASC cs_38508_LogicalChannelConfig_ASC updated to use cs_NR_LogicalChannelConfigDef |
| MAC-CellGroupConfig | intraCG-Prioritization-r17 drx-ConfigSL-r17 drx-ConfigExt-v1700 schedulingRequestID-BFR-r17 schedulingRequestID-BFR2-r17 schedulingRequestConfig-v1700 tar-Config-r17 g-RNTI-ConfigToAddModList-r17 g-RNTI-ConfigToReleaseList-r17 g-CS-RNTI-ConfigToAddModList-r17 g-CS-RNTI-ConfigToReleaseList-r17 allowCSI-SRS-Tx-MulticastDRX-Active-r17 schedulingRequestID-PosMG-Request-r17 drx-LastTransmissionUL-r17 | Added as omit in cs_NR_MAC_CellGroupConfig |
| MeasGapConfig | gapToAddModList-r17 gapToReleaseList-r17 posMeasGapPreConfigToAddModList-r17 posMeasGapPreConfigToReleaseList-r17 | Fields added as omit in cs_38508_MeasGapConfig_UE cs_NR_MeasGapConfig_FR1 cs_NR_MeasGapConfig_FR2 cs_NR_MeasGapRelease_FR2 cs_MeasGapRelease_FR2 cs_MeasGapConfig_FR1 cs_MeasGapRelease_FR1 Templates are reorganised to avoid duplication |
| MeasObjectEUTRA | associatedMeasGap-r17 | Added as omit in cs_NR_MeasObjectNR_Base |
| MeasObjectNR | associatedMeasGapSSB-r17 associatedMeasGapCSIRS-r17 smtc4list-r17 measCyclePSCell-r17 cellsToAddModListExt-v1710 associatedMeasGapSSB2-v1720 associatedMeasGapCSIRS2-v1720 | Added as omit in cs_NR_MeasObjectNR_Base |
| SSB-ConfigMobility | deriveSSB-IndexFromCellInter-r17 ssb-PositionQCL-Common-r17 ssb-PositionQCL-Cells-r17 | Added as omit in cs_SSB_ConfigMobility |
| RMTC-Config-r16 | rmtc-Bandwidth-r17 measDurationSymbols-v1700 ref-SCS-CP-v1700 tci-StateInfo-r17 ref-BWPId-r17 | No impact |
| MeasResults | measResultRxTxTimeDiff-r17 ul-PDCP-ExcessDelayResultList-r17 coarseLocationInfo-r17 | Added as omit in cr_38508_MeasResults cr_38508_MeasResults_LocationInfo cr_38508_MeasResults_CGI cr_38508_MeasResults_CA cr_38508_MeasResults_Beams cr_NR_MeasResults_Ul_PDCP_DelayValueResultList cr_38508_MeasResults_NR_EUTRA cr_38508_MeasResults_LocationInfo update to a derived template cdr_38508_MeasResults_LocationInfo cr_NR_MeasResults_Ul_PDCP_DelayValueResultList is merged into cr_38508_MeasResults |
| MeasResults | sl-MeasResultServingRelay-r17 | Added as * in cr_38508_MeasResults cr_38508_MeasResults_CA cr_38508_MeasResults_Beams cr_38508_MeasResults_NR_EUTRA cr_38508_MeasResults_LocationInfo update to a derived template cdr_38508_MeasResults_LocationInfo cr_NR_MeasResults_Ul_PDCP_DelayValueResultList is merged into cr_38508_MeasResults |
| MeasResultNR | choCandidate-r17 | Added as omit in cr_NR_MeasResultNR_CellResults cr_NR_MeasResultNR_CellResults_Beams cr_NR_MeasResultNR_CellResults_CGI cr_NR_MeasResultNR_CellResults_NPN cr_MeasResultListNR_1Entry |
| MeasResultNR | choConfig-r17 triggeredEvent-r17 | Added as * in cr_NR_MeasResultNR_CellResults cr_NR_MeasResultNR_CellResults_Beams cr_NR_MeasResultNR_CellResults_CGI cr_NR_MeasResultNR_CellResults_NPN cr_MeasResultListNR_1Entry |
| NPN-IdentityInfo-r16 | gNB-ID-Length-r17 | Added as omit in cs_NPN_IdentityInfo_Def |
| NZP-CSI-RS-ResourceSet | pdc-Info-r17 cmrGroupingAndPairing-r17 aperiodicTriggeringOffset-r17 aperiodicTriggeringOffsetL2-r17 | Added as omit in cs_NZP_CSI_RS_ResourceSet cs_TC_7_1_1_8_1_NZP_CSI_RS_ResourceSet_FR1 cs_TC_7_1_1_8_1_NZP_CSI_RS_ResourceSet_FR2 |
| PDCCH-Config | searchSpacesToAddModListExt-v1700 monitoringCapabilityConfig-v1710 searchSpaceSwitchConfig-r17 pdcch-SkippingDurationList-r17 | Added as omit in cs_38508_PDCCH_Config |
| PDCCH-ConfigCommon | sdt-SearchSpace-r17 searchSpaceMCCH-r17 searchSpaceMTCH-r17 commonSearchSpaceListExt2-r17 firstPDCCH-MonitoringOccasionOfPO-v1710 pei-ConfigBWP-r17 followUnifiedTCIstate-v1720 | Added as omit in cs_38508_PDCCH_ConfigCommon |
| PDCCH-ServingCellConfig | searchSpaceSwitchTimer-v1710 | Added as omit in cs_38508_PDCCH_ServingCellConfig |
| PDCP-Config | survivalTimeStateSupport-r17 uplinkDataCompression-r17 discardTimerExt2-r17 initialRX-DELIV-r17 | Added as omit in cs_PDCP_Config_Def cs_NR_PDCP_Config_MoreThanTwoRLC redefined using cs_PDCP_Config_Def |
| PDSCH-Config | pdsch-HARQ-ACK-OneShotFeedbackDCI-1-2-r17 pdsch-HARQ-ACK-EnhType3DCI-1-2-r17 pdsch-HARQ-ACK-EnhType3DCI-Field-1-2-r17 pdsch-HARQ-ACK-RetxDCI-1-2-r17 pucch-sSCellDynDCI-1-2-r17 dl-OrJoint-TCIStateList-r17 beamAppTime-r17 pdsch-TimeDomainAllocationListForMultiPDSCH-r17 dmrs-FD-OCC-DisabledForRank1-PDSCH-r17 minimumSchedulingOffsetK0-r17 harq-ProcessNumberSizeDCI-1-2-v1700 harq-ProcessNumberSizeDCI-1-1-r17 mcs-Table-r17 mcs-TableDCI-1-2-r17 xOverheadMulticast-r17 priorityIndicatorDCI-4-2-r17 sizeDCI-4-2-r17 | Added as omit in cs_NR_PDSCH_ConfigDef |
| PDSCH-ServingCellConfig | downlinkHARQ-FeedbackDisabled-r17 nrofHARQ-ProcessesForPDSCH-v1700 | Added as omit in cs_NR_PDSCH_ServingCellConfigDef |
| PDSCH-TimeDomainResourceAllocation-r16 | k0-v1710 | No impact |
| PHR-Config | mpe-Reporting-FR2-r17 twoPHRMode-r17 | Added as omit in cs_38508_PHR_Config cs_PHR_Config_Params cs_38508_PHR_Config updated to use cs_PHR_Config_Params |
| PhysicalCellGroupConfig | pdsch-HARQ-ACK-EnhType3ToAddModList-r17 pdsch-HARQ-ACK-EnhType3ToReleaseList-r17 pdsch-HARQ-ACK-EnhType3SecondaryToAddModList-r17 pdsch-HARQ-ACK-EnhType3SecondaryToReleaseList-r17 pdsch-HARQ-ACK-EnhType3DCI-FieldSecondaryPUCCHgroup-r17 pdsch-HARQ-ACK-EnhType3DCI-Field-r17 pdsch-HARQ-ACK-Retx-r17 pdsch-HARQ-ACK-RetxSecondaryPUCCHgroup-r17 pucch-sSCell-r17 pucch-sSCellSecondaryPUCCHgroup-r17 pucch-sSCellDyn-r17 pucch-sSCellDynSecondaryPUCCHgroup-r17 pucch-sSCellPattern-r17 pucch-sSCellPatternSecondaryPUCCHgroup-r17 uci-MuxWithDiffPrio-r17 uci-MuxWithDiffPrioSecondaryPUCCHgroup-r17 simultaneousPUCCH-PUSCH-r17 simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup-r17 prioLowDG-HighCG-r17 prioHighDG-LowCG-r17 twoQCLTypeDforPDCCHRepetition-r17 multicastConfig-r17 pdcch-BlindDetectionCA-CombIndicator-r17 simultaneousSR-PUSCH-diffPUCCH-Groups-r17 | Added as omit in cs_NR_PhysicalCellGroupConfigDef |
| PLMN-IdentityInfo | trackingAreaList-r17 gNB-ID-Length-r17 | Added as omit in cs_NR_PLMN_IdentityInfo |
| PUCCH-Config | format0-r17 format2Ext-r17 format3Ext-r17 format4Ext-r17 ul-AccessConfigListDCI-1-2-r17 mappingPattern-r17 powerControlSetInfoToAddModList-r17 powerControlSetInfoToReleaseList-r17 secondTPCFieldDCI-1-1-r17 secondTPCFieldDCI-1-2-r17 dl-DataToUL-ACK-r17 dl-DataToUL-ACK-DCI-1-2-r17 ul-AccessConfigListDCI-1-1-r17 schedulingRequestResourceToAddModListExt-v1700 dmrs-BundlingPUCCH-Config-r17 dl-DataToUL-ACK-v1700 dl-DataToUL-ACK-MulticastDCI-Format4-1-r17 sps-PUCCH-AN-ListMulticast-r17 | Added as omit in cs_38508_PUCCH_Config |
| PUCCH-ResourceExt-r16 | formatExt-v1700 pucch-RepetitionNrofSlots-r17 | No impact |
| PUCCH-ConfigCommon | nrofPRBs intra-SlotFH-r17 pucch-ResourceCommonRedCap-r17 additionalPRBOffset-r17 | Added as omit in cs_38508_PUCCH_ConfigCommon |
| PUCCH-TPC-CommandConfig | tpc-IndexPUCCH-sSCell-r17 tpc-IndexPUCCH-sScellSecondaryPUCCHgroup-r17 | No impact |
| PUSCH-Config | ul-AccessConfigListDCI-0-2-r17 betaOffsetsCrossPri0-r17 betaOffsetsCrossPri1-r17 betaOffsetsCrossPri0DCI-0-2-r17 betaOffsetsCrossPri1DCI-0-2-r17 mappingPattern-r17 secondTPCFieldDCI-0-1-r17 secondTPCFieldDCI-0-2-r17 sequenceOffsetForRV-r17 ul-AccessConfigListDCI-0-1-r17 minimumSchedulingOffsetK2-r17 availableSlotCounting-r17 dmrs-BundlingPUSCH-Config-r17 harq-ProcessNumberSizeDCI-0-2-v1700 harq-ProcessNumberSizeDCI-0-1-r17 mpe-ResourcePoolToAddModList-r17 mpe-ResourcePoolToReleaseList-r17 | Added as omit in cs_NR_PUSCH_ConfigDef |
| PUSCH-PowerControl-v1610 | sri-PUSCH-MappingToAddModList2-r17 sri-PUSCH-MappingToReleaseList2-r17 p0-PUSCH-SetList2-r17 pathlossReferenceRSToAddModListExt-v1710 | No impact |
| PUSCH-ServingCellConfig | nrofHARQ-ProcessesForPUSCH-r17 uplinkHARQ-mode-r17 | Added as omit in cs_NR_PUSCH_ServingCellConfigDef |
| PUSCH-Allocation-r16 | numberOfRepetitionsExt-r17 numberOfSlotsTBoMS-r17 extendedK2-r17 | No impact |
| RACH-ConfigCommon | ra-PrioritizationForSlicing-r17 featureCombinationPreamblesList-r17 | Added as omit in cs_NR_RACH_ConfigCommonDef |
| RACH-ConfigCommonTwoStepRA-r16 | ra-PrioritizationForSlicingTwoStep-r17 featureCombinationPreamblesList-r17 | No impact |
| RACH-ConfigGeneric | ra-ResponseWindow-v1700 | Added as omit in cs_NR_RACH_ConfigGenericDef |
| RACH-ConfigGenericTwoStepRA-r16 | msgB-ResponseWindow-v1700 | No impact |
| RadioBearerConfig | mrb-ToAddModList-r17 mrb-ToReleaseList-r17 srb4-ToAddMod-r17 srb4-ToRelease-r17 | Added as omit in cs_38508_RadioBearerConfigDef cs_NR_RadioBearer_ReleaseSRB3 redefined using cs_38508_RadioBearerConfigDef |
| SRB-ToAddMod | srb-Identity-v1700 | Added as omit in cs_NR_SRB_ToAddMod cs_NR_SRB_ToAddMod_ReestablishPDCP redefined using cs_NR_SRB_ToAddMod |
| RadioLinkMonitoringConfig | beamfailure-r17 | Added as omit in cs_38508_RadioLinkMonitoringConfigDef cs_RadioLinkMonitoringConfig |
| EventTriggerConfigInterRAT | reportQuantityRelay-r17 | Added as omit in cs_NR_ReportConfigInterRAT_Trigger_Periodical_Def cs_NR_ReportConfigInterRAT_Trigger_periodical_EUTRA cs_NR_ReportConfigInterRAT_Trigger_Periodical_UTRA cs_NR_ReportConfigInterRAT_Trigger_eventB1_EUTRA cs_NR_ReportConfigInterRAT_Trigger_eventB2_EUTRA cs_NR_ReportConfigInterRAT_Trigger_eventB2_UTRA cs_NR_ReportConfigInterRAT_Trigger_eventB1_UTRA cs_NR_ReportConfigInterRAT_Trigger_eventB1_EUTRA_Rsrq update to a derived template cds_NR_ReportConfigInterRAT_Trigger_eventB1_EUTRA_Rsrq cs_NR_ReportConfigInterRAT_Trigger_eventB2_EUTRA_Rsrq update to a derived template cds_NR_ReportConfigInterRAT_Trigger_eventB2_EUTRA_Rsrq cs_NR_ReportConfigInterRAT_Trigger_eventB2_EUTRA_Sinr update to a derived template cds_NR_ReportConfigInterRAT_Trigger_eventB2_EUTRA_Sinr cs_NR_ReportConfigInterRAT_Trigger_periodical_EUTRA and cs_NR_ReportConfigInterRAT_Trigger_Periodical_UTRA reorganised using cs_NR_ReportConfigInterRAT_Trigger_Periodical_Def |
| EventTriggerConfig | coarseLocationRequest-r17 reportQuantityRelay-r17 | Added as omit in cs_38508_ReportConfigNR_EventTriggered |
| PeriodicalReportConfig | ul-ExcessDelayConfig-r17 coarseLocationRequest-r17 reportQuantityRelay-r17 | Added as omit in cs_38508_ReportConfigNR_Periodical cs_ReportConfigNR_Periodical_UlDelayValueConfig_r16_setup cr_NR_MeasResults_Ul_PDCP_DelayValueResultList |
| RLC-BearerConfig | rlc-Config-v1700 logicalChannelIdentityExt-r17 multicastRLC-BearerConfig-r17 servedRadioBearerSRB4-r17 | Added as omit ni cs_NR_BearerConfigDef |
| ServingCellConfig | nr-dl-PRS-PDC-Info-r17 semiStaticChannelAccessConfigUE-r17 mimoParam-r17 channelAccessMode2-r17 timeDomainHARQ-BundlingType1-r17 nrofHARQ-BundlingGroups-r17 fdmed-ReceptionMulticast-r17 moreThanOneNackOnlyMode-r17 tci-ActivatedConfig-r17 directionalCollisionHandling-DC-r17 lte-NeighCellsCRS-AssistInfoList-r17 lte-NeighCellsCRS-Assumptions-r17 | Added as omit in cs_NR_ServingCellConfigDef |
| ServingCellConfigCommon | highSpeedConfig-v1700 channelAccessMode2-r17 discoveryBurstWindowLength-r17 ssb-PositionQCL-r17 highSpeedConfigFR2-r17 uplinkConfigCommon-v1700 ntn-Config-r17 featurePriorities-r17 | Added as omit in cs_NR_ServingCellConfigCommonDef |
| ServingCellConfigCommonSIB | channelAccessMode2-r17 discoveryBurstWindowLength-v1700 highSpeedConfigFR2-r17 uplinkConfigCommon-v1700 | Added as omit in cs_38508_ServingCellConfigCommonSIB |
| SlotFormatIndicator | co-DurationsPerCellToAddModList-r17 | No impact |
| SPS-Config | sps-HARQ-Deferral-r17 n1PUCCH-AN-PUCCHsSCell-r17 periodicityExt-r17 nrofHARQ-Processes-v1710 harq-ProcID-Offset-v1700 | Added as no impact in cs_SPS_Config_711161 cs_SPS_Config_711164_1 cs_SPS_Config_711164_2 |
| SRS-ResourceSet | usagePDC-r17 availableSlotOffsetList-r17 followUnifiedTCIstateSRS-r17 | Added as omit in cs_38508_SRS_ResourceSet |
| SRS-Resource | spatialRelationInfo-PDC-r17 resourceMapping-r17 partialFreqSounding-r17 transmissionComb-n8-r17 srs-TCIState-r17 | Added as omit in cs_38508_SRS_Resource |
| SRS-PosResource-r16 | periodicityAndOffset-sp-Ext-r16 periodicityAndOffset-p-Ext-r16 | Added as omit in cs_SRS_PosResourceElement |
| TCI-State | additionalPCI-r17 pathlossReferenceRS-Id-r17 ul-powerControl-r17 | Added as omit in cs_38508_TCI_State cs_TCI_State |
| BandCombination-UplinkTxSwitch-r16 | uplinkTxSwitching-PUSCH-TransCoherence-r16 | No impact |
| FeatureSets | featureSetsDownlink-v1700 featureSetsDownlinkPerCC-v1700 featureSetsUplink-v1710 featureSetsUplinkPerCC-v1700 featureSetsDownlink-v1720 featureSetsDownlinkPerCC-v1720 featureSetsUplink-v1720 | Added as * in cr_UE_NR_FeatureSets |
| MAC-ParametersCommon | enhancedUuDRX-forSidelink-r17 mg-ActivationRequestPRS-Meas-r17 mg-ActivationCommPRS-Meas-r17 intraCG-Prioritization-r17 jointPrioritizationCG-Retx-Timer-r17 survivalTime-r17 lcg-ExtensionIAB-r17 harq-FeedbackDisabled-r17 uplink-Harq-ModeB-r17 sr-TriggeredBy-TA-Report-r17 extendedDRX-CycleInactive-r17 simultaneousSR-PUSCH-DiffPUCCH-groups-r17 lastTransmissionUL-r17 | Added as * in cr_NR_mac_ParametersCommon |
| MeasAndMobParametersCommon | concurrentMeasGap-r17 nr-NeedForGapNCSG-Reporting-r17 eutra-NeedForGapNCSG-Reporting-r17 ncsg-MeasGapPerFR-r17 ncsg-MeasGapPatterns-r17 ncsg-MeasGapNR-Patterns-r17 preconfiguredUE-AutonomousMeasGap-r17 preconfiguredNW-ControlledMeasGap-r17 handoverFR1-FR2-2-r17 handoverFR2-1-FR2-2-r17 independentGapConfigPRS-r17 rrm-RelaxationRRC-ConnectedRedCap-r17 parallelMeasurementGap-r17 condHandoverWithSCG-NRDC-r17 gNB-ID-LengthReporting-r17 gNB-ID-LengthReporting-ENDC-r17 gNB-ID-LengthReporting-NEDC-r17 gNB-ID-LengthReporting-NRDC-r17 gNB-ID-LengthReporting-NPN-r17 parallelSMTC-r17 concurrentMeasGapEUTRA-r17 serviceLinkPropDelayDiffReporting-r17 ncsg-SymbolLevelScheduleRestrictionInter-r17 | Added as * in cr_NR_MeasAndMobParametersCommon |
| MIMO-ParametersPerBand | supportedSINR-meas-v1670 srs-increasedRepetition-r17 srs-partialFrequencySounding-r17 srs-startRB-locationHoppingPartial-r17 srs-combEight-r17 codebookParametersfetype2-r17 mTRP-PUSCH-twoCSI-RS-r17 mTRP-PUCCH-InterSlot-r17 mTRP-PUCCH-CyclicMapping-r17 mTRP-PUCCH-SecondTPC-r17 mTRP-BFR-twoBFD-RS-Set-r17 mTRP-BFR-PUCCH-SR-perCG-r17 mTRP-BFR-association-PUCCH-SR-r17 sfn-SimulTwoTCI-AcrossMultiCC-r17 sfn-DefaultDL-BeamSetup-r17 sfn-DefaultUL-BeamSetup-r17 srs-TriggeringOffset-r17 srs-TriggeringDCI-r17 codebookComboParameterMixedType-r17 unifiedJointTCI-r17 unifiedJointTCI-multiMAC-CE-r17 unifiedJointTCI-perBWP-CA-r17 unifiedJointTCI-ListSharingCA-r17 unifiedJointTCI-commonMultiCC-r17 unifiedJointTCI-BeamAlignDLRS-r17 unifiedJointTCI-PC-association-r17 unifiedJointTCI-Legacy-r17 unifiedJointTCI-Legacy-SRS-r17 unifiedJointTCI-Legacy-CORESET0-r17 unifiedJointTCI-SCellBFR-r17 unifiedJointTCI-InterCell-r17 unifiedSeparateTCI-r17 unifiedSeparateTCI-multiMAC-CE-r17 unifiedSeparateTCI-perBWP-CA-r17 unifiedSeparateTCI-ListSharingCA-r17 unifiedSeparateTCI-commonMultiCC-r17 unifiedSeparateTCI-InterCell-r17 unifiedJointTCI-mTRP-InterCell-BM-r17 mpe-Mitigation-r17 srs-PortReport-r17 mTRP-PDCCH-individual-r17 mTRP-PDCCH-anySpan-3Symbols-r17 mTRP-PDCCH-TwoQCL-TypeD-r17 mTRP-PUSCH-CSI-RS-r17 mTRP-PUSCH-cyclicMapping-r17 mTRP-PUSCH-secondTPC-r17 mTRP-PUSCH-twoPHR-Reporting-r17 mTRP-PUSCH-A-CSI-r17 mTRP-PUSCH-SP-CSI-r17 mTRP-PUSCH-CG-r17 mTRP-PUCCH-MAC-CE-r17 mTRP-PUCCH-maxNum-PC-FR1-r17 mTRP-inter-Cell-r17 mTRP-GroupBasedL1-RSRP-r17 mTRP-BFD-RS-MAC-CE-r17 mTRP-CSI-EnhancementPerBand-r17 codebookComboParameterMultiTRP-r17 mTRP-CSI-additionalCSI-r17 mTRP-CSI-N-Max2-r17 mTRP-CSI-CMR-r17 srs-partialFreqSounding-r17 beamSwitchTiming-v1710 beamSwitchTiming-r17 beamReportTiming-v1710 maxNumberRxTxBeamSwitchDL-v1710 srs-PortReportSP-AP-r17 sfn-ImplicitRS-twoTCI-r17 sfn-QCL-TypeD-Collision-twoTCI-r17 mTRP-CSI-numCPU-r17 | No impact |
| PDCP-Parameters | longSN-RedCap-r17 udc-r17 | Added as * in cr_NR_PDCP_Parameters |
| Phy-ParametersCommon | mux-HARQ-ACK-withoutPUCCH-onPUSCH-r16 guardSymbolReportReception-IAB-r17 restricted-IAB-DU-BeamReception-r17 recommended-IAB-MT-BeamTransmission-r17 case6-TimingAlignmentReception-IAB-r17 case7-TimingAlignmentReception-IAB-r17 dl-tx-PowerAdjustment-IAB-r17 desired-ul-tx-PowerAdjustment-r17 fdm-SoftResourceAvailability-DynamicIndication-r17 updated-T-DeltaRangeRecption-r17 slotBasedDynamicPUCCH-Rep- sps-HARQ-ACK-Deferral-r17 unifiedJointTCI-commonUpdate-r17 mTRP-PDCCH-singleSpan-r17 supportedActivatedPRS-ProcessingWindow-r17 cg-TimeDomainAllocationExtension-r17 ta-BasedPDC-TN-NonSharedSpectrumChAccess-r17 directionalCollisionDC-IAB-r17 | Added as * in cr_Phy_ParametersCommon |
| Phy-ParametersFRX-Diff | cqi-4-BitsSubbandTN-NonSharedSpectrumChAccess-r17 | Added as * in cr_NR_Phy_ParametersFRX_Diff |
| Phy-ParametersFR1 | pdcch-MonitoringSingleSpanFirst4Sym-r16 | Added as * in cr_NR_Phy_ParametersFR1 |
| RF-Parameters | supportedBandCombinationList-UplinkTxSwitch-v1670 supportedBandCombinationList-v1680 supportedBandCombinationList-v1690 supportedBandCombinationList-UplinkTxSwitch-v1690 supportedBandCombinationList-v1700 supportedBandCombinationList-UplinkTxSwitch-v1700 supportedBandCombinationListSL-RelayDiscovery-r17 supportedBandCombinationListSL-NonRelayDiscovery-r17 supportedBandCombinationListSidelinkEUTRA-NR-v1710 sidelinkRequested-r17 extendedBand-n77-2-r17 supportedBandCombinationList-v1720 supportedBandCombinationList-UplinkTxSwitch-v1720 | Added as * in cr_NR_RF_Parameters |
| BandNR | maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 txDiversity-r16 pdsch-1024QAM-FR1-r17 ue-PowerClass-v1700 fr2-2-AccessParamsPerBand-r17 rlm-Relaxation-r17 bfd-Relaxation-r17 cg-SDT-r17 locationBasedCondHandover-r17 timeBasedCondHandover-r17 eventA4BasedCondHandover-r17 mn-InitiatedCondPSCellChangeNRDC-r17 sn-InitiatedCondPSCellChangeNRDC-r17 pdcch-SkippingWithoutSSSG-r17 sssg-Switching-1BitInd-r17 sssg-Switching-2BitInd-r17 pdcch-SkippingWithSSSG-r17 searchSpaceSetGrp-switchCap2-r17 uplinkPreCompensation-r17 uplink-TA-Reporting-r17 max-HARQ-ProcessNumber-r17 type2-HARQ-Codebook-r17 type1-HARQ-Codebook-r17 type3-HARQ-Codebook-r17 ue-specific-K-Offset-r17 multiPDSCH-SingleDCI-FR2-1-SCS-120kHz-r17 multiPUSCH-SingleDCI-FR2-1-SCS-120kHz-r17 parallelPRS-MeasRRC-Inactive-r17 nr-UE-TxTEG-ID-MaxSupport-r17 prs-ProcessingRRC-Inactive-r17 prs-ProcessingWindowType1A-r17 prs-ProcessingWindowType1B-r17 prs-ProcessingWindowType2-r17 srs-AllPosResourcesRRC-Inactive-r17 olpc-SRS-PosRRC-Inactive-r17 spatialRelationsSRS-PosRRC-Inactive-r17 maxNumberPUSCH-TypeA-Repetition-r17 puschTypeA-RepetitionsAvailSlot-r17 tb-ProcessingMultiSlotPUSCH-r17 tb-ProcessingRepMultiSlotPUSCH-r17 maxDurationDMRS-Bundling-r17 pusch-RepetitionCRC-r17 sharedSpectrumChAccessParamsPerBand-v1710 parallelMeasurementWithoutRestriction-r17 maxNumber-NGSO-SatellitesWithinOneSMTC-r17 k1-RangeExtension-r17 aperiodicCSI-RS-FastScellActivation-r17 aperiodicCSI-RS-AdditionalBandwidth-r17 bwp-WithoutCD-SSB-OrNCD-SSB-RedCap-r17 halfDuplexFDD-TypeA-RedCap-r17 posSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 channelBWs-DL-SCS-480kHz-FR2-2-r17 channelBWs-UL-SCS-480kHz-FR2-2-r17 channelBWs-DL-SCS-960kHz-FR2-2-r17 channelBWs-UL-SCS-960kHz-FR2-2-r17 ul-GapFR2-r17 oneShotHARQ-feedbackTriggeredByDCI-1-2-r17 oneShotHARQ-feedbackPhy-Priority-r17 enhancedType3-HARQ-CodebookFeedback-r17 triggeredHARQ-CodebookRetx-r17 ue-OneShotUL-TimingAdj-r17 pucch-Repetition-F0-2-r17 cqi-4-BitsSubbandNTN-SharedSpectrumChAccess-r17 mux-HARQ-ACK-DiffPriorities-r17 ta-BasedPDC-NTN-SharedSpectrumChAccess-r17 ack-NACK-FeedbackForMulticastWithDCI-Enabler-r17 maxNumberG-RNTI-r17 dynamicMulticastDCI-Format4-2-r17 maxModulationOrderForMulticast-r17 dynamicSlotRepetitionMulticastTN-NonSharedSpectrumChAccess-r17 dynamicSlotRepetitionMulticastNTN-SharedSpectrumChAccess-r17 nack-OnlyFeedbackForMulticastWithDCI-Enabler-r17 ack-NACK-FeedbackForSPS-MulticastWithDCI-Enabler-r17 maxNumberG-CS-RNTI-r17 re-LevelRateMatchingForMulticast-r17 pdsch-1024QAM-2MIMO-FR1-r17 prs-MeasurementWithoutMG-r17 maxNumber-LEO-SatellitesPerCarrier-r17 prs-ProcessingCapabilityOutsideMGinPPW-r17 srs-SemiPersistent-PosResourcesRRC-Inactive-r17 channelBWs-DL-SCS-120kHz-FR2-2-r17 channelBWs-UL-SCS-120kHz-FR2-2-r17 | No impact |
| RF-ParametersMRDC | supportedBandCombinationList-UplinkTxSwitch-v1670 supportedBandCombinationList-v1700 supportedBandCombinationList-UplinkTxSwitch-v1700 supportedBandCombinationList-v1720 supportedBandCombinationListNEDC-Only-v1720 supportedBandCombinationList-UplinkTxSwitch-v1720 | Added as * in cr_RF_ParametersMRDC |
| RLC-Parameters | am-WithLongSN-RedCap-r17 | Added as * in cr_NR_RLC_Parameters |
| SidelinkParametersNR-r16 | relayParameters-r17 p0-OLPC-Sidelink-r17 | Added as * in cr_SidelinkParametersNR_r16 |
| MAC-ParametersSidelinkCommon-r16 | drx-OnSidelink-r17 | No impact |
| BandSidelink-r16 | ue-PowerClassSidelink-r16 sl-TransmissionMode2-RandomResourceSelection-r17 sync-Sidelink-v1710 enb-sync-Sidelink-v1710 rx-IUC-Scheme1-PreferredMode2Sidelink-r17 rx-IUC-Scheme1-NonPreferredMode2Sidelink-r17 rx-IUC-Scheme2-Mode2Sidelink-r17 rx-IUC-Scheme1-SCI-r17 rx-IUC-Scheme1-SCI-ExplicitReq-r17 | No impact |
| SON-Parameters-r16 | rlfReportCHO-r17 rlfReportDAPS-r17 success-HO-Report-r17 twoStepRACH-Report-r17 pscell-MHI-Report-r17 onDemandSI-Report-r17 | No impact |
| UE-BasedPerfMeas-Parameters-r16 | sigBasedLogMDT-OverrideProtect-r17 multipleCEF-Report-r17 excessPacketDelay-r17 earlyMeasLog-r17 | Added as * in cr_UE_NR_Capability_v1610_IEs |
| UE-MeasurementsAvailable-r16 | successHO-InfoAvailable-r17 sigLogMeasConfigAvailable-r17 | Added as * in cr_UE_MeasurementsAvailable_r16 and cr_RRCResumeComplete_ue_MeasurementsAvailable_r16 cr_RRCResumeComplete_ue_MeasurementsAvailable_r16 updated to use cr_UE_MeasurementsAvailable_r16 |
| VisitedCellInfo-r16 | visitedPSCellInfoListReport-r17 | No impact |
| SL-BWP-Config-r16 | sl-BWP-PoolConfigPS-r17 sl-BWP-DiscPoolConfig-r17 | No impact |
| SL-BWP-ConfigCommon-r16 | sl-BWP-PoolConfigCommonPS-r17 sl-BWP-DiscPoolConfigCommon-r17 | Added as omit in cs_38508_SL_BWP_ConfigCommon |
| SL-ConfigDedicatedNR-r16 | sl-PHY-MAC-RLC-Config-v1700 sl-DiscConfig-r17 | No impact |
| SL-PSBCH-Config-r16 | dl-P0-PSBCH-r17 | No impact |
| SL-ResourcePool-r16 | sl-PBPS-CPS-Config-r17 sl-InterUE-CoordinationConfig-r17 | Added as omit in cs_SL_ResourcePoolDef |
| SL-PowerControl-r16 | dl-P0-PSSCH-PSCCH-r17 sl-P0-PSSCH-PSCCH-r17 dl-P0-PSFCH-r17 | No impact |

### Further ASN.1 changes

| ASN.1 type affected | Type change | Impact on TTCN |
| --- | --- | --- |
| UL-DCCH-MessageType | New choice values added | No impact |
| SystemInformation-IEs | New choice values added | No impact |
| RA-Report-r16 | Embedded type PCI-ARFCN-NR-r16 created | No impact |
| RLF-Report-r16 | Embedded type PCI-ARFCN-NR-r16 created | No impact |
| MeasObjectToAddMod | New choice values added | No impact |
| MeasResults | New choice values added | No impact |
| EventTriggerConfigInterRAT | New choice values added | No impact |
| ReportConfigNR | New choice values added | No impact |
| CondTriggerConfig-r16 | New choice values added | No impact |
| VisitedCellInfo-r16 | Embedded type PCI-ARFCN-NR-r16 created | No impact |

## NR_SideLink_Preconf

### Fields added with extension marker (ellipsis)

| ASN.1 type affected | New fields | Impact on TTCN |
| --- | --- | --- |
| SidelinkPreconfigNR-r16 | sl-DRX-PreConfigGC-BC-r17 sl-TxProfileList-r17 sl-PreconfigDiscConfig-r17 | No impact |

## NR_PC5_RRC_Definitions

### Type change of nonCriticalExtension

| ASN.1 type affected | New nonCriticalExtension type |
| --- | --- |
| RRCReconfigurationSidelink-IEs-r16 | RRCReconfigurationSidelink-v1700-IEs |
| RRCReconfigurationCompleteSidelink-IEs-r16 | RRCReconfigurationCompleteSidelink-v1710-IEs |
| UECapabilityInformationSidelink-IEs-r16 | UECapabilityInformationSidelink-v1700-IEs |

no changes in TTCN

### Name changes

| ASN.1 type affected | Renaming | Impact on TTCN |
| --- | --- | --- |
| MeasurementReportSidelink-IEs-r16 | Type renamed to MeasurementReportSidelink-r16-IEs | No impact |
| RRCReconfigurationSidelink-IEs-r16 | Type renamed to RRCReconfigurationSidelink-r16-IEs | New types created and used in many templates RRCReconfigurationSidelink_SLRB_ConfigToAddModList_Type RRCReconfigurationSidelink_SLRB_ConfigToReleaseList_Type RRCReconfigurationSidelink_MeasConfig_Type RRCReconfigurationSidelink_CSI_RS_Config_Type |
| RRCReconfigurationCompleteSidelink-IEs-r16 | Type renamed to RRCReconfigurationCompleteSidelink-r16-IEs | No impact |
| RRCReconfigurationFailureSidelink-IEs-r16 | Type renamed to RRCReconfigurationFailureSidelink-r16-IEs | No impact |
| UECapabilityEnquirySidelink-IEs-r16 | Type renamed to UECapabilityEnquirySidelink-r16-IEs | No impact |
| UECapabilityInformationSidelink-IEs-r16 | Type renamed to UECapabilityInformationSidelink-r16-IEs | No impact |

### Fields added with extension marker (ellipsis)

| ASN.1 type affected | New fields | Impact on TTCN |
| --- | --- | --- |
| BandSidelinkPC5-r16 | rx-IUC-Scheme1-PreferredMode2Sidelink-r17 rx-IUC-Scheme1-NonPreferredMode2Sidelink-r17 rx-IUC-Scheme2-Mode2Sidelink-r17 rx-IUC-Scheme1-SCI-r17 rx-IUC-Scheme1-SCI-ExplicitReq-r17 scheme2-ConflictDeterminationRSRP-r17 | No impact |

### Further ASN.1 changes

| ASN.1 type affected | Type change | Impact on TTCN |
| --- | --- | --- |
| SCCH-MessageType | New choice values added | No impact |

# Baseline Moving – 38.321

38.321 is upgraded to 38.321-h20.

No change is required in TTCN.

# Baseline Moving – 38.322

  - is upgraded to 38.322-h20.

No change is required in TTCN.

# Baseline Moving – 38.323

New types required for Uplink Data Compression and Decompression

type B3_Type NR_PDCP_CtrlPduType_Type;                                /* PDU type according to TS 38.323 clause 6.3.8:

* 000  PDCP status report

* 001  Interspersed ROHC feedback

* 010  EHC feedback

* 011-111   Reserved */

* 011  UDC feedback // New  PDU type UDC

* 100-111 Reserved */

type record NR_PDCP_CtrlPduUdcFeedback_Type {

B1_Type                             D_C,         /* 1 bit,  '0'B for Ctrl PDU */

NR_PDCP_CtrlPduType_Type            PDU_Type,    /* 3 bits, '011'B for UDC feedback */

B1_Type                             FE,          /* 1 bit, '0' for NoError,  '1' for Checksum Error Notification  */

B3_Type                             Reserved     /* 3 bit reserved */

};

type union NR_PDCP_PDU_Type {

NR_PDCP_DataPduSN12Bits_Type                DataPduSN12Bits,

NR_PDCP_DataPduSN18Bits_Type                DataPduSN18Bits,

NR_PDCP_CtrlPduStatus_Type                  CtrlPduStatus,

NR_PDCP_CtrlPduRohcFeedback_Type            CtrlPduRohcFeedback,

NR_PDCP_CtrlPduEhcFeedbackCID7Bits_Type     CtrlPduEhcFeedbackCID7Bits,

NR_PDCP_CtrlPduEhcFeedbackCID15Bits_Type    CtrlPduEhcFeedbackCID15Bits,

NR_PDCP_CtrlPduUdcFeedback_Type	            CtrlPduUdcFeedback

};

# Baseline Moving – 38.212 38.214

The changes in DCI ASP types documented in R5w220305 have impact on the following templates:

- cs_NR_DciFormat_0_1_ChannelAccessCPextCAPC_None also renamed to cs_NR_DciFormat_0_X_ChannelAccessCPextCAPC_None

- cs_NR_DciFormat_0_1_Params

- cs_NR_DciFormat_0_1_AllOmit

- cs_NR_DciFormat_1_0_P_RNTI_Params

- cs_NR_DciFormat_1_1_Params

- cs_NR_DciFormat_1_1_AllOmit

- cs_NR_DciFormat_1_2_Params

- cs_NR_DciFormat_0_2_Params

- cs_NR_DciFormat_0_1_Prioirity0, also modified to make use of cs_NR_DciFormat_0_1_AllOmit and renamed to cds_NR_DciFormat_0_1_Priority0

- cs_NR_DciFormat_0_1_Prioirity1, also modified to make use of cs_NR_DciFormat_0_1_AllOmit and renamed to cds_NR_DciFormat_0_1_Priority1

- cs_PSCCH_Sci1A

# Baseline Moving – 37.324

37.324 is upgraded to 37.324-h00.

No change is required in TTCN.

# Baseline Moving – 24.501 (NAS)

24.501 is upgraded to 24.501-h80.

## Modified Message Definitions

| PDU Definition | Changes Required | Ref: 3GPP 24.501 clause |
| --- | --- | --- |
| Registration Request | New optional fields: UE Request Type, Paging Restriction, Service-level-AA container, NID, MS determined PLMN with disaster condition, Requested PEIPS assistance information | 8.2.6 |
| Registration Accept | New optional fields: Extended rejected NSSAI, Service-level-AA container, Negotiated PEIPS assistance information, 5GS additional request result, NSSRG information, Disaster roaming wait range, Disaster return wait range, List of PLMNs to be used in disaster condition, Forbidden TAI(s) for the list of 5GS forbidden tracking areas for roaming, Forbidden TAI(s) for the list of 5GS forbidden tracking areas for regional provision of service, Extended CAG information list, NSAG information | 8.2.7 |
| Registration Reject | New optional fields: Extended rejected NSSAI, Disaster return wait range, Extended CAG information list, Lower bound timer value, Forbidden TAI(s) for the list of 5GS forbidden tracking areas for roaming, Forbidden TAI(s) for the list of 5GS forbidden tracking areas for regional provision of service | 8.2.9 |
| DL NAS Transport | New optional field: Lower bound timer value | 8.2.11 |
| Deregistration Request MT | New optional fields: Extended rejected NSSAI, Disaster return wait range, Extended CAG information list, Lower bound timer value, Forbidden TAI(s) for the list of 5GS forbidden tracking areas for roaming, Forbidden TAI(s) for the list of 5GS forbidden tracking areas for regional provision of service | 8.2.14 |
| Service Request | New optional fields: UE request type, Paging restriction | 8.2.16 |
| Service Accept | New optional fields: 5GS additional request result, Forbidden TAI(s) for the list of 5GS forbidden tracking areas for roaming, Forbidden TAI(s) for the list of 5GS forbidden tracking areas for regional provision of service | 8.2.17 |
| Service Reject | New optional fields: Disaster return wait range, Extended CAG information list, Lower bound timer value, Forbidden TAI(s) for the list of 5GS forbidden tracking areas for roaming, Forbidden TAI(s) for the list of 5GS forbidden tracking areas for regional provision of service | 8.2.18 |
| Configuration Update Command | New optional fields: Extended rejected NSSAI, Service-level-AA container, NSSRG information, Disaster roaming wait range, Disaster return wait range, List of PLMNs to be used in disaster condition, Extended CAG information list, Updated PEIPS assistance information, NSAG information, Priority indicator | 8.2.19 |
| Control Plane Service Request | New optional fields: Allowed PDU session status, UE request type, Paging restriction | 8.2.30 |
| PDU Session Establishment Request | New optional fields: Service-level-AA container, Requested MBS container, PDU session pair id, RSN | 8.3.1 |
| PDU Session Establishment Accept | New optional fields: Service-level-AA container, Received MBS container | 8.3.2 |
| PDU Session Establishment Reject | New optional field: Service-level-AA container | 8.3.3 |
| PDU Session Modification Request | New optional fields: Requested MBS container, Service-level-AA container | 8.3.7 |
| PDU Session Modification Command | New optional fields: Received MBS container, Service-level-AA container | 8.3.9 |
| PDU Session Release Command | New optional field: Service-level-AA container | 8.3.14 |

## Modified Information Elements

| PDU Definition | Changes Required | Ref: 3GPP 24.501 clause |
| --- | --- | --- |
| 5GMM Capability | New fields: Spare, 5G ProSe-l2 relay, 5G ProSe- direct communication, 5G ProSe-direct discovery, Extd Rejected-NSSAI, Paging Restriction, Reject Paging Request, Paging Ind for Voice, N1 NAS signalling connection release, NR-Paging subgroup support ind, 5G ProSe-l3 remote, 5G ProSe-l2 remote, 5G ProSe-l3 relay, NSAG, Extd CAG, SOR-SNPN-SI, Event Notification, Minimization of service interruption, NSSRG Removed fields:Spare | 9.3.11.1 |
| 5GMM Network Feature Support | New fields: Paging Restriction, Reject Paging Request, Paging Ind for Voice, N1 NAS signalling connection release Removed fields:Spare | 9.3.11.5 |
| Registration Result | New field: Disaster roaming registration result value Removed field:Spare | 9.11.3.6 |
| SOR Header | New field: Additional Parameters Removed field:Spare Renamed fields: ListType to ListType_MSSNPNSI, ListInd to Listnd_MSSI | 9.11.3.51 |
| 5GSM Capability | New fields: Spare, Access Performance Measurements per QoS flow Rule | 9.11.4.1 |
| 5GSM Congestion Re-attempt Indicator | New field: Current Access Type Back off Timer Removed field:Spare | 9.11.4.21 |

## New Information Elements

| PDU Definition | Ref: 3GPP 24.501 clause |
| --- | --- |
| Service-level-AA Container | 9.11.2.10 |
| Service-level Device Id | 9.11.2.11 |
| Service-level-AA Server Address | 9.11.2.12 |
| Service-level-AA Payload | 9.11.2.13 |
| Service-level- Response | 9.11.2.14 |
| Service-level-AA Payload Type | 9.11.2.15 |
| Service-level-AA Pending Indication | 9.11.2.17 |
| Extended Rejected NSSAI | 9.11.3.75 |
| UE Request Type | 9.11.3.76 |
| Paging Restriction | 9.11.3.77 |
| NID | 9.11.3.79 |
| PEIPS Assistance Information | 9.11.3.80 |
| 5GS Additional Request Result | 9.11.3.81 |
| NSSRG Information | 9.11.3.82 |
| List of PLMNs to be used in disaster condition | 9.11.3.83 |
| Registration Wait Range | 9.11.3.84 |
| PLMN Identity | 9.11.3.85 |
| Extended CAG Information List | 9.11.3.86 |
| NSAG Information | 9.11.3.87 |
| Priority Indicator | 9.11.3.91 |
| Requested MBS Container | 9.11.4.30 |
| Received MBS Container | 9.11.4.31 |
| PDU Session Pair Id | 9.11.4.32 |
| RSN | 9.11.4.33 |

# Baseline Moving – 24.587 (V2X)

24.587 is upgraded to 24.587-h70.

## Modified Information Elements

| PDU Definition | Changes Required | Ref: 3GPP 24.587 clause |
| --- | --- | --- |
| Requested UE Policies | New fields: 5G ProSe USage Information Reporting Indicator, 5G ProSe L2 Remote UE Indicator , 5G ProSe L3 Remote UE Indicator, 5G ProSe L2 UE to Network Relay Indicator, 5G ProSe L3 UE to Network Relay Indicator, 5G ProSe Direct Communications Indicator, 5G ProSe Direct Discovery Indicator, Spare, 5G ProSe Usage Information Reporting Indicator Removed fields:Spare | 8.3.2 |

# Baseline Moving – 24.588 (V2X)

24.588 is upgraded to 24.588-h10.

## Modified Information Elements

| PDU Definition | Changes Required | Ref: 3GPP 24.588 clause |
| --- | --- | --- |
| V2X Service Identifier to PC5 RAT and Tx Profiles Mapping Rule | New fields: Broadcast and Groupcast mode NR PC5 Tx Profile Indicator, Broadcast and Groupcast mode NR PC5 Tx Profile Removed field:Spare | 5.3.1.13 |
| V2X Communication over PC5 in NR-PC5 | New fields: PC5 DRX Configuration for Broadcast and Groupcast Indicator, PC5 DRX Configuration for Broadcast and Groupcast Removed field:Spare | 5.3.1.31 |

## New Information Elements

| PDU Definition | Ref: 3GPP 24.588 clause |
| --- | --- |
| PC5 DRX Configuration for Broadcast and Groupcast | 5.3.1.55 |
| PC5 QoS Profile to PC5 DRX Cycle Mapping Rules | 5.3.1.56 |
| PC5 QoS Profile to PC5 DRX Cycle Mapping Rule | 5.3.1.57 |
| Default PC5 DRX Configuration | 5.3.1.58 |

# Other changes

## ASP type SPS_Config_R16

The type definition of SPS_Config_R16 does not follow the normal naming rules and the field names start with lower case instead of capital letter.

type record SPS_Config_R16 {

SPS_ConfigToAddModList_r16          Sps_ConfigToAddModList_r16          optional,

SPS_ConfigToReleaseList_r16         Sps_ConfigToReleaseList_r16         optional,

SPS_ConfigDeactivationStateList_r16 Sps_ConfigDeactivationStateList_r16 optional

};

## f_NR5GC_RRC_ProcedureDelayReestablishment

The reception of HARQ error is duplicated in the function f_NR5GC_RRC_ProcedureDelayReestablishment.

function f_NR5GC_RRC_ProcedureDelayReestablishment(NR_CellId_Type                        p_NR_CellId,

template (value) NR_SRB_COMMON_REQ    p_SRB_COMMON_REQ,

template (present) NR_SRB_COMMON_IND  p_SRB_COMMON_IND,

integer                               p_Nms,

charstring                            p_TestcaseStep

) runs on NR_BASE_PTC return NR_SRB_COMMON_IND

{

var integer v_T1Slots;

var integer v_T2Slots;

…

alt {

/* HARQ NACK */

[]SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, nack)) {

v_DL_RetransmissionCnt := v_DL_RetransmissionCnt + 1;

if (v_DL_RetransmissionCnt > 4) {

t_Watchdog.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "DL Msg retransmissions > 4");

} else {

repeat;

}

}

/* HARQ ACK */

[]SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_NR_CellId, ack))-> value v_SYS_IND {

v_T2 := v_SYS_IND.Common.TimingInfo.SubFrame;

f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_UL_HARQ(disable), -, tsc_NoCnfReq);    /* disable indication of HARQ ack/nack; @sic R5s220683 sic@*/

v_T2Flag := true;

repeat;

}

/* HARQ error indication */

[] SYSIND.receive (car_NR_SYSTEM_HARQ_ERROR(p_NR_CellId, cr_NR_HarqErrorUL_Any)) {

f_SetVerdictInconc(__FILE__, __LINE__, p_TestcaseStep & "HARQ error for UL transmission in RRC Delay test");

t_Watchdog.stop;

}

/* UE initiates RACH procedure */

[] SYSIND.receive(car_NR_PRACH_Preamble_IND(?, cr_TimingInfo_Any, ?)) {

f_NR_SS_SystemIndCtrlConfig(p_NR_CellId, cds_NR_SystemIndCtrl_RachPreamble(disable), -, tsc_NoCnfReq);  // @sic R5s220683 sic@

v_RACH_ProcedureStarted := true;

repeat;   //@sic R5s220813 sic@

}

[] SRB.receive(p_SRB_COMMON_IND) -> value v_ReceivedNR_SRB_COMMON_IND {

v_T4 := v_ReceivedNR_SRB_COMMON_IND.Common.TimingInfo.SubFrame;

v_T4Flag := true;

t_Watchdog.stop;

}

[] t_Watchdog.timeout {  /* timeout => after T5 */

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, p_TestcaseStep & "t_Watchdog timeout");

}

}

…

}

## Changes for Audit test cases

Module RRC_UECapability_ENDC_EUTRA

function f_EUTRA38_TxUECapEnquiry_RxUECapabilityInformation_TP1(EUTRA_CellId_Type         p_CellId,

RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI_Def) runs on EUTRA_PTC

{ // @sic R5-211501 R5-216277 sic@

var UE_EUTRA_Capability v_RxEutraCapability;

var template UE_EUTRA_Capability_v1510_IEs v_EutraCapability_Expected;

var UE_EUTRA_Capability_v1510_IEs v_Rx_Decoded_EutraCapability;

var boolean v_Completed_AllSegments := false;

//@siclog "Step 1" siclog@

// The SS transmits a UECapabilityEnquiry message including eutra with rrc-segAllowed-r16 set to enabled

SRB.send(cas_SRB1_RrcPdu_REQ(p_CellId, cs_TimingInfo_Now, cs_508_UeCapabilityEnquiry_MRDC_r15IEs_true_SegAllowed_r16_enabled(p_RRC_TI)));

v_RxEutraCapability := f_EUTRA_GetCaps(p_CellId, p_RRC_TI);  // @sic R5-217903 sic@

v_Rx_Decoded_EutraCapability := v_RxEutraCapability.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension;

select (v_RxEutraCapability.accessStratumRelease) { //As per TS36.523-1 Table 8.5.4.1.3.3/2; details not checked in the scope of the present NR RRC TC 8.2.1.1.1

case (rel8, rel9, rel10, rel11, rel12, rel13, rel14) {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "AccessStratumRelease < release15");

}

case (rel15) {

v_EutraCapability_Expected := cr_UeCapabilitiesRAT_Container_ENDC_r15;

}

case (rel16) { // @sic R5s201387 BASELINE MOVING 2020 sic@

v_EutraCapability_Expected := cr_UeCapabilitiesRAT_Container_ENDC_r16;

fl_EUTRACap_CommonChecks_r16(v_RxEutraCapability);

}

case (rel17) { // @sic R5s221179 BASELINE MOVING 2022 sic@

v_EutraCapability_Expected := cr_UeCapabilitiesRAT_Container_ENDC_r17;

fl_EUTRACap_CommonChecks_r17(v_RxEutraCapability);

}

case else  {

f_SetVerdictInconc (__FILE__, __LINE__, "AccessStratumRelease not applicable for the purpose of tc8.2.1.1.1");

}

}

// Match the received UE-EUTRA-CapabilityInformation with the expected UE_EUTRA_Capability template

fl_EUTRA_UE_Capability_MatchTemplates(v_Rx_Decoded_EutraCapability, v_EutraCapability_Expected);

if (v_Completed_AllSegments) {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2a2");

}

else {

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2b1");

}

};

…

template UE_EUTRA_Capability_v1550_IEs cr_UE_EUTRA_Capability_v1550_IEs (template UE_EUTRA_Capability_v1630_IEs p_UE_EUTRA_Capability_v1630_IEs := *) := // @sic R5s201387 BASELINE MOVING 2020; R5s221178 BASELINE MOVING 2022 sic@

{ /* @status    APPROVED (ENDC) */

…

neighCellSI_AcquisitionParameters_v1610 := *,

mobilityParameters_v1610                := *

} ifpresent,  //UE_EUTRA_CapabilityAddXDD_Mode_v1610

nonCriticalExtension := p_UE_EUTRA_Capability_v1630_IEs

} ifpresent // UE-EUTRA-Capability-v1610-IEs

} ifpresent // UE-EUTRA-Capability-v15a0-IEs

} ifpresent // UE-EUTRA-Capability-v1570-IEs

} ifpresent // UE-EUTRA-Capability-v1560-IEs

} ifpresent; // UE-EUTRA-Capability-v1550-IEs

…

template UE_EUTRA_Capability_v1630_IEs cr_UE_EUTRA_Capability_v1630_IEs :=

{ /* @sic R5s221178 BASELINE MOVING 2022 sic@ */

/* @status    */

rf_Parameters_v1630 := *,

sl_Parameters_v1630 := *,

earlySecurityReactivation_r16 := *,

mac_Parameters_v1630 := ?,

measParameters_v1630 := *,

fdd_Add_UE_EUTRA_Capabilities_v1630 := ?,

tdd_Add_UE_EUTRA_Capabilities_v1630 := ?,

nonCriticalExtension := {

otherParameters_v1650 := *,

nonCriticalExtension := {

irat_ParametersNR_v1660 := ?,

nonCriticalExtension := {

other_Parameters_v1690 := ?,

nonCriticalExtension := {

measParameters_v1700 := *,

ue_BasedNetwPerfMeasParameters_v1700 := *,

phyLayerParameters_v1700 := ?,

ntn_Parameters_r17 := *,

irat_ParametersNR_v1700 := *,

mbms_Parameters_v1700 := ?,

nonCriticalExtension := {

irat_ParametersNR_v1710 := ?,

neighCellSI_AcquisitionParameters_v1710 := *,

sl_Parameters_v1710 := *,

sidelinkRequested_r17 := *,

nonCriticalExtension := {

ntn_Parameters_v1720 := ?,

nonCriticalExtension := *

} ifpresent // UE-EUTRA-Capability-v1720-IEs

} ifpresent // UE-EUTRA-Capability-v1710-IEs

} ifpresent // UE-EUTRA-Capability-v1700-IEs

} ifpresent // UE-EUTRA-Capability-v1690-IEs

} ifpresent // UE-EUTRA-Capability-v1660-IEs

} ifpresent // UE-EUTRA-Capability-v1650-IEs

} ifpresent; // UE-EUTRA-Capability-v1630-IEs

…

template (present) UE_EUTRA_Capability_v1510_IEs cr_UeCapabilitiesRAT_Container_ENDC_r17 := // @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@

/* @status    APPROVED (ENDC) */

cr_UE_EUTRA_Capability_v1510_IEs(cr_UE_EUTRA_Capability_v1550_IEs(cr_UE_EUTRA_Capability_v1630_IEs));

…

/*

* @desc      Function to check some Rel 17 UE EUTRA capabilities. Prints out all the PICS wrongly set specific within r17

* @param     p_EUTRACapability

* @status    APPROVED (ENDC)

*/

function fl_EUTRACap_CommonChecks_r17(UE_EUTRA_Capability p_EUTRACapability)

{

fl_EUTRACap_CommonChecks_r16(p_EUTRACapability);

} // end of fl_EUTRACap_CommonChecks_r17

Module NR_CapabilityFunctions

template UE_NR_Capability_v1640 cr_UE_NR_Capability_v1640_IEs := // @sic R5s211246 R5-225384 R5s221155 R5s221179 sic@

{ /* @status    APPROVED (ENDC, NR5GC) */

redirectAtResumeByNAS_r16                   := *,

phy_ParametersSharedSpectrumChAccess_r16    := *,

nonCriticalExtension                        := {

mpsPriorityIndication_r16     := *,

highSpeedParameters_v1650     := *,

nonCriticalExtension          := {

ul_RRC_Segmentation_r16   := *,

nonCriticalExtension      := {

inactiveStatePO_Determination_r17 := *,

highSpeedParameters_v1700         := *,

powSav_Parameters_v1700           := *,

mac_Parameters_v1700              := *,

ims_Parameters_v1700              := *,

measAndMobParameters_v1700        := ?, //MeasAndMobParameters_v1700 non optional

appLayerMeasParameters_r17        := *,

redCapParameters_r17              := *,

ra_SDT_r17                        := *,

srb_SDT_r17                       := *,

gNB_SideRTT_BasedPDC_r17          := *,

bh_RLF_DetectionRecovery_Indication_r17  := *,

nrdc_Parameters_v1700                    := *,

bap_Parameters_v1700                     := *,

musim_GapPreference_r17                  := *,

musimLeaveConnected_r17                  := *,

mbs_Parameters_r17                       := ?, //MBS_Parameters_r17 non optional

nonTerrestrialNetwork_r17                := *,

ntn_ScenarioSupport_r17                  := *,

sliceInfoforCellReselection_r17          := *,

ue_RadioPagingInfo_r17                   := *,

ul_GapFR2_Pattern_r17                    := *,

ntn_Parameters_r17                       := *,

nonCriticalExtension                     := *

} ifpresent         // UE_NR_Capability_v1700 IEs

} ifpresent             // UE_NR_Capability_v1690 IEs

} ifpresent                 // UE_NR_Capability_v1650 IEs

} ifpresent; // UE_NR_Capability_v1640 IEs

template UE_NR_Capability_v1610 cr_UE_NR_Capability_v1610_IEs (template UE_NR_Capability_v1640 p_UE_NR_Capability_v1640_IEs := *) := // @sic R5s211246 R5-225384 R5s221155 R5s22XXXX sic@

{ /* @status    APPROVED (ENDC, NR5GC) */

inDeviceCoexInd_r16                    := *,

dl_DedicatedMessageSegmentation_r16    := *,

nrdc_Parameters_v1610                  := *,

powSav_Parameters_r16                  := f_Derive_PowSav_Parameters_r16(), // @sic R5-225384 sic@

fr1_Add_UE_NR_Capabilities_v1610       := *,

fr2_Add_UE_NR_Capabilities_v1610       := *,

bh_RLF_Indication_r16                  := *,

directSN_AdditionFirstRRC_IAB_r16      := *,

bap_Parameters_r16                     := *,

referenceTimeProvision_r16             := *,

sidelinkParameters_r16                 := { // Further checked in fl_Check_sidelinkParameters_r16 @sic R5-225384 sic@

sidelinkParametersNR_r16       := cr_SidelinkParametersNR_r16 ifpresent,

sidelinkParametersEUTRA_r16    := *

} ifpresent,

highSpeedParameters_r16                := *,

mac_Parameters_v1610                   := *,

mcgRLF_RecoveryViaSCG_r16              := *,

resumeWithStoredMCG_SCells_r16         := *,

resumeWithStoredSCG_r16                := *,

resumeWithSCG_Config_r16               := *,

ue_BasedPerfMeas_Parameters_r16        := {

barometerMeasReport_r16      := f_Derive_barometer_r16(), // pc_barometer_r16

immMeasBT_r16                := *,

immMeasWLAN_r16              := f_Derive_immMeasWLAN_r16(),  // pc_immMeasWLAN_r16

loggedMeasBT_r16             := *,

loggedMeasurements_r16       := f_Derive_loggedMeasurements_r16(),  // pc_loggedMeasurements_r16

loggedMeasWLAN_r16           := f_Derive_loggedMeasWLAN_r16(),      // pc_loggedMeasWLAN_r16

orientationMeasReport_r16    := f_Derive_orientation_r16(),         // pc_orientation_r16

speedMeasReport_r16          := f_Derive_speed_r16(),               // pc_speed_r16

gnss_Location_r16            := f_Derive_GNSS_location_r16(),       // pc_GNSS_location_r16

ulPDCP_Delay_r16             := f_Derive_PDCP_Delay_r16(),          // pc_PDCP_Delay_r16

sigBasedLogMDT_OverrideProtect_r17 := *, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

multipleCEF_Report_r17       := *,       /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

excessPacketDelay_r17        := *,       /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

earlyMeasLog_r17             := *        /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */

} ifpresent,

son_Parameters_r16              := f_Derive_son_Parameters_r16(),

onDemandSIB_Connected_r16              := *,

nonCriticalExtension                   := p_UE_NR_Capability_v1640_IEs

} ifpresent;

…

template (present) UE_NR_Capability cr_UeCapabilitiesRAT_Container_NR_r17 :=  /* @status    APPROVED (ENDC, NR5GC) */

cr_UE_NR_Capability(-,-, cr_UE_NR_Capability_v1530_IEs(cr_UE_NR_Capability_v1610_IEs(cr_UE_NR_Capability_v1640_IEs)));

…

function f_NR_CheckReleaseIndicator(AccessStratumRelease p_ReleaseIndicator_UE)

{

var integer v_RelMin;

var integer v_Rel;

var template (omit) integer v_UE_Release := f_NR_ConvertAccessStratumReleaseToInteger(p_ReleaseIndicator_UE);

if (isvalue(v_UE_Release)) {

v_Rel := valueof(v_UE_Release);

//select (p_ReleaseIndicator_UE) {

select (f_GetTestcaseAttrib_NR_Release (testcasename())) {

case ("") { // no attribute is assigned for Rel-15

v_RelMin := 15;  // >= rel15

}

case ("rel_16"){

v_RelMin := 16;  // >= rel16 - @sic R5s201387 BASELINE MOVING 2020 sic@

}

case ("rel_17"){

v_RelMin := 17;  // >= rel17 - @sic R5s221179 BASELINE MOVING 2022 sic@

}

case else {

FatalError(__FILE__, __LINE__, "");

}

}

if (v_Rel < v_RelMin) {

f_SetVerdictInconc (__FILE__, __LINE__, "Wrong Access Stratum release indicator");

}

}

}

…

function f_NR_NRFullCapabilityCheck (UE_NR_Capability p_NRCapability,

charstring p_OutputText := "PICS for condition on (pc_nrFDD or pc_nrTDD) not set",

charstring p_OutputText2 := "Received UE Capability template does not match with the expected template",

boolean p_TestIs_NR5GCOnly := false)

{ //@sic R5-194844 R5-225384 sic@

var UE_NR_Capability v_Received_NRCapability;

var template UE_NR_Capability v_NRCapability;

if ((not (pc_nrFDD or pc_nrTDD))){

FatalError(__FILE__, __LINE__, p_OutputText);

}

v_Received_NRCapability := p_NRCapability;

select (v_Received_NRCapability.accessStratumRelease) {

case (spare5, spare4, spare3, spare2, spare1) {  /* @sic R5s201387 BASELINE MOVING 2020 sic@ */

f_SetVerdictInconc (__FILE__, __LINE__, "AccessStratumRelease < rel15");

// a UE implementing relesase older than rel15 is not considered

}

case (rel15) { // @sic R5s200028 R5s200542 sic@

v_NRCapability := cr_UeCapabilitiesRAT_Container_NR_r15; // check on Rel 15 IEs vs. IEs provided

fl_NRCap_CommonChecks_r15(v_Received_NRCapability);

}

case (rel16) { /* @sic R5s201387 BASELINE MOVING 2020 sic@ */

v_NRCapability := cr_UeCapabilitiesRAT_Container_NR_r16; // check on Rel 16 IEs vs. IEs provided

fl_NRCap_CommonChecks_r16(v_Received_NRCapability, p_TestIs_NR5GCOnly);

}

case (rel17) { /* @sic R5s221179 BASELINE MOVING 2022 sic@ */

v_NRCapability := cr_UeCapabilitiesRAT_Container_NR_r17; // check on Rel 17 IEs vs. IEs provided

fl_NRCap_CommonChecks_r16(v_Received_NRCapability, p_TestIs_NR5GCOnly);

}

case else  {

f_SetVerdictInconc (__FILE__, __LINE__, "AccessStratumRelease not applicable");

// a UE implementing an unsupported release must not be failed

}

}

// Match the received UE-NR-CapabilityInformation with the expected UE_NR_Capability template

fl_NR_UE_Capability_MatchTemplates(v_Received_NRCapability, v_NRCapability, p_OutputText2);

}

Module NR_CapabilityFunctions_MRDC

template UE_MRDC_Capability_v1610 cr_UE_MRDC_Capability_v1610_IEs(template UE_MRDC_Capability_v1700 p_UE_MRDC_Capability_v1700_IEs := *):= /* @sic R5s201387 BASELINE MOVING 2020 sic@ */

{  /* @status    APPROVED (ENDC) */

measAndMobParametersMRDC_v1610   := {

measAndMobParametersMRDC_Common_v1610 := {

condPSCellChangeParametersCommon_r16  :=   {

condPSCellChangeFDD_TDD_r16 := *,

condPSCellChangeFR1_FR2_r16 := *

} ifpresent,

pscellT312_r16 := *

} ifpresent,

interNR_MeasEUTRA_IAB_r16    := *

} ifpresent,

generalParametersMRDC_v1610    := {

f1c_OverEUTRA_r16 := *

} ifpresent,

pdcp_ParametersMRDC_v1610      := *,

nonCriticalExtension           := p_UE_MRDC_Capability_v1700_IEs

} ifpresent;

template UE_MRDC_Capability_v1700 cr_UE_MRDC_Capability_v1700_IEs := /* @sic R5s221179 BASELINE MOVING 2022 sic@ */

{  /* @status    APPROVED (ENDC) */

measAndMobParametersMRDC_v1700   := {

measAndMobParametersMRDC_Common_v1700  := {

condPSCellChangeParameters_r17 := *,

condHandoverWithSCG_ENDC_r17   := *,

condHandoverWithSCG_NEDC_r17   := *

} ifpresent // MeasAndMobParametersMRDC-Common-v1700

},   // MeasAndMobParametersMRDC-v1700

nonCriticalExtension           := *

} ifpresent; //UE-MRDC-Capability-v1700

…

template (present) UE_MRDC_Capability cr_UeCapabilitiesRAT_Container_MRDC_r17 := /* @sic R5s221179 BASELINE MOVING 2022 sic@ */

/* @status    APPROVED (ENDC) */

cr_UE_MRDC_Capability(-, cr_UE_MRDC_Capability_v1560_IEs(cr_UE_MRDC_Capability_v1610_IEs(cr_UE_MRDC_Capability_v1700_IEs)));

…

function f_NR_MRDCFullCapabilityCheck (UE_MRDC_Capability p_MRDC_Capability, UE_NR_Capability.accessStratumRelease p_UE_AccessStratumRelease, NR_Configuration_Type p_Configuration_Type := EN_DC)

{

var UE_MRDC_Capability v_Received_MRDCCapability;

var template UE_MRDC_Capability v_MRDCCapability;

if (p_Configuration_Type == EN_DC) // Option 3

{

if ((not (pc_EN_DC))){

FatalError(__FILE__, __LINE__, "Table 8.2.1.1.1.3.3-7: PICS for condition on pc_EN_DC not set");

}

}

else if (p_Configuration_Type == NE_DC) // Option 4

{

if ((not (pc_NE_DC))){

FatalError(__FILE__, __LINE__, "Table 8.2.1.1.2.3.3-5: PICS for condition on pc_NE_DC not set");

}

}

else {

f_SetVerdictInconc (__FILE__, __LINE__, "Selected NR Configuration Type is not applicable");

}

v_Received_MRDCCapability := p_MRDC_Capability;

select (p_UE_AccessStratumRelease) {

case (rel15) {

v_MRDCCapability := cr_UeCapabilitiesRAT_Container_MRDC_r15;

fl_MRDCCap_CommonChecks_r15(v_Received_MRDCCapability, p_Configuration_Type); // to check some Rel 15 MRDC_Cap contents

}

case (rel16, rel17) {

v_MRDCCapability := cr_UeCapabilitiesRAT_Container_MRDC_r16;

fl_MRDCCap_CommonChecks_r16(v_Received_MRDCCapability, p_Configuration_Type);

}

case else  {

f_SetVerdictInconc (__FILE__, __LINE__, "AccessStratumRelease not applicable");

// UE with unsupported release is not failed. Anyway it will never reach this point if it is not an expected UE Release

}

}

// Match the received UE-NR-CapabilityInformation with the expected UE_NR_Capability template

fl_EUTRANR_UE_Capability_MatchTemplates(v_Received_MRDCCapability, v_MRDCCapability);

}

Module RRC_UECapability_NEDC_NR

function f_TC_8_2_1_1_2_NEDC_NR5GC_TestBody() runs on NR_BASE_PTC

{

…

f_NR_MobileInfo_NRCap_Set (v_NRCapability);

// Check the UE release

f_NR_CheckReleaseIndicator (v_NRCapability.accessStratumRelease);

select (v_NRCapability.accessStratumRelease) {

case (rel15, rel16, rel17){

}

case else{

FatalError (__FILE__, __LINE__, "accessStratumRelease not yet considered");

}

}

for (i := 0; i < lengthof(v_NRCapability.rf_Parameters.supportedBandListNR); i := i+1) {

if (match(v_NRCapability.rf_Parameters.supportedBandListNR[i].bandNR, v_NRBandInd)) {

v_Found1stBand := true;

break;

}

}

Module NR5GC_RRCSteps

function f_NR_Capability(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC

{

…

// Check the UE release

f_NR_CheckReleaseIndicator(v_NrCapability.accessStratumRelease);

select (v_NrCapability.accessStratumRelease) {

case (rel15, rel16, rel17) {  // @sic R5s201387 BASELINE MOVING 2020 R5s221179 BASELINE MOVING 2022 rel-17 sic@

}

case else{

FatalError (__FILE__, __LINE__, "accessStratumRelease not yet considered");

}

}

## NAS Changes

### Modified the definition of the CAG_Info NSSAI and NG_TrackingAreaUpdateList Types

The above types are used in the TLV, LV, and even V formats in several different messages.  In order to have a single and consistent type, these type definitions were split into LV (and V) parts.

type record CAG_Info {                              /* 24.501 cl. 9.11.3.18A */

Type4Length_Type            iel,

CAG_Info_V                  cagValue

};

type record CAG_Info_V {                              /* 24.501 cl. 9.11.3.18A, 9.11.3.86 */

NAS_PlmnId                  plmn,

B7_Type                     spareBits,

B1_Type                     cagOnly,

record of O4_Type  listOfCAGIds optional

};

type record S_NSSAI_Type {                               // 24.501 cl. 9.11.2.8

IEI8_Type                   iei                   optional, /* present in case of TLV; omit in case of LV */

S_NSSAI_LV                  lvPart

};

type record S_NSSAI_LV {                               // 24.501 cl. 9.11.2.8

Type4Length_Type            iel                   optional, /* present in case of LV or TLV; omit in case of V */

S_NSSAI_V                   vPart

};

type record S_NSSAI_V {                               // 24.501 cl. 9.11.2.8

O1_Type                     sst,

O3_Type                     sd                    optional,

O1_Type                     mappedSST             optional,

O3_Type                     mappedSD              optional

};

type record NG_TrackingAreaIdList {                              /* 24.501 cl. 9.11.3.9 */

IEI8_Type                   iei                   optional, /* present in case of TLV; omit in case of LV */

NG_TrackingAreaIdList_LV    lvPart

};

type record NG_TrackingAreaIdList_LV {                        /* 24.501 cl. 9.11.3.9 */

Type4Length_Type            iel,

record length (1..16) of NG_ListOfPartialTaiList  listOfPartialTais

};

The following message definitions have also been updated:

| Network Slice Specific Authentication Command | Modified field: changed S_NSSAI_Type (TLV) to S_NSSAI_LV (LV) |
| --- | --- |
| Network Slice Specific Authentication Complete | Modified field: changed S_NSSAI_Type (TLV) to S_NSSAI_LV (LV) |
| Network Slice Specific Authentication Result | Modified field: changed S_NSSAI_Type (TLV) to S_NSSAI_LV (LV) |

### Changes related to new DNNs

Updated PDU Session Establishment Request and Accept templates to include specific values if an URLLC DNN is selected