3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221220

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2825 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 8.1.6.1.2.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-10-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | From 38.331 clause 5.7.10.3, If the logMeasReport is included in the UEInformationResponse then UE will submit the UEInformationResponse message to lower layers for transmission via SRB2. #Associated prsoeCR rasied at RAN5#97 meeting |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified SRB for UEInformationResponse at step 19 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function f_TC_8_1_6_1_2_7_NR5GC_TestBody() |
| --- | --- |
| Reason for change | From 38.331 clause 5.7.10.3, If the logMeasReport is included in the UEInformationResponse then UE will submit the UEInformationResponse message to lower layers for transmission via SRB2. #Associated prsoeCR rasied at RAN5#97 meeting |
| Summary of change | Modified SRB for UEInformationResponse at step 19. |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_6_1_2_7_NR5GC_TestBody() runs on NR5GC_PTC { var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); var template (present) UL_DCCH_Message v_UEInformationResponse;//@sic R5-223993 sic@ var template (value) NR_CellPowerList_Type v_CellPowerList_T0; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var float v_WaitToLogging := 30.0; v_CellPowerList_T0 := { cs_NR_CellPower (nr_Cell1, -85, -82), cs_NR_CellPower (nr_Cell2, -91, -91) }; v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),//@sic R5-216303, R5s211676 sic@ cs_NR_CellPower (nr_Cell2, -85, -82)//@sic R5-221563 sic@ }; f_NR_SetCellPowerList(v_CellPowerList_T0); //@siclog "Step 1" siclog@ // SS transmits a LoggedMeasurementConfigurationmessage with reportType is set to periodical and LoggingInterval is set to ms10240, to configure the UE to perform logging of measurement results while in RRC_IDLE. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo_r16, cs_LoggedMeasurementConfiguration_reportType_Periodical(ms10240)))); //@siclog "Step 2" siclog@ //The SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3" siclog@ //Wait 30s to allow UE to activate logging f_Delay(v_WaitToLogging); //@siclog "Step 4-11" siclog@ //Steps 1 to 8 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure on NR Cell 1. f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1); //@siclog "Step 12" siclog@ //The SS changes NR Cell 1 and NR Cell 2 levels according to the row "T1" in table 8.1.6.1.2.7.3.2-1 for FR1 or table 8.1.6.1.2.7.3.2-2 for FR2. f_NR_SetCellPowerList(v_CellPowerList_T1); //@siclog "Step 13-17" siclog@ //The UE sends RRCReestablishmentRequest message on NR Cell 2. //The SS transmits RRCReestablishmentmessage. //Check: Does The UE transmits RRCReestablishmentCompletemessage including logMeasAvailable IE? //The SS transmits an RRCReconfigurationmessage to resume existing radio bearer. //The UE transmit an RRCConnectionReconfigurationComplete message. f_NR5GC_RRCReestablishment_SON_MDT(nr_Cell1, nr_Cell2, logMeasAvailable_andRLF,otherFailure);//@sic R5s220217 R5-225389 sic@ //@siclog "Step 18" siclog@ //@sic R5-223993 sic@ //The SS sends an UEInformationRequest message to get logMeasReport and rlf-Report. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell2, -, cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, true_, -, -, true_))); //@siclog "Step 19" siclog@ //@sic R5-223993 sic@ //The UE sends an UEInformationResponse message including logMeasReport and rlf-Report. v_UEInformationResponse := cr_UEInformationResponse_r16_Common(tsc_NR_RRC_TI_Def, -, -, ?, -, -, ?); SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell2, tsc_NR_RbId_SRB1, v_UEInformationResponse)); }//end of f_TC_8_1_6_1_2_7_NR5GC_TestBody

After Change:

function f_TC_8_1_6_1_2_7_NR5GC_TestBody() runs on NR5GC_PTC { var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); var template (present) UL_DCCH_Message v_UEInformationResponse;//@sic R5-223993 sic@ var template (value) NR_CellPowerList_Type v_CellPowerList_T0; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var float v_WaitToLogging := 30.0; v_CellPowerList_T0 := { cs_NR_CellPower (nr_Cell1, -85, -82), cs_NR_CellPower (nr_Cell2, -91, -91) }; v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE),//@sic R5-216303, R5s211676 sic@ cs_NR_CellPower (nr_Cell2, -85, -82)//@sic R5-221563 sic@ }; f_NR_SetCellPowerList(v_CellPowerList_T0); //@siclog "Step 1" siclog@ // SS transmits a LoggedMeasurementConfigurationmessage with reportType is set to periodical and LoggingInterval is set to ms10240, to configure the UE to perform logging of measurement results while in RRC_IDLE. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo_r16, cs_LoggedMeasurementConfiguration_reportType_Periodical(ms10240)))); //@siclog "Step 2" siclog@ //The SS transmits an RRCRelease message. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3" siclog@ //Wait 30s to allow UE to activate logging f_Delay(v_WaitToLogging); //@siclog "Step 4-11" siclog@ //Steps 1 to 8 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are executed to successfully complete the service request procedure on NR Cell 1. f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1); //@siclog "Step 12" siclog@ //The SS changes NR Cell 1 and NR Cell 2 levels according to the row "T1" in table 8.1.6.1.2.7.3.2-1 for FR1 or table 8.1.6.1.2.7.3.2-2 for FR2. f_NR_SetCellPowerList(v_CellPowerList_T1); //@siclog "Step 13-17" siclog@ //The UE sends RRCReestablishmentRequest message on NR Cell 2. //The SS transmits RRCReestablishmentmessage. //Check: Does The UE transmits RRCReestablishmentCompletemessage including logMeasAvailable IE? //The SS transmits an RRCReconfigurationmessage to resume existing radio bearer. //The UE transmit an RRCConnectionReconfigurationComplete message. f_NR5GC_RRCReestablishment_SON_MDT(nr_Cell1, nr_Cell2, logMeasAvailable_andRLF,otherFailure);//@sic R5s220217 R5-225389 sic@ //@siclog "Step 18" siclog@ //@sic R5-223993 sic@ //The SS sends an UEInformationRequest message to get logMeasReport and rlf-Report. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell2, -, cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, true_, -, -, true_))); //@siclog "Step 19" siclog@ //@sic R5-223993 sic@ //The UE sends an UEInformationResponse message including logMeasReport and rlf-Report. v_UEInformationResponse := cr_UEInformationResponse_r16_Common(tsc_NR_RRC_TI_Def, -, -, ?, -, -, ?); SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell2, tsc_NR_RbId_SRB2, v_UEInformationResponse)); }//end of f_TC_8_1_6_1_2_7_NR5GC_TestBody