3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221245

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4701 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_TC_8_5_4_2or3_RequestedFrequencyBandList |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11_Test |  |  |  |  |  | Date: |  |  | 2022-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For UEs reporting in step 2 some 2DL+1UL CA band combination(s) that contain(s) extendedBands (i.e. band no. > 64), a matching error may occur in subsequent steps 3 - 4 as these steps do not yet cater for the possibility of extendedBands among the reported ones. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhanced function fl_TC_8_5_4_2or3_RequestedFrequencyBandList () to handle also supportedBandCombination_v1090 and thus extendedBands. Note that the proposed TTCN correction does not yet cover cases in which the UE reports several CA band combinations with different extended bands. Use of supportedBandCombination-v1090 alone would not be sufficient to resolve the resulting ambiguity. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC implementation may FAIL a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.4.2, 8.5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2. Correction to fl_TC_8_5_4_2or3_RequestedFrequencyBandList	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## 2. Correction to fl_TC_8_5_4_2or3_RequestedFrequencyBandList

| Function name | fl_TC_8_5_4_2or3_RequestedFrequencyBandList () |
| --- | --- |
| Reason for change | For UEs reporting in step 2 some 2DL+1UL CA band combination(s) that contain(s) extendedBands (i.e. band no. > 64), a matching error may occur in subsequent steps 3 - 4 as these steps do not yet cater for the possibility of extendedBands among the reported ones. |
| Summary of change | Enhanced function fl_TC_8_5_4_2or3_RequestedFrequencyBandList () to handle also SupportedBandCombination_v1090 and thus extendedBands. |
| TTCN module | ATS_LTE_A_R10_r11\8_5\RRC_Others_CA.ttcn |
| MCC160 Comment |  |

Before Change:

/* * @desc Function to set the requestedbands which will be needed in step3/4 * @param p_ReceivedCapabilityMsg * @param p_TestVariable * @return RequestedFrequencyBands_r11_Type * @status APPROVED (LTE_A_R10_R11) */ function fl_TC_8_5_4_2or3_RequestedFrequencyBandList(UE_EUTRA_Capability p_ReceivedCapabilityMsg, TC_8_5_4_2or3_TestVariable_Type p_TestVariable) return RequestedFrequencyBands_r11_Type { var SupportedBandListEUTRA v_SupportedBandListEUTRA := p_ReceivedCapabilityMsg.rf_Parameters.supportedBandListEUTRA; var RequestedFrequencyBands_r11_Type v_RequestedFrequencyBands; var integer v_FreqBandIndicator; //@sic R5-163085 FrequencyBandIndicator type changed to integer sic@ var integer i; var integer k; v_RequestedFrequencyBands[0] := p_TestVariable.B3; v_RequestedFrequencyBands[1] := p_TestVariable.B1; i := 2; for (k := 0; (k < lengthof(v_SupportedBandListEUTRA)) and (i < 16); k := k+1) { // v_RequestedFrequencyBands shall have a maxiimum of 16 entries v_FreqBandIndicator := v_SupportedBandListEUTRA[k].bandEUTRA; if ((v_FreqBandIndicator != p_TestVariable.B1) and (v_FreqBandIndicator != p_TestVariable.B3)) { v_RequestedFrequencyBands[i] := v_FreqBandIndicator; i := i + 1; } } return v_RequestedFrequencyBands; }

After Change:

/* * @desc Function to set the requestedbands which will be needed in step3/4 * @param p_ReceivedCapabilityMsg * @param p_TestVariable * @return RequestedFrequencyBands_r11_Type * @status APPROVED (LTE_A_R10_R11) */ function fl_TC_8_5_4_2or3_RequestedFrequencyBandList(UE_EUTRA_Capability p_ReceivedCapabilityMsg, TC_8_5_4_2or3_TestVariable_Type p_TestVariable) return RequestedFrequencyBands_r11_Type { var SupportedBandListEUTRA v_SupportedBandListEUTRA := p_ReceivedCapabilityMsg.rf_Parameters.supportedBandListEUTRA; var SupportedBandCombination_v1090 v_SupportedBandCombination_v1090 := p_ReceivedCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension. nonCriticalExtension.nonCriticalExtension.rf_Parameters_v1090.supportedBandCombination_v1090; var RequestedFrequencyBands_r11_Type Type v_RequestedFrequencyBands; var integer v_FreqBandIndicator; //@sic R5-163085 FrequencyBandIndicator type changed to integer sic@ var integer i; var integer k; v_RequestedFrequencyBands[0] := p_TestVariable.B3; v_RequestedFrequencyBands[1] := p_TestVariable.B1; i := 2; for (k := 0; (k < lengthof(v_SupportedBandListEUTRA)) and (i < 16); k := k+1) { // v_RequestedFrequencyBands shall have a maxiimum of 16 entries v_FreqBandIndicator := v_SupportedBandListEUTRA[k].bandEUTRA; if ((v_FreqBandIndicator != p_TestVariable.B1) and (v_FreqBandIndicator != p_TestVariable.B3)) { if (v_FreqBandIndicator < 64 ){ v_RequestedFrequencyBands[i] := v_FreqBandIndicator; } else { v_RequestedFrequencyBands[i] := v_SupportedBandCombination_v1090[i][0].bandEUTRA_v1090; } i := i + 1; } } return v_RequestedFrequencyBands; }