3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221266

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0842 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to emergency call test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are several coordination issues in the D22wk23 TTCN implementation of the emergency call test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | All coordination issues concerning the emergency test cases are adressed below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Coordination between IMS1, IMS2 and NR5GC components is incorrect causing the affected test cases to be stuck in various steps. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Emergency voice call testcases (10.1 through 10.10) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

2.2	Change 2	4

2.3	Change 3	7

2.4	Change 4	10

2.5	Change 5	14

2.6	Change 6	15

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_IMS_CC_Postamble |
| --- | --- |
| Reason for change | The trigger to IPCAN (“IPCAN.send(cms_IMS_IPCAN_Trigger);”) is handled by f_NR5GC_EmergencyCallRelease which is called by f_IMS_CC_ReleaseConnection, therefore the f_IMS_CC_ReleaseConnection needs to be called before this trigger and not after. |
| Summary of change | Moved calling of the f_IMS_CC_ReleaseConnection before sending the trigger to IPCAN. |
| TTCN module | IMS_CC_CommonFunctions.ttcn |
| MCC160 Comment |  |

Before Change

function f_IMS_CC_Postamble(template (omit) IMS_TestProcedure_Type p_TestProcedure := omit, template (omit) IMS_TCP_CloseHandling_Type p_TcpCloseHandling := omit, float p_DelayForDL_SIP := 2.0) runs on IMS_PTC { /* @sic R5s140801: new parameter p_TcpCloseHandling, R5s170028: removed parameter p_WaitForDeRegistration sic@ */ /* @sic R5s190785: new parameter p_DelayForDL_SIP sic@ */ var boolean v_AllowRetransmission := f_IMS_PTC_RanTypeIsUTRAN(); var float v_DelayForDL_SIP := p_DelayForDL_SIP; if (isvalue(p_TestProcedure)) { if (valueof(p_TestProcedure) == IPCAN_EmergencyCall_NormalService or valueof(p_TestProcedure) == IPCAN_EmergencyCall_NoRegistration and f_IMS_PTC_RanTypeIsNR5GC()) { // @sic R5s220531 sic@ // the IMS messages have already been sent, so tell IPCAN not to wait for it IPCAN.send(cms_IMS_IPCAN_Trigger); } f_IMS_CC_ReleaseConnection(valueof(p_TestProcedure), p_TcpCloseHandling, v_DelayForDL_SIP); IPCAN.send(cms_IMS_IPCAN_ReleaseReq); if (valueof(p_TestProcedure) != IPCAN_Deregistration) { // @sic R5s211495 sic@ f_IMS_IpcanReleaseWithOptionalDeregistration(v_AllowRetransmission); /* @sic R5s140346 - MCC160 implementation: new function f_IMS_CC_IpcanReleaseWithOptionalDeregistration, R5s170541 sic@ */ } } else { // @sic R5s211495 sic@ IPCAN.send(cms_IMS_IPCAN_ReleaseReq); f_IMS_IpcanReleaseWithOptionalDeregistration(v_AllowRetransmission); /* @sic R5s140346 - MCC160 implementation: new function f_IMS_CC_IpcanReleaseWithOptionalDeregistration, R5s170541 sic@ */ } }

After Change

function f_IMS_CC_Postamble(template (omit) IMS_TestProcedure_Type p_TestProcedure := omit, template (omit) IMS_TCP_CloseHandling_Type p_TcpCloseHandling := omit, float p_DelayForDL_SIP := 2.0) runs on IMS_PTC { /* @sic R5s140801: new parameter p_TcpCloseHandling, R5s170028: removed parameter p_WaitForDeRegistration sic@ */ /* @sic R5s190785: new parameter p_DelayForDL_SIP sic@ */ var boolean v_AllowRetransmission := f_IMS_PTC_RanTypeIsUTRAN(); var float v_DelayForDL_SIP := p_DelayForDL_SIP; if (isvalue(p_TestProcedure)) { f_IMS_CC_ReleaseConnection(valueof(p_TestProcedure), p_TcpCloseHandling, v_DelayForDL_SIP); if (valueof(p_TestProcedure) == IPCAN_EmergencyCall_NormalService or valueof(p_TestProcedure) == IPCAN_EmergencyCall_NoRegistration and f_IMS_PTC_RanTypeIsNR5GC()) { // @sic R5s220531 sic@ // the IMS messages have already been sent, so tell IPCAN not to wait for it IPCAN.send(cms_IMS_IPCAN_Trigger); } IPCAN.send(cms_IMS_IPCAN_ReleaseReq); if (valueof(p_TestProcedure) != IPCAN_Deregistration) { // @sic R5s211495 sic@ f_IMS_IpcanReleaseWithOptionalDeregistration(v_AllowRetransmission); /* @sic R5s140346 - MCC160 implementation: new function f_IMS_CC_IpcanReleaseWithOptionalDeregistration, R5s170541 sic@ */ } } else { // @sic R5s211495 sic@ IPCAN.send(cms_IMS_IPCAN_ReleaseReq); f_IMS_IpcanReleaseWithOptionalDeregistration(v_AllowRetransmission); /* @sic R5s140346 - MCC160 implementation: new function f_IMS_CC_IpcanReleaseWithOptionalDeregistration, R5s170541 sic@ */ }

## Change 2

| Function name | f_NR5GC_IPCAN_ProcedureEnd |
| --- | --- |
| Reason for change | In case of emergency calls the f_NR5GC_EmergencyCallRelease has to be called with the “MOtriggerRequired” because its reception is awaited in f_IMS_CC_Postamble function->f_IMS_IPCAN_EndProcedure. |
| Summary of change | f_NR5GC_EmergencyCallRelease is now called with “MOtriggerRequired” enabled. |
| TTCN module | IPCAN_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function f_NR5GC_IPCAN_ProcedureEnd(IMS_IPCAN_CO_ORD_PORT p_Port, //parameter left for future use NR_CellId_Type p_NR_CellId, IMS_TestProcedure_Type p_ProcedureType, TriggerResult_Type p_CallEstablishmentResult) runs on NR5GC_PTC { select (p_ProcedureType) { case (IPCAN_InitialRegistration) { // just release RRC connection } case (IPCAN_MO_IMS_Signalling, IPCAN_MT_IMS_Signalling) { // just release RRC connection } case (IPCAN_EmergencyCall_NormalService, IPCAN_EmergencyCall_NoRegistration) { f_NR5GC_EmergencyCallRelease(p_NR_CellId, false); // @sic R5s220531 sic@ If MO, UT will already have been sent } case (IPCAN_EmergencyCall_LimitedService) { } case (IPCAN_MO_SpeechCall, IPCAN_MT_SpeechCall) { if (p_CallEstablishmentResult == NORMAL) { // NOTE: in test cases for unsuccessful behaviour no speech flow is established */ f_NR5GC_ModifyPDUSession_RemoveIMSVoice(p_NR_CellId); // @sic R5s210096: f_NR5GC_ModifyPDUSession_RemoveIMSVoice instead of f_NR5GC_CallRelease sic@ } } case (IPCAN_MO_VideoCall, IPCAN_MT_VideoCall) { if (p_CallEstablishmentResult == NORMAL) { // @sic R5s210877 sic@ f_NR5GC_ModifyPDUSession_RemoveIMSVideo(p_NR_CellId); } } case (IPCAN_XCAP_Signalling) { // just release RRC connection } } }

After Change

function f_NR5GC_IPCAN_ProcedureEnd(IMS_IPCAN_CO_ORD_PORT p_Port, //parameter left for future use NR_CellId_Type p_NR_CellId, IMS_TestProcedure_Type p_ProcedureType, TriggerResult_Type p_CallEstablishmentResult) runs on NR5GC_PTC { select (p_ProcedureType) { case (IPCAN_InitialRegistration) { // just release RRC connection } case (IPCAN_MO_IMS_Signalling, IPCAN_MT_IMS_Signalling) { // just release RRC connection } case (IPCAN_EmergencyCall_NormalService, IPCAN_EmergencyCall_NoRegistration) { f_NR5GC_EmergencyCallRelease(p_NR_CellId, true); // @sic R5s220531 sic@ If MO, UT will already have been sent } case (IPCAN_EmergencyCall_LimitedService) { } case (IPCAN_MO_SpeechCall, IPCAN_MT_SpeechCall) { if (p_CallEstablishmentResult == NORMAL) { // NOTE: in test cases for unsuccessful behaviour no speech flow is established */ f_NR5GC_ModifyPDUSession_RemoveIMSVoice(p_NR_CellId); // @sic R5s210096: f_NR5GC_ModifyPDUSession_RemoveIMSVoice instead of f_NR5GC_CallRelease sic@ } } case (IPCAN_MO_VideoCall, IPCAN_MT_VideoCall) { if (p_CallEstablishmentResult == NORMAL) { // @sic R5s210877 sic@ f_NR5GC_ModifyPDUSession_RemoveIMSVideo(p_NR_CellId); } } case (IPCAN_XCAP_Signalling) { // just release RRC connection } } }

## Change 3

| Function name | f_TC_10_6_NR5GC_IMS1 |
| --- | --- |
| Reason for change | The f_IMS_CC_Postamble must be called from IMS2 instead of IMS1 because f_NR5GC_EmergencyCallRelease is interacting explicitly with IMS2 (and not with the component where the coordination command comes from). |
| Summary of change | Moved f_IMS_CC_Postamble from IMS1 to IMS2 and addressed the coordination between IMS1 and IMS2 of the 10.6 accordingly. |
| TTCN module | IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_10_6_NR5GC_IMS1() runs on IMS_PTC { // Non-UE detectable emergency call / IM CN sends 380 with an Alternative Service / Previous emergency IMS registration not expired / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var template (omit) MessageBody v_MessageBody := omit; var template (present) MessageHeader v_MessageHeader_Ack; var template (value) MessageHeader v_MessageHeader_AltService_380; var INVITE_Request v_InviteRequest; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var charstring v_CalleeUri := px_IMS_CalleeUri; var template (present) SipUrl v_CalleeSipUrl := f_SIP_BuildSipUri_RX(px_IMS_CalleeUri); var boolean v_GeolocationUsage := pc_IMS_Geolocation; f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED); // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } f_IMS_TestBody_Set(true); f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); //@siclog "Step 19" siclog@ // UE is made to attempt an IMS voice call f_UT_RequestIMSCall(MMI, v_CalleeUri); //@siclog "Step 20" siclog@ //UE sends INVITE with the first SDP offer Check: Does the UE send correctly composed INVITE request for normal voice call? v_InviteRequestWithSdp := f_IMS_MOCallSetup_ReceiveINVITE_Common(v_CalleeSipUrl); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20"); v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_InviteRequestWithSdp.RoutingInfo); v_InviteRequest := v_InviteRequestWithSdp.Invite; //@siclog "Step 21" siclog@ // SS responds with 380 Alternative Service v_MessageHeader_AltService_380 := f_IMS_InviteResponse_380_MessageHeaderTX(v_InviteRequest); v_MessageBody := cs_MessageBody_XML_Ims_3gpp(cs_Ims_3gpp_Emergency("emergency-registration")); // XSD schema needed according to 24.229 section 7.6.2. IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine380, v_MessageHeader_AltService_380, v_MessageBody))); //@siclog "Step 22" siclog@ // UE acknowledges the receipt of 380 Alternative Service response v_MessageHeader_Ack := f_IMS_AckRequest_MessageHeaderRX(v_InviteRequest, c_statusLine380); f_IMS_ReceiveRequest(car_IMS_Ack_Request(cr_ACK_Request(v_CalleeSipUrl, v_MessageHeader_Ack))); f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService); }

After Change

function f_TC_10_6_NR5GC_IMS1() runs on IMS_PTC { // Non-UE detectable emergency call / IM CN sends 380 with an Alternative Service / Previous emergency IMS registration not expired / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var template (omit) MessageBody v_MessageBody := omit; var template (present) MessageHeader v_MessageHeader_Ack; var template (value) MessageHeader v_MessageHeader_AltService_380; var INVITE_Request v_InviteRequest; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var charstring v_CalleeUri := px_IMS_CalleeUri; var template (present) SipUrl v_CalleeSipUrl := f_SIP_BuildSipUri_RX(px_IMS_CalleeUri); var boolean v_GeolocationUsage := pc_IMS_Geolocation; f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED); // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } f_IMS_TestBody_Set(true); f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); //@siclog "Step 19" siclog@ // UE is made to attempt an IMS voice call f_UT_RequestIMSCall(MMI, v_CalleeUri); //@siclog "Step 20" siclog@ //UE sends INVITE with the first SDP offer Check: Does the UE send correctly composed INVITE request for normal voice call? v_InviteRequestWithSdp := f_IMS_MOCallSetup_ReceiveINVITE_Common(v_CalleeSipUrl); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20"); v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_InviteRequestWithSdp.RoutingInfo); v_InviteRequest := v_InviteRequestWithSdp.Invite; //@siclog "Step 21" siclog@ // SS responds with 380 Alternative Service v_MessageHeader_AltService_380 := f_IMS_InviteResponse_380_MessageHeaderTX(v_InviteRequest); v_MessageBody := cs_MessageBody_XML_Ims_3gpp(cs_Ims_3gpp_Emergency("emergency-registration")); // XSD schema needed according to 24.229 section 7.6.2. IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine380, v_MessageHeader_AltService_380, v_MessageBody))); //@siclog "Step 22" siclog@ // UE acknowledges the receipt of 380 Alternative Service response v_MessageHeader_Ack := f_IMS_AckRequest_MessageHeaderRX(v_InviteRequest, c_statusLine380); f_IMS_ReceiveRequest(car_IMS_Ack_Request(cr_ACK_Request(v_CalleeSipUrl, v_MessageHeader_Ack))); f_IMS_IMS_SendCoOrdMsg(OtherPDN); //REMOVED f_IMS_IMS_WaitForTrigger(OtherPDN); //REMOVED f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService); }

## Change 4

| Function name | f_TC_10_6_NR5GC_IMS2 |
| --- | --- |
| Reason for change | - The f_IMS_CC_Postamble must be called from IMS2 instead of IMS1 because f_NR5GC_EmergencyCallRelease is interacting explicitly with IMS2 (and not with the component where the coordination command comes from). Associated with Change 3 - In case of multiple emergency calls there may not be a need to coordinate with NR5GC the PDU session modification procedure every time. In 10.6 the 2nd call of f_IMS_EmergencyCallSetup_NR5GC_A_6 is exactly the case where the coordination with NR5GC should be disabled as it was already handled during the 1st call. |
| Summary of change | - Moved f_IMS_CC_Postamble from IMS1 to IMS2 and addressed the coordination between IMS1 and IMS2 of the 10.6 accordingly. - Added new parameters to f_IMS_EmergencyCallSetup_NR5GC_A_6 to optionally disable the coordination with NR5GC if needed in given scenario. |
| TTCN module | IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_10_6_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var SipUrl v_ContactUrlUE; f_IMS_PTC_Init(PDN_2); f_IMS_IMS_WaitForTrigger(OtherPDN); f_IMS_TestBody_Set(true); // @siclog "Steps 2-11" siclog@ // Steps 1-10 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 are performed // EXCEPTION: In parallel to the events described in steps 12-16 below the events specified in 11-13 of Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed. f_IMS_CC_StartCall(IPCAN_EmergencyCall_NormalService); // @siclog "Steps 12-16" siclog@ // Steps 1-5 of Annex A.6 happens f_IMS_EmergencyRegistration(); f_IMS_EmergencyCallSetup_NR5GC_A_6(); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 16"); //@siclog "Steps 17-18" siclog@ //Steps 1 -2 of Annex A.8 happens v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); // @sic R5s220555 sic@ f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 18"); f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); //EXCEPTION: In parallel to the events described in steps 23-27 below the events specified in 11-13 of Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed. //@siclog "Steps 23-27" siclog@ //Step 1 of Annex A.6 happens //Check: Does UE send INVITE message for emergency call? //Steps 2-5 of Annex A.6 happens f_IMS_EmergencyCallSetup_NR5GC_A_6(); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 23"); //@siclog "Steps 28-29" siclog@ //Step 1 of Annex A.8 happens //Step 2 of Annex A.8 happens //Check: Does the UE send 200 OK for the BYE request and ends the call? f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 29"); f_IMS_TestBody_Set(false); f_IMS_IMS_SendCoOrdMsg(OtherPDN); }

After Change

function f_TC_10_6_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var SipUrl v_ContactUrlUE; f_IMS_PTC_Init(PDN_2); f_IMS_IMS_WaitForTrigger(OtherPDN); f_IMS_TestBody_Set(true); // @siclog "Steps 2-11" siclog@ // Steps 1-10 of generic procedure specified in Table 4.9.11.2.2-1 of TS 38.508-1 are performed // EXCEPTION: In parallel to the events described in steps 12-16 below the events specified in 11-13 of Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed. f_IMS_CC_StartCall(IPCAN_EmergencyCall_NormalService); // @siclog "Steps 12-16" siclog@ // Steps 1-5 of Annex A.6 happens f_IMS_EmergencyRegistration(); f_IMS_EmergencyCallSetup_NR5GC_A_6(); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 16"); //@siclog "Steps 17-18" siclog@ //Steps 1 -2 of Annex A.8 happens v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); // @sic R5s220555 sic@ f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 18"); f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); //EXCEPTION: In parallel to the events described in steps 23-27 below the events specified in 11-13 of Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed. //@siclog "Steps 23-27" siclog@ //Step 1 of Annex A.6 happens //Check: Does UE send INVITE message for emergency call? //Steps 2-5 of Annex A.6 happens f_IMS_EmergencyCallSetup_NR5GC_A_6(-,-,-,false, false); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 23"); //@siclog "Steps 28-29" siclog@ //Step 1 of Annex A.8 happens //Step 2 of Annex A.8 happens //Check: Does the UE send 200 OK for the BYE request and ends the call? f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 29"); f_IMS_TestBody_Set(false); //REMOVED f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_CC_Postamble(IPCAN_EmergencyCall_NormalService); }

## Change 5

| Function name | f_IMS_EmergencyCallSetup_NR5GC_A_6 |
| --- | --- |
| Reason for change | In case of multiple emergency calls there may not be a need to coordinate with NR5GC the PDU session modification procedure every time. Associated with change 4 |
| Summary of change | Added new parameters to f_IMS_EmergencyCallSetup_NR5GC_A_6 to optionally disable the coordination with NR5GC if needed in given scenario. |
| TTCN module | IMS_Procedures_5G.ttcn |
| MCC160 Comment |  |

Before Change

function f_IMS_EmergencyCallSetup_NR5GC_A_6(INVITE_A_2_1_Context_Type p_Context := A_2_1_A7, boolean p_GeolocationInfoAvailable := false, // @sic R5-220214 sic@ template SipUrl p_OtherCalleeUri := omit) runs on IMS_PTC return INVITE_Request { var IMS_DATA_REQ v_IMS_DATA_REQ; v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(p_Context, p_GeolocationInfoAvailable, p_OtherCalleeUri); return f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ, p_GeolocationInfoAvailable); // @sic R5s210875 sic@ }

After Change

function f_IMS_EmergencyCallSetup_NR5GC_A_6(INVITE_A_2_1_Context_Type p_Context := A_2_1_A7, boolean p_GeolocationInfoAvailable := false, // @sic R5-220214 sic@ template SipUrl p_OtherCalleeUri := omit, boolean p_WaitForTrigger := true, boolean p_SessionModification := true) runs on IMS_PTC return INVITE_Request { var IMS_DATA_REQ v_IMS_DATA_REQ; v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(p_Context, p_GeolocationInfoAvailable, p_OtherCalleeUri); return f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ, p_GeolocationInfoAvailable, p_WaitForTrigger, p_SessionModification); // @sic R5s210875 sic@ }

## Change 6

| Function name | f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd |
| --- | --- |
| Reason for change | In case of multiple emergency calls there may not be a need to coordinate with NR5GC the PDU session modification procedure every time. Associated with change 4 |
| Summary of change | Added new parameters to f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd to optionally disable the coordination with NR5GC if needed in given scenario. |
| TTCN module | IMS_Procedures_5G.ttcn |
| MCC160 Comment |  |

Before Change

function f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(IMS_DATA_REQ p_IMS_DATA_REQ, boolean p_GeolocationInfoAvailable := false) runs on IMS_PTC return INVITE_Request // @sic R5-220214 sic@ { var INVITE_Request v_INVITE_Request; f_IMS_IPCAN_SendCoOrdMsg(IPCAN); v_INVITE_Request := f_IMS_EmergencyCallSetup_Step2ToEnd (p_IMS_DATA_REQ, p_GeolocationInfoAvailable); f_IMS_IPCAN_WaitForTrigger(IPCAN); // @sic R5s220531 sic@ return v_INVITE_Request; }

After Change

function f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(IMS_DATA_REQ p_IMS_DATA_REQ, boolean p_GeolocationInfoAvailable := false, boolean p_WaitForTrigger := true, boolean p_SessionModification := true) runs on IMS_PTC return INVITE_Request // @sic R5-220214 sic@ { var INVITE_Request v_INVITE_Request; if (p_SessionModification) { f_IMS_IPCAN_SendCoOrdMsg(IPCAN); } v_INVITE_Request := f_IMS_EmergencyCallSetup_Step2ToEnd (p_IMS_DATA_REQ, p_GeolocationInfoAvailable); if (p_WaitForTrigger) { f_IMS_IPCAN_WaitForTrigger(IPCAN); } return v_INVITE_Request; }