3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221268

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2620 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for EN-DC CA test cases 8.2.4.2.1.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - At step 5-6 when Scell 3 is removed and replaced with Scell 12, sCellToReleaseList with ScellIndex 2 should be included in RRCReconfiguration. - At step 7-8, when Scell 12 is made PCell with no SCell, sCellToReleaseList with ScellIndex 2 should be included in RRCReconfiguration. - The sCellIndex of NR cell 12 is ‘2 ‘ in RRCReconfiguration at step 5-6 but this is not configured to SS. - During inter-cell handover security key is changed due to reconfiguration with Sync, re-establishPDCP set to true and reestablishRLC has to be set to true - If “p_SCGConfig.sCellToAddModList” is not present, the function _NR_ConfigSCell() will give a run-time error when attempting to access “SCGConfig.sCellToAddModList[0].sCellConfigDedicated.tag_Id”. This happens at step 7 where this function is called through f_NR_RRC_PSCellChangeCA_InterCell() but sCellToAddModList is not present. - If IP ping is not supported, UE should have test loop mode B activated in the preamble and open for the data path check at step 3A and 6A, and closed again before postamble sequence. - Missing coordaintion message coordination message to trigger change power in NR cell at step 1 - The function f_NR_SS_SRBs_DRBs_Release() is called through f_NR_ReconfigurePCellToNormalCell() and is not previously used for EN-DC test cases, and now needs to take into account that SRBs will not have been configured on the SS NR cell in these scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - sCellToReleaseList set with ScellIndex 2 in secondary cell group config. - sCellToReleaseList set with ScellIndex 2 in secondary cell group config. - Update SS with new SCell configuration - Set reestablishRLC and call f_NR_RetrieveRadioBearerConfig_PSCellChange invoked with key change paramter set true. - Added a check for SCGConfig.sCellToAddModList being present before accessing - Update preamble condition and test loop mode B is opened before step 3A and closed after step 8 - Added the coordination message for ‘change Power’ - SRB1 and SRB2 not added to the release list in the case of EN-DC. Note : An associated prose CR on38.523-1 will be submitted at RAN5#96 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.2.1.1.ENDC, 8.2.4.2.1.3.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	9

2.3	Change 3	10

2.4	Change 4	12

2.5	Change 5	15

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody() |
| --- | --- |
| Reason for change | - At step 5-6 when Scell 3 is removed and replaced with Scell 12, sCellToReleaseList with ScellIndex 2 should be included in RRCReconfiguration. - At step 7-8, when Scell 12 is made PCell with no SCell, sCellToReleaseList with ScellIndex 2 should be included in RRCReconfiguration. - The sCellIndex of NR cell 12 is ‘2 ‘ in RRCReconfiguration at step 5-6 but this is not configured to SS. - During inter-cell handover security key is changed due to reconfiguration with Sync, re-establishPDCP set to true and reestablishRLC has to be set to true |
| Summary of change | - sCellToReleaseList set with ScellIndex 2 in secondary cell group config. - sCellToReleaseList set with ScellIndex 2 in secondary cell group config. - Update SS with new SCell configuration - f_NR_RetrieveRadioBearerConfig_PSCellChange invoked with first argument as true Note : An associated prose CR on38.523-1 will be submitted at RAN5#96 |
| TTCN module | RRC_CarrierAggregation_ENDC_NR |
| MCC160 Comment |  |

Before Change

function fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody(NR_CellId_Type p_PSCell1, NR_CellId_Type p_PSCell2, NR_CellId_Type p_SCell1, NR_CellId_Type p_SCell2) runs on ENDC_NR_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) CellGroupConfig v_SCGConfig := f_NR_GetSCGConfigDef(p_PSCell2); var template(value) CellGroupConfig v_SCGConfig_SCell := f_NR_GetSCGConfig_SCellAdd(p_SCell2, 2); var template (value) RadioBearerConfig v_RadioBearerConfig := cs_NR_RadioBearer(tsc_NR_DRB2, secondary, f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()); var template (value) NR_RadioBearerList_Type v_RadioBearerToReleaseList := {cs_NR_SS_DRB_Release(tsc_NR_DRB2)}; v_CellPowerList_AtT0 := {cs_NR_CellPower(p_PSCell1, -88, -91), cs_NR_CellPower(p_SCell1, -88, -91)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(p_PSCell2, -88, -91), cs_NR_CellPower(p_SCell2, -88, -91)}; // @siclog "Step 1" siclog@ //The SS changes Cell parameters according to the row "T0" in table 8.2.4.2.1.1.3.2-1. // Send trigger to NR to change power level of NR Cell1 f_NR_WaitForEUTRATrigger_And_ChangePower(v_CellPowerList_AtT0); //@siclog "Steps 2 - 3" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to configure NR Cell 1 as the NR PSCell and sCellToAddModList with an sCellIndex set to NR Cell 3 as the configured SCell. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? f_NR_AddMod_PSCell_SCell_ENDC( p_PSCell1, p_SCell1 ); //@siclog "Step 3A" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1A indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 3? //@sic R5-203666 sic@ f_NR_CheckDataPath_CA(p_PSCell1, p_SCell1, "3A", true,tsc_SCellIndex_2); //@siclog "Step 4" siclog@ //The SS changes NR Cell 2 and NR Cell 12 parameters according to the row "T1" in table 8.2.4.2.1.1.3.2-1. f_NR_WaitForEUTRATrigger_And_ChangePower(v_CellPowerList_AtT1); //@siclog "Steps 5 - 6" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to perform both NR Cell 1 and NR Cell 3 changed to the configured target NR Cell 2 and NR Cell 12. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message configuring the new PSCell and SCell and configure lower layers to consider the SCell to be in deactivated state? v_SCGConfig.sCellToAddModList := v_SCGConfig_SCell.sCellToAddModList; v_RadioBearerConfig := f_NR_RetrieveRadioBearerConfig_PSCellChange( false, -, secondary); f_NR_RRC_PSCellChangeCA_InterCell(p_PSCell1, p_SCell1, p_PSCell2, p_SCell2, false, v_RadioBearerConfig, v_SCGConfig, v_RadioBearerToReleaseList); //@siclog "Step 6A" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1A indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 12? //@sic R5-203666 sic@ f_NR_CheckDataPath_CA(p_PSCell2, p_SCell2, "6A", true, tsc_SCellIndex_2); //@siclog "Steps 7 - 8" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to change the current PSCell (NR Cell 2) to NR Cell 12 equaling to one of the current UE SCell configuration. //Check: Does the UE send an RRCConnectionReconfigurationComplete message and reconfigure the current NR SCell to PSCell? v_SCGConfig := f_NR_GetSCGConfigDef(p_SCell2); v_SCGConfig_SCell := f_NR_GetSCGConfigDef(p_PSCell2); v_SCGConfig.sCellToAddModList := v_SCGConfig_SCell.sCellToAddModList; f_NR_ReconfigSCellToNormalCell(p_SCell2, {f_NR_GetRadioBearerConfig_FullAM_DRB2(p_PSCell2)}); f_NR_RRC_PSCellChangeCA_InterCell(p_PSCell2, p_SCell2, p_SCell2, p_PSCell2, false, v_RadioBearerConfig, v_SCGConfig, v_RadioBearerToReleaseList); f_NR_ReconfigurePCellToNormalCell( p_PSCell2, tsc_PCellIndex_1, -, {tsc_NR_DRB2} ); }

After Change

function fl_TC_8_2_4_2_1_x_ENDC_NR_TestBody(NR_CellId_Type p_PSCell1, NR_CellId_Type p_PSCell2, NR_CellId_Type p_SCell1, NR_CellId_Type p_SCell2) runs on ENDC_NR_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) CellGroupConfig v_SCGConfig := f_NR_GetSCGConfigDef(p_PSCell2); var template(value) CellGroupConfig v_SCGConfig_SCell := f_NR_GetSCGConfig_SCellAdd(p_SCell2, 2); var template (value) RadioBearerConfig v_RadioBearerConfig := cs_NR_RadioBearer(tsc_NR_DRB2, secondary, f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()); var template (value) NR_RadioBearerList_Type v_RadioBearerToReleaseList := {cs_NR_SS_DRB_Release(tsc_NR_DRB2)}; var template (omit) TAG_Id v_TAG_Id ; v_CellPowerList_AtT0 := {cs_NR_CellPower(p_PSCell1, -88, -91), cs_NR_CellPower(p_SCell1, -88, -91)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(p_PSCell2, -88, -91), cs_NR_CellPower(p_SCell2, -88, -91)}; // @siclog "Step 1" siclog@ //The SS changes Cell parameters according to the row "T0" in table 8.2.4.2.1.1.3.2-1. // Send trigger to NR to change power level of NR Cell1 f_NR_WaitForEUTRATrigger_And_ChangePower(v_CellPowerList_AtT0); //@siclog "Steps 2 - 3" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to configure NR Cell 1 as the NR PSCell and sCellToAddModList with an sCellIndex set to NR Cell 3 as the configured SCell. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? f_NR_AddMod_PSCell_SCell_ENDC( p_PSCell1, p_SCell1 ); //Get security info from EUTRA,calculate latest keys and configure SS f_ENDC_GetKeysActivateSecurity(p_SCell1,EUTRA); //@siclog "Step 3A" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1A indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 3? //@sic R5-203666 sic@ f_NR_CheckDataPath_CA(p_PSCell1, p_SCell1, "3A", true,tsc_SCellIndex_2); //@siclog "Step 4" siclog@ //The SS changes NR Cell 2 and NR Cell 12 parameters according to the row "T1" in table 8.2.4.2.1.1.3.2-1. f_NR_WaitForEUTRATrigger_And_ChangePower(v_CellPowerList_AtT1); //@siclog "Steps 5 - 6" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to perform both NR Cell 1 and NR Cell 3 changed to the configured target NR Cell 2 and NR Cell 12. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message configuring the new PSCell and SCell and configure lower layers to consider the SCell to be in deactivated state? v_SCGConfig.sCellToAddModList := v_SCGConfig_SCell.sCellToAddModList; f_NR_ConfigSCell(p_SCell2,v_SCGConfig,cs_TimingInfo_Now); v_SCGConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; v_SCGConfig.sCellToReleaseList[0] := tsc_SCellIndex_2; v_RadioBearerConfig := f_NR_RetrieveRadioBearerConfig_PSCellChange( true, -, secondary); f_NR_RRC_PSCellChangeCA_InterCell(p_PSCell1, p_SCell1, p_PSCell2, p_SCell2, false, v_RadioBearerConfig, v_SCGConfig, v_RadioBearerToReleaseList); //@siclog "Step 6A" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1A indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 12? //@sic R5-203666 sic@ f_NR_CheckDataPath_CA(p_PSCell2, p_SCell2, "6A", true, tsc_SCellIndex_2); //@siclog "Steps 7 - 8" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to change the current PSCell (NR Cell 2) to NR Cell 12 equaling to one of the current UE SCell configuration. //Check: Does the UE send an RRCConnectionReconfigurationComplete message and reconfigure the current NR SCell to PSCell? v_SCGConfig := f_NR_GetSCGConfigDef(p_SCell2); v_SCGConfig_SCell := f_NR_GetSCGConfigDef(p_PSCell2); v_SCGConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; v_SCGConfig.sCellToAddModList := v_SCGConfig_SCell.sCellToAddModList; f_NR_ReconfigSCellToNormalCell(p_SCell2, {f_NR_GetRadioBearerConfig_FullAM_DRB2(p_PSCell2)}); v_SCGConfig.sCellToReleaseList[0] := tsc_SCellIndex_2; f_NR_RRC_PSCellChangeCA_InterCell(p_PSCell2, p_SCell2, p_SCell2, p_PSCell2, false, v_RadioBearerConfig, v_SCGConfig, v_RadioBearerToReleaseList); f_NR_ReconfigurePCellToNormalCell( p_PSCell2, tsc_PCellIndex_1, -, {tsc_NR_DRB2} ); }

## Change 2

| Function name | f_NR_ConfigSCell() |
| --- | --- |
| Reason for change | If “p_SCGConfig.sCellToAddModList” is not present, this function will give a run-time error when attempting to access “SCGConfig.sCellToAddModList[0].sCellConfigDedicated.tag_Id”. This happens at step 7 where this function is called through f_NR_RRC_PSCellChangeCA_InterCell() but sCellToAddModList is not present. |
| Summary of change | Added a check for SCGConfig.sCellToAddModList being present or not before accessing |
| TTCN module | NR_CommonFunctions_CA |
| MCC160 Comment |  |

Before Change

function f_NR_ConfigSCell(NR_CellId_Type p_SCellId, template(value) CellGroupConfig p_SCGConfig, template(value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now ) runs on NR_BASE_PTC { var template (omit) TAG_Id v_TAG_Id := p_SCGConfig.sCellToAddModList[0].sCellConfigDedicated.tag_Id; var template (value) NR_ServingCellIndex_Type v_SCellIndex := p_SCGConfig.sCellToAddModList[0].sCellIndex; //other PCell reconfiguration depending on the TC // CQI report //@sic R5s200462 change 2 sic@ // Configure SCell as Scell f_NR_SS_CommonCellConfig(p_SCellId, cads_NR_ServingCellConfig_REQ(p_SCellId, p_TimingInfo, //@sic R5s200462 change 2 sic@ cs_NR_ServingCellConfig_SCell(v_SCellIndex, v_TAG_Id))); }

After Change

function f_NR_ConfigSCell(NR_CellId_Type p_SCellId, template(value) CellGroupConfig p_SCGConfig, template(value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now ) runs on NR_BASE_PTC { if(ispresent(p_SCGConfig.sCellToAddModList)) { var template (omit) TAG_Id v_TAG_Id := p_SCGConfig.sCellToAddModList[0].sCellConfigDedicated.tag_Id; var template (value) NR_ServingCellIndex_Type v_SCellIndex := p_SCGConfig.sCellToAddModList[0].sCellIndex; //other PCell reconfiguration depending on the TC // CQI report //@sic R5s200462 change 2 sic@ // Configure SCell as Scell f_NR_SS_CommonCellConfig(p_SCellId, cads_NR_ServingCellConfig_REQ(p_SCellId, p_TimingInfo, //@sic R5s200462 change 2 sic@ cs_NR_ServingCellConfig_SCell(v_SCellIndex, v_TAG_Id))); } }

## Change 3

| Function name | f_TC_8_2_4_2_1_x_EUTRA_Common() |
| --- | --- |
| Reason for change | If IP ping is not supported, UE should have test loop mode B activated for the data path check at step 3A and 6A |
| Summary of change | p_TestModeState is set to PING_Or_TESTModeB_ON in EUTRA preamble |
| TTCN module | RRC_CarrierAggregation_ENDC_EUTRA |
| MCC160 Comment |  |

Before Change

function f_TC_8_2_4_2_1_x_EUTRA_Common() runs on EUTRA_5GS_PTC { // SCG Change / Reconfiguration with sync / Split DRB / EN-DC f_EUTRA38_Init_CA(EN_DC, c1); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //Preamble to enter UE into RRC Connected //The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (EN-DC) according to TS 38.508-1 [4]. f_EUTRA38_ENDC_Preamble (eutra_Cell1, TESTMode_OFF, MCG_Only, RRC_CONNECTED); f_EUTRA_TestBody_Set(true); fl_TC_8_2_4_2_1_x_ENDC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA38_ENDC_Postamble(eutra_Cell1, E2_CONNECTED); //@sic R5-204475 sic@ // If finishing in E2_CONNECTED, need to wait until connection released before taking down the NR cell f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

After Change

function f_TC_8_2_4_2_1_x_EUTRA_Common() runs on EUTRA_5GS_PTC { // SCG Change / Reconfiguration with sync / Split DRB / EN-DC f_EUTRA38_Init_CA(EN_DC, c1); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //Preamble to enter UE into RRC Connected //The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (EN-DC) according to TS 38.508-1 [4]. f_EUTRA38_ENDC_Preamble (eutra_Cell1, PING_Or_TESTModeB_ON, MCG_Only, RRC_CONNECTED); f_EUTRA_TestBody_Set(true); fl_TC_8_2_4_2_1_x_ENDC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA38_ENDC_Postamble(eutra_Cell1, E2_CONNECTED); //@sic R5-204475 sic@ // If finishing in E2_CONNECTED, need to wait until connection released before taking down the NR cell f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

## Change 4

| Function name | fl_TC_8_2_4_2_1_x_ENDC_EUTRA_TestBody() |
| --- | --- |
| Reason for change | - If IP ping is not supported, UE should have test loop mode B open for the data path check at step 3A and 6A, and vlosed again before postamble sequence. - Missing coordaintion message coordination message to trigger change power in NR cell at step 1 |
| Summary of change | - Test loop mode B is open before step 3A and closed after step 8 - Added the coordination message for ‘change Power’ |
| TTCN module | RRC_CarrierAggregation_ENDC_EUTRA |
| MCC160 Comment |  |

function fl_TC_8_2_4_2_1_x_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { var EUTRA38_SecurityParamsExtd_Type v_Security_Params; // @siclog "Step 1" siclog@ //The SS changes Cell parameters according to the row "T0" in table 8.2.4.2.1.1.3.2-1. // Send trigger to NR to change power level of NR Cell1 f_EUTRA38_SetCellPower(eutra_Cell1, -85, -96); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@siclog "Steps 2 - 3" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to configure NR Cell 1 as the NR PSCell and sCellToAddModList with an sCellIndex set to NR Cell 3 as the configured SCell. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? // Initiate SgNB Keys @sic R5s200647 sic@ v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); f_EUTRA38_Security_Set(v_Security_Params); f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, MCG_SCG, v_Security_Params.SK_Counter); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 3"); //@siclog "Step 4" siclog@ //The SS changes NR Cell 2 and NR Cell 12 parameters according to the row "T1" in table 8.2.4.2.1.1.3.2-1. f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //@siclog "Steps 5 - 6" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to perform both NR Cell 1 and NR Cell 3 changed to the configured target NR Cell 2 and NR Cell 12. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message configuring the new PSCell and SCell and configure lower layers to consider the SCell to be in deactivated state? v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR, 1); f_EUTRA38_RRC_PSCellChange_InterCell (eutra_Cell1, v_Security_Params.SK_Counter); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6"); //@siclog "Steps 7 - 8" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to change the current PSCell (NR Cell 2) to NR Cell 12 equaling to one of the current UE SCell configuration. v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR, 1); f_EUTRA38_RRC_PSCellChange_InterCell (eutra_Cell1, v_Security_Params.SK_Counter); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 8"); }

After Change

function fl_TC_8_2_4_2_1_x_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC { var EUTRA38_SecurityParamsExtd_Type v_Security_Params; // @siclog "Step 1" siclog@ //The SS changes Cell parameters according to the row "T0" in table 8.2.4.2.1.1.3.2-1. // Send trigger to NR to change power level of NR Cell1 f_EUTRA38_SetCellPower(eutra_Cell1, -85, -96); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("ChangePower")); //@siclog "Steps 2 - 3" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to configure NR Cell 1 as the NR PSCell and sCellToAddModList with an sCellIndex set to NR Cell 3 as the configured SCell. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message? // Initiate SgNB Keys @sic R5s200647 sic@ v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); f_EUTRA38_Security_Set(v_Security_Params); f_EUTRA38_508_ENDC_Reconfiguration (eutra_Cell1, MCG_SCG, v_Security_Params.SK_Counter); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 3"); if(not pc_IP_Ping) { f_EUTRA_CloseUE_TestLoopModeB(eutra_Cell1,'00'O); } //Initiate SgNB Keys @sic R5s200647 sic@ v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); f_EUTRA38_Security_Set(v_Security_Params); //@siclog "Step 4" siclog@ //The SS changes NR Cell 2 and NR Cell 12 parameters according to the row "T1" in table 8.2.4.2.1.1.3.2-1. f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("ChangePower")); //@siclog "Steps 5 - 6" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to perform both NR Cell 1 and NR Cell 3 changed to the configured target NR Cell 2 and NR Cell 12. //Check: Does the UE transmit an RRCConnectionReconfigurationComplete message configuring the new PSCell and SCell and configure lower layers to consider the SCell to be in deactivated state? v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR, 1); f_EUTRA38_RRC_PSCellChange_InterCell (eutra_Cell1, v_Security_Params.SK_Counter); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6"); //@siclog "Steps 7 - 8" siclog@ //The SS transmits an RRCConnectionReconfiguration message with nr-Config IE including NR RRCReconfiguration message to change the current PSCell (NR Cell 2) to NR Cell 12 equaling to one of the current UE SCell configuration. v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR, 1); f_EUTRA38_RRC_PSCellChange_InterCell (eutra_Cell1, v_Security_Params.SK_Counter); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 8"); if (not pc_IP_Ping) { //@sic R5s201471 R5-205577 sic@ f_EUTRA_OpenUE_TestLoopMode_Deactivate_TestMode (eutra_Cell1); } }

## Change 5

| Function name | f_NR_SS_SRBs_DRBs_Release() |
| --- | --- |
| Reason for change | This function is called through f_NR_ReconfigurePCellToNormalCell() and is not previously used for EN-DC test cases, and now needs to take into account that SRBs will not have been configured on the SS NR cell in these scenarios. |
| Summary of change | SRB1 and SRB2 not added to the release list in the case of EN-DC. |
| TTCN module | NR_ConfigurationSteps |
| MCC160 Comment |  |

function f_NR_SS_SRBs_DRBs_Release(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, IntegerList_Type p_DrbList, template (omit) boolean p_CnfFlag := omit, boolean p_FollowOnFlag := tsc_NoFollowOnFlag ) runs on NR_BASE_PTC { var template (value) NR_RadioBearerList_Type v_SrbDrbList := { cs_NR_SS_RadioBearer_Release(tsc_NR_RbId_SRB1), cs_NR_SS_RadioBearer_Release(tsc_NR_RbId_SRB2) }; var integer i := lengthof(v_SrbDrbList); var integer k; for (k:=0; k < lengthof(p_DrbList); k:= k+1) { v_SrbDrbList[i+k] := cs_NR_SS_DRB_Release(p_DrbList[k]); } f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, v_SrbDrbList, p_TimingInfo, p_CnfFlag, p_FollowOnFlag); }

After Change

function f_NR_SS_SRBs_DRBs_Release(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, IntegerList_Type p_DrbList, template (omit) boolean p_CnfFlag := omit, boolean p_FollowOnFlag := tsc_NoFollowOnFlag ) runs on NR_BASE_PTC { var template (value) NR_RadioBearerList_Type v_SrbDrbList := {}; var integer i; var integer k; if(not f_GetTestcaseAttrib_ENDC(testcasename())) { v_SrbDrbList := { cs_NR_SS_RadioBearer_Release(tsc_NR_RbId_SRB1), cs_NR_SS_RadioBearer_Release(tsc_NR_RbId_SRB2) }; i := lengthof(v_SrbDrbList); for (k:=0; k < lengthof(p_DrbList); k:= k+1) { v_SrbDrbList[i+k] := cs_NR_SS_DRB_Release(p_DrbList[k]); } f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, v_SrbDrbList, p_TimingInfo, p_CnfFlag, p_FollowOnFlag); } }