3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221274

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2648 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to testcase 8.1.6.1.3.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to 38.331 sec 5.3.5.3 Reception of an RRCReconfiguration by the UE 2> if the RRCReconfiguration includes the reconfigurationWithSync in spCellConfig of an MCG: 3> if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport: 4> include connEstFailInfoAvailable in the RRCReconfigurationComplete message; 3> if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or 3> if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]: 4> include rlf-InfoAvailable in the RRCReconfigurationComplete message; since in this test cases we don't include reconfigurationwithSync in RRCReconfiguration complete message its incorrect to expect the UE to send rlf-InfoAvailable |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the check for cr_RRCReconfigurationComplete_v1530 when receiving RRCReconfiguration Complete message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.3.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required for test 8.1.6.1.3.4	3

2.1	Correction to f_TC_8_1_6_1_3_4_TestBody	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for test 8.1.6.1.3.4

## Correction to f_TC_8_1_6_1_3_4_TestBody

| Function name | f_TC_8_1_6_1_3_4_TestBody |
| --- | --- |
| Reason for change | According to 38.331 sec 5.3.5.3 Reception of an RRCReconfiguration by the UE 2> if the RRCReconfiguration includes the reconfigurationWithSync in spCellConfig of an MCG: 3> if the UE has connection establishment failure or connection resume failure information available in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport: 4> include connEstFailInfoAvailable in the RRCReconfigurationComplete message; 3> if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or 3> if the UE has radio link failure or handover failure information available in VarRLF-Report of TS 36.331 [10] and if the UE is capable of cross-RAT RLF reporting and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report of TS 36.331 [10]: 4> include rlf-InfoAvailable in the RRCReconfigurationComplete message; since in this test cases we don't include reconfigurationwithSync in RRCReconfiguration complete message its incorrect to expect the UE to send rlf-InfoAvailable |
| Summary of change | Removed the check for cr_RRCReconfigurationComplete_v1530 when receiving RRCReconfiguration Complete message |
| TTCN module | RRC_MDT_IntraNR_RLF_NR5GC |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_6_1_3_4_TestBody() runs on NR5GC_PTC { var template (value) DL_DCCH_Message v_RRCReconfigurationHO; var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); var template RRCReestablishmentComplete_v1610_IEs v_RRCReestablishmentComplete_v1610_IEs;// := cr_RRCReestablishmentComplete_v1610_IEs_RLF; var RNTI_Value v_RNTI_Value:=f_NR_CellInfo_GetRNTI(nr_Cell1) ;//@sic R5s220421 sic@ var IntegerList_Type v_DrbIdList;//@sic R5s220421 sic@ // Reconfigure nr_Cell2 to no respond to any RA Preamble // Table 8.1.4.1.5.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond. f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 1" siclog@ // The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. // Note: failure case UE doesn't go to nr_Cell2 => the standard function need not be used v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, v_RRCReconfigurationHO)); //@siclog "Step 2" siclog@ //The SS changes the power level setting according to the row "T1". f_NR_SetCellPower (nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE);//@sic R5-220054 R5s220421 sic@ //@siclog "Step 3-6" siclog@ //Re-establishment procedure v_DrbIdList := f_NR_GetActiveDRBs(nr_Cell1);//@sic R5s220421 sic@ f_NR_SS_SRBs_DRBs_Release(nr_Cell1, -, v_DrbIdList, tsc_NoCnfReq);//@sic R5s220421 sic@ f_NR_SS_SRBs_DRBs_Config(nr_Cell1, -, omit, true);//@sic R5s220421 sic@ v_RRCReestablishmentComplete_v1610_IEs := cr_RRCReestablishmentComplete_v1610_IEs_RLF; f_NR5GC_RRCReestablishment_Steps0To2(nr_Cell1, nr_Cell1); //@sic R5s220421 sic@ f_NR5GC_RRCReestablishment_Steps3To7(nr_Cell1, 2, -, v_RRCReestablishmentComplete_v1610_IEs); //@sic R5s220421 sic@ f_NR5GC_RRCReestablishment_Steps8To10(nr_Cell1, nr_Cell1,-, -, false); //@sic R5s220421 sic@ //@siclog "Step 7" siclog@ //Re-establishment procedure //Check: Does the UE transmit an RRCReconfigurationtComplete message with rlf-InfoAvailable included? SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete( cr_RRCReconfigurationComplete_v1530(-, cr_RRCReconfigurationComplete_v1560(-,cr_RRCReconfigurationComplete_v1610_RLF))))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7");

After Change:

function f_TC_8_1_6_1_3_4_TestBody() runs on NR5GC_PTC { var template (value) DL_DCCH_Message v_RRCReconfigurationHO; var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); var template RRCReestablishmentComplete_v1610_IEs v_RRCReestablishmentComplete_v1610_IEs;// := cr_RRCReestablishmentComplete_v1610_IEs_RLF; var RNTI_Value v_RNTI_Value:=f_NR_CellInfo_GetRNTI(nr_Cell1) ;//@sic R5s220421 sic@ var IntegerList_Type v_DrbIdList;//@sic R5s220421 sic@ // Reconfigure nr_Cell2 to no respond to any RA Preamble // Table 8.1.4.1.5.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond. f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 1" siclog@ // The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. // Note: failure case UE doesn't go to nr_Cell2 => the standard function need not be used v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, v_RRCReconfigurationHO)); //@siclog "Step 2" siclog@ //The SS changes the power level setting according to the row "T1". f_NR_SetCellPower (nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE);//@sic R5-220054 R5s220421 sic@ //@siclog "Step 3-6" siclog@ //Re-establishment procedure v_DrbIdList := f_NR_GetActiveDRBs(nr_Cell1);//@sic R5s220421 sic@ f_NR_SS_SRBs_DRBs_Release(nr_Cell1, -, v_DrbIdList, tsc_NoCnfReq);//@sic R5s220421 sic@ f_NR_SS_SRBs_DRBs_Config(nr_Cell1, -, omit, true);//@sic R5s220421 sic@ v_RRCReestablishmentComplete_v1610_IEs := cr_RRCReestablishmentComplete_v1610_IEs_RLF; f_NR5GC_RRCReestablishment_Steps0To2(nr_Cell1, nr_Cell1); //@sic R5s220421 sic@ f_NR5GC_RRCReestablishment_Steps3To7(nr_Cell1, 2, -, v_RRCReestablishmentComplete_v1610_IEs); //@sic R5s220421 sic@ f_NR5GC_RRCReestablishment_Steps8To10(nr_Cell1, nr_Cell1,-, -, false); //@sic R5s220421 sic@ //@siclog "Step 7" siclog@ //Re-establishment procedure //Check: Does the UE transmit an RRCReconfigurationtComplete message with rlf-InfoAvailable included? SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)); // removed checking for non critical extension f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7");