3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221287

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0881 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS 5GS Call Control test case 7.21 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During test case 7.21 execution after a MO audio call has been established, SS tries to modify the call adding video media. Current implementation, in order to accept the call modification sends the MMI command “ATA” which is used to actually accept a call. In this scenario, the call is already accepted hence sending this command to make UE accept the addition of video media might be inaccurate. In addition, MMI command to modify the video call to an audio call is duplicated: first is sent at Step 8 and then sent again while executing f_IMS_VideoCall_NR5GC_ReleaseVideoMO function. Proposal is to send an MMI command aligned with the action expected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Changed the MMI command sent in Step 3a6 and Step 3b1. Removed duplicated MMI command sent in Step 8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.21.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	3

2.1	New function f_UT_AcceptVideoCallModification	3

2.2	Correction to enumerated Ut_Common_Req_Type	3

2.3	New template cs_AT_ACCEPT_VIDEO_CALL_MODIFICATION	4

2.4	Correction to function fl_UT_HandleCmd	4

2.5	Correction to function f_TC_7_21_NR5GC_IMS1	5

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## New function f_UT_AcceptVideoCallModification

| Function name | f_UT_AcceptVideoCallModification |
| --- | --- |
| Reason for change | Current implementation, in order to accept the call modification sends the MMI command “ATA” which is used to actually accept a call. In this scenario, the call is already accepted hence sending this command to make UE accept the addition of video media might be inaccurate. |
| Summary of change | Added new function to send a MMI command to accept video media for current audio call. |
| TTCN module | \Common\Common\UpperTesterFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_UT_AcceptVideoCallModification(UT_PTC_MTC_PORT p_Port, charstring p_CDUIdentifier) { fl_UT_ApplyCommand(p_Port, cas_UT_Req(ACCEPT_VIDEO_CALL_MODIFICATION, NO_CNF_REQUIRED)); } <<SKIPPED CODE>>

## Correction to enumerated Ut_Common_Req_Type

| Enumerated name | Ut_Common_Req_Type |
| --- | --- |
| Reason for change | Current implementation, in order to accept the call modification sends the MMI command “ATA” which is used to actually accept a call. In this scenario, the call is already accepted hence sending this command to make UE accept the addition of video media might be inaccurate. |
| Summary of change | Added new entry to type enumerated Ut_Common_Req_Type. |
| TTCN module | \Common\Common\UpperTesterFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> type enumerated Ut_Common_Req_Type { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, MCX, NBIOT, NR5GC, NR5GC_IRAT, POS, SSNITZ, UTRAN) */ ACCEPT_MTSI_TEXT, ACTIVATE_BEARER, ACTIVATE_MESSAGE_WAIT_INDICATION, ACTIVATE_TEST_MODE_E, APPLAYER_MEASREPORT, <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> type enumerated Ut_Common_Req_Type { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, MCX, NBIOT, NR5GC, NR5GC_IRAT, POS, SSNITZ, UTRAN) */ ACCEPT_MTSI_TEXT, ACCEPT_VIDEO_CALL_MODIFICATION, ACTIVATE_BEARER, ACTIVATE_MESSAGE_WAIT_INDICATION, ACTIVATE_TEST_MODE_E, APPLAYER_MEASREPORT, <<SKIPPED CODE>>

## New template cs_AT_ACCEPT_VIDEO_CALL_MODIFICATION

| Template name | cs_AT_ACCEPT_VIDEO_CALL_MODIFICATION |
| --- | --- |
| Reason for change | Current implementation, in order to accept the call modification sends the MMI command “ATA” which is used to actually accept a call. In this scenario, the call is already accepted hence sending this command to make UE accept the addition of video media might be inaccurate. |
| Summary of change | Added new template to send MMI "ACCEPT_VIDEO_CALL_MODIFICATION”. |
| TTCN module | \Common\Common\UpperTesterCmds.ttcn |
| MCC160 Comment |  |

New template:

<<SKIPPED CODE>> template (value) Ut_System_Cmd_Type cs_AT_ACCEPT_VIDEO_CALL_MODIFICATION := cs_AT_Cmd("ACCEPT_VIDEO_CALL_MODIFICATION"); <<SKIPPED CODE>>

## Correction to function fl_UT_HandleCmd

| Function name | fl_UT_HandleCmd |
| --- | --- |
| Reason for change | Current implementation, in order to accept the call modification sends the MMI command “ATA” which is used to actually accept a call. In this scenario, the call is already accepted hence sending this command to make UE accept the addition of video media might be inaccurate. |
| Summary of change | Added new case to function fl_UT_HandleCmd to handle video media addition to an audio call. |
| TTCN module | \Common\MTC\MTC_UpperTester.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> case (RELEASE_IMS_CALL) { v_SystemCmd := cs_AT_RELEASE_IMS_CALL (v_ParamListVal[0].Value); } case (INITIATE_VIDEO_CALL) { v_SystemCmd := cs_AT_START_IMS_VIDEO_CALL(v_ParamListVal[0].Value, v_ParamListVal[1].Value); } case (IMS_CONFIGURE_VIDEO_CALL) { v_SystemCmd := cs_AT_IMS_CONFIGURE_VIDEO_CALL; } case (REQUEST_CALL_INFO) { v_SystemCmd := cs_AT_REQUEST_CALL_INFO; } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> case (RELEASE_IMS_CALL) { v_SystemCmd := cs_AT_RELEASE_IMS_CALL (v_ParamListVal[0].Value); } case (INITIATE_VIDEO_CALL) { v_SystemCmd := cs_AT_START_IMS_VIDEO_CALL(v_ParamListVal[0].Value, v_ParamListVal[1].Value); } case (IMS_CONFIGURE_VIDEO_CALL) { v_SystemCmd := cs_AT_IMS_CONFIGURE_VIDEO_CALL; } case (ACCEPT_VIDEO_CALL_MODIFICATION) { v_SystemCmd := cs_AT_ACCEPT_VIDEO_CALL_MODIFICATION; } case (REQUEST_CALL_INFO) { v_SystemCmd := cs_AT_REQUEST_CALL_INFO; } <<SKIPPED CODE>>

## Correction to function f_TC_7_21_NR5GC_IMS1

| Function name | f_TC_7_21_NR5GC_IMS1 |
| --- | --- |
| Reason for change | - Current implementation, in order to accept the call modification sends the MMI command “ATA” which is used to actually accept a call. In this scenario, the call is already accepted hence sending this command to make UE accept the addition of video media might be inaccurate. - In addition, MMI command to modify the video call to an audio call is duplicated: first is sent at Step 8 and then sent again while executing f_IMS_VideoCall_NR5GC_ReleaseVideoMO function. |
| Summary of change | 1. Changed the MMI command sent in Step 3a6 and Step 3b1. 2. Removed duplicated MMI command sent in Step 8. |
| TTCN module | \Common\Common\UpperTesterFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> if (isvalue(v_IMS_DATA_RSP)) { v_Response := valueof(v_IMS_DATA_RSP.Response); v_SdpAnswer := f_IMS_SIP_SdpMessageBody_DecodeMatchAndCheckSDP(v_Response.messageBody, v_ExpectedSdpAnswer); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3a1"); // @siclog "Step 3a2" siclog@ Stop timer t_Timer.stop; // @siclog "Step 3a3 - 3a4" siclog@ PRACK, 200 OK f_IMS_MTCallSetup_SendPRACK_ReceiveOK(v_InviteRequestWithSdpVideo.Invite, v_Response.msgHeader); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3a4"); // @siclog "Step 3a5" siclog@ f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MT_AddVideo); f_IMS_IPCAN_WaitForTrigger(IPCAN); // @siclog "Step 3a6 - 3a8" siclog@ MMI (Accept Call), 200 OK, ACK f_IMS_MTCallSetup_AcceptAndAcknowledge(v_InviteRequestWithSdpVideo); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3a7"); } else { // @siclog "Step 3b1" siclog@ MMI (Accept Call) f_UT_AnswerCall(MMI); // @siclog "Step 3b2" siclog@ 200 OK v_ResponseWithSdp := f_IMS_MTCallSetup_Receive200OKforINVITE(v_InviteRequestWithSdpVideo, v_ExpectedSdpAnswer); v_SdpAnswer := v_ResponseWithSdp.SdpMessage; f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3b2"); // @siclog "Step 3b3" siclog@ f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MT_AddVideo); f_IMS_IPCAN_WaitForTrigger(IPCAN); // @siclog "Step 3b4" siclog@ ACK f_IMS_InviteMT_SendACK(v_InviteRequestWithSdpVideo); } // @siclog "Step 4 - 7" Void siclog@ // @siclog "Step 8" siclog@ f_UT_IMS_RemoveVideoMedia(MMI, v_CallIdentification); // @siclog "Step 8 - 13" siclog@ MMI (Remove Video), INVITE, 100 Trying, 200 OK, ACK f_IMS_VideoCall_NR5GC_ReleaseVideoMO(v_SdpAnswer, v_CallIdentification); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 9, 13"); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> if (isvalue(v_IMS_DATA_RSP)) { v_Response := valueof(v_IMS_DATA_RSP.Response); v_SdpAnswer := f_IMS_SIP_SdpMessageBody_DecodeMatchAndCheckSDP(v_Response.messageBody, v_ExpectedSdpAnswer); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3a1"); // @siclog "Step 3a2" siclog@ Stop timer t_Timer.stop; // @siclog "Step 3a3 - 3a4" siclog@ PRACK, 200 OK f_IMS_MTCallSetup_SendPRACK_ReceiveOK(v_InviteRequestWithSdpVideo.Invite, v_Response.msgHeader); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3a4"); // @siclog "Step 3a5" siclog@ f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MT_AddVideo); f_IMS_IPCAN_WaitForTrigger(IPCAN); // @siclog "Step 3a6 - 3a8" siclog@ MMI (Accept Call), 200 OK, ACK f_UT_AcceptVideoCallModification(MMI); // @sic R5-210056 sic@ f_IMS_MTCallSetup_Receive200OK_SendACK(v_InviteRequestWithSdpVideo); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3a7"); } else { // @siclog "Step 3b1" siclog@ MMI (Accept Call) f_UT_AcceptVideoCallModification(MMI); // @siclog "Step 3b2" siclog@ 200 OK v_ResponseWithSdp := f_IMS_MTCallSetup_Receive200OKforINVITE(v_InviteRequestWithSdpVideo, v_ExpectedSdpAnswer); v_SdpAnswer := v_ResponseWithSdp.SdpMessage; f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3b2"); // @siclog "Step 3b3" siclog@ f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_MT_AddVideo); f_IMS_IPCAN_WaitForTrigger(IPCAN); // @siclog "Step 3b4" siclog@ ACK f_IMS_InviteMT_SendACK(v_InviteRequestWithSdpVideo); } // @siclog "Step 4 - 7" Void siclog@ // @siclog "Step 8" siclog@ //f_UT_IMS_RemoveVideoMedia(MMI, v_CallIdentification); // @siclog "Step 8 - 13" siclog@ MMI (Remove Video), INVITE, 100 Trying, 200 OK, ACK f_IMS_VideoCall_NR5GC_ReleaseVideoMO(v_SdpAnswer, v_CallIdentification); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 9, 13"); // @siclog "Step 14 - 16" siclog@ MMI (Release Call), BYE, 200 OK f_IMS_CallReleaseMO(v_InviteRequestWithSdpAudio.Invite); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MO_SpeechCall); } <<SKIPPED CODE>>