3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221290

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2853 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the NR5GC testcase 11.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of 11.4.1, the function f_NR5GC_EmergencyCallRelease is called at Step 4. It is observed that a fast UE can send a De-registration REQ immediately after the PDU Session Release Complete, which causes testcase failure. What happens is that in the sequence of calling f_NR5GC_EmergencyCallRelease f_NR5GC_PDUSessionRelease f_IP_RtpRtcpLoopback f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release"); The De-registration REQ is received when control is in function f_IP_RtpRtcpLoopback. This needs to be handled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to create and call a local default handler which handles the De-registration REQ, and activate this prior to calling function f_NR5GC_EmergencyCallRelease. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to function fl_TC_11_4_1_TestBody	3

2.2	a_NR5GC_Deregistration_REQ	5

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to function fl_TC_11_4_1_TestBody

| Template name | fl_TC_11_4_1_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation of 11.4.1, the function f_NR5GC_EmergencyCallRelease is called at Step 4. It is observed that a fast UE can send a De-registration REQ immediately after the PDU Session Release Complete, which causes testcase failure. What happens is that in the sequence of calling f_NR5GC_EmergencyCallRelease f_NR5GC_PDUSessionRelease f_IP_RtpRtcpLoopback f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release"); The De-registration REQ is received when control is in function f_IP_RtpRtcpLoopback. This needs to be handled. |
| Summary of change | It is proposed to create and call a local default handler which handles the De-registration REQ, and activate this prior to calling function f_NR5GC_EmergencyCallRelease. |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 3" siclog@ //Make the UE release the emergency call. //@siclog "Step 4" siclog@ //The Generic test procedure for IMS MO Emergency call release f_NR5GC_EmergencyCallRelease(nr_Cell1); //@siclog "Step 4A" siclog@ //Start Timer T1=5 seconds. //@sic R5s221039 sic@ t_WaitTime.start(5.0); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> var default v_MyDefaultVar := null; //WA#11_4_1 . . . //@siclog "Step 3" siclog@ //Make the UE release the emergency call. //@siclog "Step 4" siclog@ //The Generic test procedure for IMS MO Emergency call release v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(nr_Cell1)); //WA#11_4_1 log("Parikshit : Default to handle Deregistration activated"); f_NR5GC_EmergencyCallRelease(nr_Cell1); deactivate(v_MyDefaultVar); //WA#11_4_1 //@siclog "Step 4A" siclog@ //Start Timer T1=5 seconds. //@sic R5s221039 sic@ t_WaitTime.start(5.0); <<SKIPPED CODE>>

## a_NR5GC_Deregistration_REQ

| Template name | a_NR5GC_Deregistration_REQ |
| --- | --- |
| Reason for change | Please see change 2.1 above |
| Summary of change | New function |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment |  |

altstep a_NR5GC_Deregistration_REQ(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var boolean v_DeregistrationReceived := false; [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered, cr_NG_DEREGISTRATION_REQUEST_MO(crs_DeregisterType('0'B, '0'B,'01'B),?,?)))) -> value v_ReceivedAsp { SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB2, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_DEREGISTRATION_ACCEPT))); repeat; } }