3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221304

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4688 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE MFBI testcase 6.1.2.20 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2022-09-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per Table 6.1.2.20.3.3-2 [TS 36.523-1], the value of freqBandIndicator IE in SIB5 should be set to 64, when mFBI band > 64. But currently ttcn sets it with the value of px_MFBI_FrequencyBand parameter, which causes encoding error during runtime. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The value of freqBandIndicator IE in SIB5 is set to 64, when mFBI band > 64. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case ends up in runtime error |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_6_1_2_20_EUTRA |
| --- | --- |
| Reason for change | As per Table 6.1.2.20.3.3-2 [TS 36.523-1], the value of freqBandIndicator IE in SIB5 should be set to 64, when mFBI band > 64. But currently ttcn sets it with the value of px_MFBI_FrequencyBand parameter, which causes encoding error during runtime. |
| Summary of change | The value of freqBandIndicator IE in SIB5 is set to 64, when mFBI band > 64. |
| TTCN module | LTE\6_1\Idle_CellReselection.ttcn |
| MCC160 Comment |  |

Before change:

function f_TC_6_1_2_20_EUTRA() runs on EUTRA_PTC

{

var template (value) CellPowerList_Type v_CellPowerList_AtT1;

var Frequency_fList_Type v_Freq;

var ChannelBandwidthDependency_Type v_ChannelBandwidthDependency;

var template (value) InterFreqCarrierFreqList  v_InterFreqCarrierList;

var template (omit) InterFreqCarrierFreqInfo_v9e0_List_Type v_InterFreqCarrierListExt := omit;  //@sic R5s201503 sic@

var integer v_Band; //@sic R5-171110 band>64 sic@

v_CellPowerList_AtT1 := {

cs_CellPower (eutra_Cell3, -73)

};

//Initialise all cell, security and mobile parameters

f_EUTRA_Init(c3);

//Initialise the Band in SIB1 FreqBandIndicator and the MFBI frequency

//@sic R5-142974 sic@

//Cell 1 belongs to the absolute centre frequency which overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBandMFBI.

//Frequencies are defined in 36.508 Table 6.2.3.1-1b, Table 6.2.3.1-1c, Table 6.2.3.1-2b and Table 6.2.3.1-2c

f_EUTRA_InitBandFrequency_MFBI(eutra_Cell1, px_OverlappingNotSupportedFrequencyBand_MFBI, px_MFBI_FrequencyBand, px_MFBI_BandChannelBandwidth, f_ConvertDL_BandwidthToUL(px_MFBI_BandChannelBandwidth));

//Cell 3 belongs to another absolute centre frequency which again overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBandMFBI

//Frequencies are defined in 36.508 Table 6.2.3.1-1b, Table 6.2.3.1-1c, Table 6.2.3.1-2b and Table 6.2.3.1-2c

f_EUTRA_InitBandFrequency_MFBI(eutra_Cell3, px_OverlappingNotSupportedFrequencyBand_MFBI, px_MFBI_FrequencyBand, px_MFBI_BandChannelBandwidth, f_ConvertDL_BandwidthToUL(px_MFBI_BandChannelBandwidth));

//Set TAC according to 36.523-1 6.0.1-2

//TAC for Cell 1 is set to 1 in f_EUTRA_Init ();

f_EUTRA_CellInfo_SetTAC(eutra_Cell3, tsc_IdleMode_TAC_Cell3);

// Set SIB1 according to Table 6.1.2.20.3.3-1

//@sic R5-142974 R5-144651 sic@

// Set New SIB5 neighbour cell list

v_Freq := f_EUTRA_InitFrequency_f1Tof4_MFBI(px_OverlappingNotSupportedFrequencyBand_MFBI, px_MFBI_FrequencyBand, px_MFBI_BandChannelBandwidth, f_ConvertDL_BandwidthToUL(px_MFBI_BandChannelBandwidth));

v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(px_MFBI_BandChannelBandwidth, f_ConvertDL_BandwidthToUL(px_MFBI_BandChannelBandwidth));

//@sic R5-166250: band extension sic@

v_Band := f_EUTRA_CellInfo_GetBand(eutra_Cell1);

v_InterFreqCarrierList := { cs_InterFreqCarrierFreq(v_Freq.f2.dl_CarrierFreq, v_ChannelBandwidthDependency.AllowedMeasBandwidth)};

if (v_Band > 64) {

v_InterFreqCarrierListExt := { cs_InterFreqCarrierFreq_v9e0(v_Freq.f2.dl_CarrierFreq_v9e0)};

}

f_EUTRA_CellInfo_SetSIB5( eutra_Cell1, cs_508_SystemInformationBlockType5_Def(v_InterFreqCarrierList, v_InterFreqCarrierListExt));

//@sic R5s140586 R5-166250 sic@

v_InterFreqCarrierList := { cs_InterFreqCarrierFreq(v_Freq.f1.dl_CarrierFreq, v_ChannelBandwidthDependency.AllowedMeasBandwidth)};

if (v_Band > 64){

v_InterFreqCarrierListExt := { cs_InterFreqCarrierFreq_v9e0(v_Freq.f1.dl_CarrierFreq_v9e0)};

}

f_EUTRA_CellInfo_SetSIB5( eutra_Cell3, cs_508_SystemInformationBlockType5_Def(v_InterFreqCarrierList, v_InterFreqCarrierListExt));

// Set InterFreqCarrierFreqInfo-v8h0 in SIB5 according to Table 6.1.2.20.3.3-2

if (px_MFBI_FrequencyBand<maxFBI) {

f_EUTRA_CellInfo_SetSysInfo_SIB5_v8h0_IEs(eutra_Cell1, cs_SystemInformationBlockType5_v8h0_IEs({cs_InterFreqCarrierFreqInfo_v8h0({px_MFBI_FrequencyBand})})); //@sic R5s130969 sic@

f_EUTRA_CellInfo_SetSysInfo_SIB5_v8h0_IEs(eutra_Cell3, cs_SystemInformationBlockType5_v8h0_IEs({cs_InterFreqCarrierFreqInfo_v8h0({px_MFBI_FrequencyBand})})); //@sic R5s130969 sic@

} else {

v_InterFreqCarrierListExt := { cs_InterFreqCarrierFreq_v9e0(omit, cs_MultiBandInfoList1_v9e0(px_MFBI_FrequencyBand))};

f_EUTRA_CellInfo_SetSysInfo_SIB5_v8h0_IEs(eutra_Cell1, cs_SystemInformationBlockType5_v8h0_Freq( v_InterFreqCarrierListExt,

{cs_InterFreqCarrierFreqInfo_v8h0({px_MFBI_FrequencyBand})}));

f_EUTRA_CellInfo_SetSysInfo_SIB5_v8h0_IEs(eutra_Cell3, cs_SystemInformationBlockType5_v8h0_Freq( v_InterFreqCarrierListExt,

{cs_InterFreqCarrierFreqInfo_v8h0({px_MFBI_FrequencyBand})}));

}

//Set SIB2 acc to Table 6.1.2.20.3.3-1A @sic R5-145162 sic@

f_EUTRA_CellInfo_SetSysInfo_SIB2_v8h0_IEs(eutra_Cell1,cs_SystemInformationBlockType2_v8h0_IEs({1}));

f_EUTRA_CellInfo_SetSysInfo_SIB2_v8h0_IEs(eutra_Cell3,cs_SystemInformationBlockType2_v8h0_IEs({1})); //@sic R5s141366 sic@

//Set maximum cell power level for cell 3

f_EUTRA_CellInfo_InitMaxReferencePower(eutra_Cell3, -73);

//Create and configure all cells

f_EUTRA_CellConfig_Def(eutra_Cell1);

f_EUTRA_CellConfig_Def(eutra_Cell3);

//Bring UE to initial state

f_EUTRA_Preamble(eutra_Cell1, STATE2_IDLEUPDATE);

// Check that the "px_OverlappingNotSupportedFrequencyBand_MFBI" band is not supported by UE

//@sic R5-142974 sic@

f_EUTRA_CheckNotSupportedBand( px_OverlappingNotSupportedFrequencyBand_MFBI );

f_EUTRA_TestBody_Set(true);

//@siclog "Step 0" siclog@

//Wait 1 second. (to ensure than 1 second has elapsed since the UE camped on the current serving cell)

f_Delay(1.0);

//@siclog "Step 1" siclog@

//The SS changes the cells power level setting according to the row "T1" in table 6.1.2.20.3.2-1.

f_EUTRA_SetCellPowerList(v_CellPowerList_AtT1);

//@siclog "Step 2" siclog@

//Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 2?

//NOTE: The UE performs a TAU procedure and the RRC connection is released.

f_EUTRA_IdleMode_TAU(eutra_Cell3);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.2.20 Step 2");

f_EUTRA_TestBody_Set(false);

//Switch/power off UE

f_EUTRA_Postamble(eutra_Cell3, E1_IDLE);

}

After change:

function f_TC_6_1_2_20_EUTRA() runs on EUTRA_PTC

{

var template (value) CellPowerList_Type v_CellPowerList_AtT1;

var Frequency_fList_Type v_Freq;

var ChannelBandwidthDependency_Type v_ChannelBandwidthDependency;

var template (value) InterFreqCarrierFreqList  v_InterFreqCarrierList;

var template (omit) InterFreqCarrierFreqInfo_v9e0_List_Type v_InterFreqCarrierListExt := omit;  //@sic R5s201503 sic@

var integer v_Band; //@sic R5-171110 band>64 sic@

v_CellPowerList_AtT1 := {

cs_CellPower (eutra_Cell3, -73)

};

//Initialise all cell, security and mobile parameters

f_EUTRA_Init(c3);

//Initialise the Band in SIB1 FreqBandIndicator and the MFBI frequency

//@sic R5-142974 sic@

//Cell 1 belongs to the absolute centre frequency which overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBandMFBI.

//Frequencies are defined in 36.508 Table 6.2.3.1-1b, Table 6.2.3.1-1c, Table 6.2.3.1-2b and Table 6.2.3.1-2c

f_EUTRA_InitBandFrequency_MFBI(eutra_Cell1, px_OverlappingNotSupportedFrequencyBand_MFBI, px_MFBI_FrequencyBand, px_MFBI_BandChannelBandwidth, f_ConvertDL_BandwidthToUL(px_MFBI_BandChannelBandwidth));

//Cell 3 belongs to another absolute centre frequency which again overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBandMFBI

//Frequencies are defined in 36.508 Table 6.2.3.1-1b, Table 6.2.3.1-1c, Table 6.2.3.1-2b and Table 6.2.3.1-2c

f_EUTRA_InitBandFrequency_MFBI(eutra_Cell3, px_OverlappingNotSupportedFrequencyBand_MFBI, px_MFBI_FrequencyBand, px_MFBI_BandChannelBandwidth, f_ConvertDL_BandwidthToUL(px_MFBI_BandChannelBandwidth));

//Set TAC according to 36.523-1 6.0.1-2

//TAC for Cell 1 is set to 1 in f_EUTRA_Init ();

f_EUTRA_CellInfo_SetTAC(eutra_Cell3, tsc_IdleMode_TAC_Cell3);

// Set SIB1 according to Table 6.1.2.20.3.3-1

//@sic R5-142974 R5-144651 sic@

// Set New SIB5 neighbour cell list

v_Freq := f_EUTRA_InitFrequency_f1Tof4_MFBI(px_OverlappingNotSupportedFrequencyBand_MFBI, px_MFBI_FrequencyBand, px_MFBI_BandChannelBandwidth, f_ConvertDL_BandwidthToUL(px_MFBI_BandChannelBandwidth));

v_ChannelBandwidthDependency := f_EUTRA_BandDependentParam(px_MFBI_BandChannelBandwidth, f_ConvertDL_BandwidthToUL(px_MFBI_BandChannelBandwidth));

//@sic R5-166250: band extension sic@

v_Band := f_EUTRA_CellInfo_GetBand(eutra_Cell1);

v_InterFreqCarrierList := { cs_InterFreqCarrierFreq(v_Freq.f2.dl_CarrierFreq, v_ChannelBandwidthDependency.AllowedMeasBandwidth)};

if (v_Band > 64) {

v_InterFreqCarrierListExt := { cs_InterFreqCarrierFreq_v9e0(v_Freq.f2.dl_CarrierFreq_v9e0)};

}

f_EUTRA_CellInfo_SetSIB5( eutra_Cell1, cs_508_SystemInformationBlockType5_Def(v_InterFreqCarrierList, v_InterFreqCarrierListExt));

//@sic R5s140586 R5-166250 sic@

v_InterFreqCarrierList := { cs_InterFreqCarrierFreq(v_Freq.f1.dl_CarrierFreq, v_ChannelBandwidthDependency.AllowedMeasBandwidth)};

if (v_Band > 64){

v_InterFreqCarrierListExt := { cs_InterFreqCarrierFreq_v9e0(v_Freq.f1.dl_CarrierFreq_v9e0)};

}

f_EUTRA_CellInfo_SetSIB5( eutra_Cell3, cs_508_SystemInformationBlockType5_Def(v_InterFreqCarrierList, v_InterFreqCarrierListExt));

// Set InterFreqCarrierFreqInfo-v8h0 in SIB5 according to Table 6.1.2.20.3.3-2

if (px_MFBI_FrequencyBand<maxFBI) {

f_EUTRA_CellInfo_SetSysInfo_SIB5_v8h0_IEs(eutra_Cell1, cs_SystemInformationBlockType5_v8h0_IEs({cs_InterFreqCarrierFreqInfo_v8h0({px_MFBI_FrequencyBand})})); //@sic R5s130969 sic@

f_EUTRA_CellInfo_SetSysInfo_SIB5_v8h0_IEs(eutra_Cell3, cs_SystemInformationBlockType5_v8h0_IEs({cs_InterFreqCarrierFreqInfo_v8h0({px_MFBI_FrequencyBand})})); //@sic R5s130969 sic@

} else {

v_InterFreqCarrierListExt := { cs_InterFreqCarrierFreq_v9e0(omit, cs_MultiBandInfoList1_v9e0(px_MFBI_FrequencyBand))};

f_EUTRA_CellInfo_SetSysInfo_SIB5_v8h0_IEs(eutra_Cell1, cs_SystemInformationBlockType5_v8h0_Freq( v_InterFreqCarrierListExt,

{cs_InterFreqCarrierFreqInfo_v8h0({maxFBI})}));

f_EUTRA_CellInfo_SetSysInfo_SIB5_v8h0_IEs(eutra_Cell3, cs_SystemInformationBlockType5_v8h0_Freq( v_InterFreqCarrierListExt,

{cs_InterFreqCarrierFreqInfo_v8h0({maxFBI})}));

}

//Set SIB2 acc to Table 6.1.2.20.3.3-1A @sic R5-145162 sic@

f_EUTRA_CellInfo_SetSysInfo_SIB2_v8h0_IEs(eutra_Cell1,cs_SystemInformationBlockType2_v8h0_IEs({1}));

f_EUTRA_CellInfo_SetSysInfo_SIB2_v8h0_IEs(eutra_Cell3,cs_SystemInformationBlockType2_v8h0_IEs({1})); //@sic R5s141366 sic@

//Set maximum cell power level for cell 3

f_EUTRA_CellInfo_InitMaxReferencePower(eutra_Cell3, -73);

//Create and configure all cells

f_EUTRA_CellConfig_Def(eutra_Cell1);

f_EUTRA_CellConfig_Def(eutra_Cell3);

//Bring UE to initial state

f_EUTRA_Preamble(eutra_Cell1, STATE2_IDLEUPDATE);

// Check that the "px_OverlappingNotSupportedFrequencyBand_MFBI" band is not supported by UE

//@sic R5-142974 sic@

f_EUTRA_CheckNotSupportedBand( px_OverlappingNotSupportedFrequencyBand_MFBI );

f_EUTRA_TestBody_Set(true);

//@siclog "Step 0" siclog@

//Wait 1 second. (to ensure than 1 second has elapsed since the UE camped on the current serving cell)

f_Delay(1.0);

//@siclog "Step 1" siclog@

//The SS changes the cells power level setting according to the row "T1" in table 6.1.2.20.3.2-1.

f_EUTRA_SetCellPowerList(v_CellPowerList_AtT1);

//@siclog "Step 2" siclog@

//Check: Does the test result of generic test procedure in TS 36.508 subclause 6.4.2.7 indicate that the UE is camped on E-UTRAN Cell 2?

//NOTE: The UE performs a TAU procedure and the RRC connection is released.

f_EUTRA_IdleMode_TAU(eutra_Cell3);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.1.2.20 Step 2");

f_EUTRA_TestBody_Set(false);

//Switch/power off UE

f_EUTRA_Postamble(eutra_Cell3, E1_IDLE);

}