3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221305

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4690 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NBIOT testcase 22.4.26 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To verify NBIOT testcase 22.4.26 using IWD 22wk23 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NBIOT testcase 22.4.26 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.26 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Correction to function f_TC_22_4_26_NBIOT	3

3	Branches executed	4

4	Execution Log Files	5

4.1	Hisilicon Hi15EVK	5

5	References	5

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NBIOT testcase 22.4.26 which is part of the iwd-TTCN3-B2020-09_D22wk23.zip test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case:	22.4.26

ATS Version:	iwd-TTCN3-B2020-09_D22wk23.zip

System Simulator used:	R&S CMW 500

UE used:	Hisilicon Hi15EVK UE

Verification Status:	PASS

# Corrections required

## Correction to function f_TC_22_4_26_NBIOT

| Function name | f_TC_22_4_26_NBIOT |
| --- | --- |
| Reason for change | In the current TTCN implementation at step 3 the UE transmits the RR Connection REQ, step 4 RRC Connection setup is sent, test case expects to receive RRC connection Setup Complete at step 5. However, calling function f_NBIOT_RRC_ConnEst_DefWithNas would again expect to receive RRC connection REQ which is incorrect. |
| Summary of change | Removed the call to function f_NBIOT_RRC_ConnEst_DefWithNas and updated the code to expect RRC connection setup complete with security protected NAS_IndicationWithPiggybacking directly at step 5. Please see the details below |
| TTCN module | NBIOT_RRC_ControlPlane.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 5" siclog@ v_NasInd := f_NBIOT_RRC_ConnEst_DefWithNas(nbiot_Cell1, cr_EstablishmentCause_NB_Any, cr_NAS_IndicationWithPiggybacking(tsc_SHT_NoSecurityProtection, cr_CIOT_ATTACH_REQUEST(cr_AdditionalUpdateType_CIOT), v_ESMMessage)); f_NBIOT_PreliminaryPass (__FILE__, __LINE__, "Step 5"); //@siclog "Step 6-15" siclog@ // Steps 5 to 14b1 of the registration procedure described in TS 36.508 subclause 8.1.5.2 are performed on Ncell 1. f_NBIOT_InitialRegistration_Step5_14(nbiot_Cell1, CONTROL_PLANE, STATE2_NB_CONNECTED_UPDATED, v_NasInd); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 5" siclog@ // WA# v_NasInd := f_NBIOT_RRCConnectionSetupComplete_Def(nbiot_Cell1, cr_508_RRCConnectionSetupComplete_NB(tsc_RRC_TI_Def, f_NBIOT_CellInfo_GetSelectedPlmnIndex(nbiot_Cell1)), cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected, cr_CIOT_ATTACH_REQUEST(cr_AdditionalUpdateType_CIOT), v_ESMMessage)); f_NBIOT_PreliminaryPass (__FILE__, __LINE__, "Step 5"); //@siclog "Step 6-15" siclog@ // Steps 5 to 14b1 of the registration procedure described in TS 36.508 subclause 8.1.5.2 are performed on Ncell 1. f_NBIOT_InitialRegistration_Step5_14(nbiot_Cell1, CONTROL_PLANE, STATE2_NB_CONNECTED_UPDATED, v_NasInd); <<SKIPPED CODE>>

# Branches executed

The test case was executed on Eutra band 5 with EEA1/EIA1.

# Execution Log Files

## Hisilicon Hi15EVK

The Hisilicon HiEVK UE passed this test case on R&S CMW 500 Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_22_4_26_Hisilicon.log

(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s221076: Supporting information for Addition of NBIOT testcase 22.4.26 |
| --- | --- |