3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221313

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2745 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR EPS Fallback testcase 11.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-09-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the Prose, for running this testcase the UE should be configured without pre-conditions for IMS. In TTCN, the PICS pc_Preconditions is used to deduce whether the UE is configured with pre-conditions or not. However, pc_Preconditions is a PICS that only represents a capability. Setting this to TRUE would only mean that UE “can be” configured with pre-conditions, it doesn’t mean that the UE “has been” configured with pre-conditions. The TTCN implementation of 11.1.3 makes use of pc_Preconditions, which is incorrect and should be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the functions fl_IMSFallback_RegistrationToMOSpeechCall, f_IMS_EPS_Fallback_Part1 and f_TC_11_1_3_NR5GC_IMS1 and removed the use of pc_Preconditions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required for NR testcase 11.1.3	2

2.1	Correction to fl_IMSFallback_RegistrationToMOSpeechCall	2

2.2	Correction to function f_IMS_EPS_Fallback_Part1	3

2.3	Correction to function f_TC_11_1_3_NR5GC_IMS1	4

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for NR testcase 11.1.3

## Correction to fl_IMSFallback_RegistrationToMOSpeechCall

| Function name | fl_IMSFallback_RegistrationToMOSpeechCall |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function, fl_IMSFallback_RegistrationToMOSpeechCallf_IMS_EPS_Fallback_Part1 makes use of pc_Preconditions. As explained in main change description it is incorrect to use this PICS for this purpose. |
| Summary of change | It is proposed to use a Boolean variable instead of pc_Preconditions and based on that the further flow is decided. Please see below. |
| TTCN module | EPS_Fallback_NR5GC_IMS.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function fl_IMSFallback_RegistrationToMOSpeechCall() runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { fl_IMSFallback_PreambleRegistrations(); return f_IMS_EPS_Fallback_Part1(); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function fl_IMSFallback_RegistrationToMOSpeechCall(boolean p_Preconditions := TRUE) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { fl_IMSFallback_PreambleRegistrations(); if(p_Preconditions) { return f_IMS_EPS_Fallback_Part1(); }else{ return f_IMS_EPS_Fallback_Part1(p_Preconditions); } } <<SKIPPED CODE>>

## Correction to function f_IMS_EPS_Fallback_Part1

| Function name | f_IMS_EPS_Fallback_Part1 |
| --- | --- |
| Reason for change | Please refer to change 2.1 As a result of the above change the function f_IMS_EPS_Fallback_Part1 also needs to be parameterised. |
| Summary of change | Parameterised the function f_IMS_EPS_Fallback_Part1. |
| TTCN module | EPS_Fallback_NR5GC_IMS.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_IMS_EPS_Fallback_Part1() runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { … … } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_IMS_EPS_Fallback_Part1(boolean p_Preconditions := TRUE) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { … … } <<SKIPPED CODE>>

## Correction to function f_TC_11_1_3_NR5GC_IMS1

| Function name | f_TC_11_1_3_NR5GC_IMS1 |
| --- | --- |
| Reason for change | As a result of changes above, the TTCN for function f_TC_11_1_3_NR5GC_IMS1 needs to be updated. |
| Summary of change | After making changes 2.1 and 2.2 above updated the implementation of function f_TC_11_1_3_NR5GC_IMS1 to ensure that the flow for no preconditions is followed, Please see below. |
| TTCN module | EPS_Fallback_NR5GC_IMS.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_TC_11_1_3_NR5GC_IMS1()runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; v_InviteRequestWithSdp := fl_IMSFallback_RegistrationToMOSpeechCall(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_EPS_Fallback_Part2(v_InviteRequestWithSdp); … } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_TC_11_1_3_NR5GC_IMS1()runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; v_InviteRequestWithSdp := fl_IMSFallback_RegistrationToMOSpeechCall(false); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_EPS_Fallback_Part2(v_InviteRequestWithSdp ,-,-,tsc_SDP_WithoutPreconditions); … } <<SKIPPED CODE>>