3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221336

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0090 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of function fl_MCPTT_Info_AmbientListening_RX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some TCs use function fl_MCPTT_Info_AmbientListening_RX to return the expected MCPTT_Info_Type including the XML extensions to be matched with the received MCPTT-Info. These XML extensions are produced by encoding the expected values with the Tester encode. The problem is that XML extensions produced by the Tester encoder results in character strings, which may be different to the XML encoded strings by the UE encoder: for example tabs, \n, blancs and chars like ‘ instead of “ may differ resulting in a mismatch. There is also an issue with namespaces where different encoder will produce different namespace notations. Matching XML subparts which are character strings does not work in general! We need to match unambiguous TTCN objects. |  |  |  |  |  |  |  |  |
| Summary of change: |  | XML extensions are not generated, instead the expected values in the extensions are matched via function f_MCX_McpttInfoExt_DecodeAndMatch. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases will incorrectly fail. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.14.MCPTT, 6.2.18.MCPTT |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk43 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Corrections required

## 2.1 Change 1

| Function name | fl_MCPTT_Info_AmbientListening_RX |
| --- | --- |
| Reason for change | As explained above, matching XML character strings resulting from different encoders does in general not work. We need to match unambiguous TTCN objects instead. |
| Summary of change | Function is changed to produce the MCPTT-Info expected without the extension objects. |
| TTCN module | MCPTT_PrivateCall_MCX_IMS.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_MCPTT_Info_AmbientListening_RX(MCPTT_Info_AmbientListening_Type p_AmbientListeningType) return template (present) MCPTT_Info_Type { var template (present) MCPTT_Info_Type v_MCPTT_Info := cr_MCPTT_Info_Base; // var template (value) MCPTT_Info_Type v_MCPTT_Info := f_MCX_CallControl_MCPTTInfo_RX(PRIVATE_CALL, INVITE_REFER); var McpttInfoExt_Element_Type v_McpttInfoExtAmbientListening := { AmbientListening := p_AmbientListeningType }; var universal charstring v_AnyExtElem := f_MCX_McpttInfoExt_EncodeElem(v_McpttInfoExtAmbientListening); v_MCPTT_Info.mcptt_Params.session_type := "ambient-listening"; v_MCPTT_Info.mcptt_Params.anyExt.elem_list := { *, v_AnyExtElem, * }; return v_MCPTT_Info; }

After Change:

function fl_MCPTT_Info_AmbientListening_RX() return template (present) MCPTT_Info_Type { var template (present) MCPTT_Info_Type v_MCPTT_Info := cr_MCPTT_Info_Base; // var template (value) MCPTT_Info_Type v_MCPTT_Info := f_MCX_CallControl_MCPTTInfo_RX(PRIVATE_CALL, INVITE_REFER); // var McpttInfoExt_Element_Type v_McpttInfoExtAmbientListening := { AmbientListening := p_AmbientListeningType }; // var universal charstring v_AnyExtElem := f_MCX_McpttInfoExt_EncodeElem(v_McpttInfoExtAmbientListening); v_MCPTT_Info.mcptt_Params.session_type := "ambient-listening"; v_MCPTT_Info.mcptt_Params.anyExt.elem_list := ?; return v_MCPTT_Info; }

## 2.2 Change 2

| Function name | f_MCX_McpttInfoExt_DecodeAndMatchAmbientListeningType |
| --- | --- |
| Reason for change | New wrapper function for matching ambient listening XML extension needed at different code locations. |
| Summary of change | New wrapper function implemented. |
| TTCN module | MCPTT_PrivateCall_MCX_IMS.ttcn |
| MCC160 Comment |  |

Before Change:

After Change:

function f_MCX_McpttInfoExt_DecodeAndMatchAmbientListeningType( AnyExtType p_McpttInfoExt, MCPTT_Info_AmbientListening_Type p_AmbientListeningType) runs on IMS_PTC { var template (present) McpttInfoExt_Element_Type v_McpttInfoExtElement := { AmbientListening := p_AmbientListeningType }; var template (present) McpttInfoExt_ElementList_Type v_McpttInfoExtElementList := { v_McpttInfoExtElement }; f_MCX_McpttInfoExt_DecodeAndMatch(p_McpttInfoExt, v_McpttInfoExtElementList); }

## 2.3 Change 3

| Function name | fl_MCX_CallSetupMO_AmbientListening |
| --- | --- |
| Reason for change | As explained above, MCPT-Info extensions containing ambient-listening info need to be checked separately. |
| Summary of change | Ambient listening extensions in received MCPTT info are decoded and checked separately. |
| TTCN module | MCPTT_PrivateCall_MCX_IMS.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_MCX_CallSetupMO_AmbientListening(MCX_CommencementMode_Type p_CommencementMode) runs on MCX_IMS_PTC return IMS_InviteRequestWithSdp_Type { var MCX_InviteRequestWithSdpAndXml_Type v_InviteRequestWithSdpAndXml; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var INVITE_Request v_InviteRequest; var MCX_MediaControlCondition_Type v_MediaControlConditionSdpOffer; var MCX_MediaControlConditionSdpAnswer_Type v_MediaControlConditionSdpAnswer; var MCPTT_Info_AmbientListening_Type v_AmbientListeningType; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (value) Contact v_Contact; var template (value) MessageHeader v_MessageHeader200OK; var template (value) SDP_Message v_SDP_Answer; var template (value) SDP_attribute v_DirectionAttributeRX; var template (present) MCX_SdpInfoRX_Type v_SdpInfoRX; var template (present) MCX_MIME_XmlBodies_Type v_XmlBodies; select (p_CommencementMode) { case (AMBIENT_LISTENING_REMOTE) { v_AmbientListeningType := remote_init; v_DirectionAttributeRX := cs_SDP_Attribute_recvonly; v_MediaControlConditionSdpOffer := withoutImplictGrantRequest; v_MediaControlConditionSdpAnswer := implicitRequestNoted; } case (AMBIENT_LISTENING_LOCAL) { v_AmbientListeningType := local_init; v_DirectionAttributeRX := cs_SDP_Attribute_sendonly; v_MediaControlConditionSdpOffer := withImplictGrantRequest; v_MediaControlConditionSdpAnswer := implicitRequestGranted; } } // SDP info v_SdpInfoRX := f_MCX_SdpInfo_RX(MCPTT, ON_DEMAND, PRIVATE_CALL, SDP_OFFER, v_MediaControlConditionSdpOffer, v_DirectionAttributeRX); // XML Bodies v_XmlBodies := f_MCX_InviteMO_XmlBodies_Init(MCPTT, PRIVATE_CALL); v_XmlBodies.McpttInfo := fl_MCPTT_Info_AmbientListening_RX(v_AmbientListeningType); // @sic R5s220344 sic@ // Receive INVITE v_InviteRequestWithSdpAndXml := f_MCX_InviteMO_ReceiveINVITE(MCPTT, PRIVATE_CALL, -, p_CommencementMode, v_SdpInfoRX, v_XmlBodies); v_InviteRequestWithSdp := v_InviteRequestWithSdpAndXml.InviteRequestWithSdp; v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_InviteRequestWithSdp.RoutingInfo); v_InviteRequest := v_InviteRequestWithSdp.Invite; // Send 100 Trying f_MCX_InviteMO_Send100Trying(v_InviteRequestWithSdp); // Send 200 OK v_Contact := f_MCX_CallSetupMO_ContactSS(MCPTT); v_MessageHeader200OK := f_MCX_CommonResponse_200_MessageHeaderTX(v_InviteRequest.msgHeader, v_Contact, cs_ContentTypeSDP); v_MessageHeader200OK.featureCaps := f_MCX_FeatureCaps_MCPTT_AmbientListeningCallRelease(); v_SDP_Answer := f_MCX_SdpAnswer_TX(MCPTT, ON_DEMAND, v_InviteRequestWithSdp.SdpOffer, v_MediaControlConditionSdpAnswer); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine200, v_MessageHeader200OK, cs_MessageBody_SDP(v_SDP_Answer)))); // Receive ACK f_MCX_InviteMO_ReceiveACK(v_InviteRequest); return v_InviteRequestWithSdp; }

After Change:

function fl_MCX_CallSetupMO_AmbientListening(MCX_CommencementMode_Type p_CommencementMode) runs on MCX_IMS_PTC return IMS_InviteRequestWithSdp_Type { var MCX_InviteRequestWithSdpAndXml_Type v_InviteRequestWithSdpAndXml; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var INVITE_Request v_InviteRequest; var MCX_MediaControlCondition_Type v_MediaControlConditionSdpOffer; var MCX_MediaControlConditionSdpAnswer_Type v_MediaControlConditionSdpAnswer; var MCPTT_Info_AmbientListening_Type v_AmbientListeningType; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (value) Contact v_Contact; var template (value) MessageHeader v_MessageHeader200OK; var template (value) SDP_Message v_SDP_Answer; var template (value) SDP_attribute v_DirectionAttributeRX; var template (present) MCX_SdpInfoRX_Type v_SdpInfoRX; var template (present) MCX_MIME_XmlBodies_Type v_XmlBodies; select (p_CommencementMode) { case (AMBIENT_LISTENING_REMOTE) { v_AmbientListeningType := remote_init; v_DirectionAttributeRX := cs_SDP_Attribute_recvonly; v_MediaControlConditionSdpOffer := withoutImplictGrantRequest; v_MediaControlConditionSdpAnswer := implicitRequestNoted; } case (AMBIENT_LISTENING_LOCAL) { v_AmbientListeningType := local_init; v_DirectionAttributeRX := cs_SDP_Attribute_sendonly; v_MediaControlConditionSdpOffer := withImplictGrantRequest; v_MediaControlConditionSdpAnswer := implicitRequestGranted; } } // SDP info v_SdpInfoRX := f_MCX_SdpInfo_RX(MCPTT, ON_DEMAND, PRIVATE_CALL, SDP_OFFER, v_MediaControlConditionSdpOffer, v_DirectionAttributeRX); // XML Bodies v_XmlBodies := f_MCX_InviteMO_XmlBodies_Init(MCPTT, PRIVATE_CALL); v_XmlBodies.McpttInfo := fl_MCPTT_Info_AmbientListening_RX(); // @sic R5s220344 sic@ // Receive INVITE v_InviteRequestWithSdpAndXml := f_MCX_InviteMO_ReceiveINVITE(MCPTT, PRIVATE_CALL, -, p_CommencementMode, v_SdpInfoRX, v_XmlBodies); // Check extensions f_MCX_McpttInfoExt_DecodeAndMatchAmbientListeningType( v_InviteRequestWithSdpAndXml.XmlBodies.McpttInfo.mcptt_Params.anyExt, v_AmbientListeningType); v_InviteRequestWithSdp := v_InviteRequestWithSdpAndXml.InviteRequestWithSdp; v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_InviteRequestWithSdp.RoutingInfo); v_InviteRequest := v_InviteRequestWithSdp.Invite; // Send 100 Trying f_MCX_InviteMO_Send100Trying(v_InviteRequestWithSdp); // Send 200 OK v_Contact := f_MCX_CallSetupMO_ContactSS(MCPTT); v_MessageHeader200OK := f_MCX_CommonResponse_200_MessageHeaderTX(v_InviteRequest.msgHeader, v_Contact, cs_ContentTypeSDP); v_MessageHeader200OK.featureCaps := f_MCX_FeatureCaps_MCPTT_AmbientListeningCallRelease(); v_SDP_Answer := f_MCX_SdpAnswer_TX(MCPTT, ON_DEMAND, v_InviteRequestWithSdp.SdpOffer, v_MediaControlConditionSdpAnswer); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfo_DL, cs_Response(c_statusLine200, v_MessageHeader200OK, cs_MessageBody_SDP(v_SDP_Answer)))); // Receive ACK f_MCX_InviteMO_ReceiveACK(v_InviteRequest); return v_InviteRequestWithSdp; }

## 2.4 Change 4

| Function name | ff_TC_6_2_18_MCPTT_MCX_IMS |
| --- | --- |
| Reason for change | As explained above, MCPT-Info extensions containing ambient-listening info need to be checked separately. |
| Summary of change | Ambient listening extensions in received MCPTT info are decoded and checked separately. |
| TTCN module | MCPTT_PrivateCall_MCX_IMS.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_2_18_MCPTT_MCX_IMS (…) { … v_McpttInfo := fl_MCPTT_Info_AmbientListening_RX(remote_init); f_MCX_PreestablishedSession_CallSetupMO_ReceiveREFER_Send200OK(MCPTT, PRIVATE_CALL, -, -, AMBIENT_LISTENING_REMOTE, v_McpttInfo, -, v_SdpInfoRX); f_MCPTT_PreestablishedSession_Connect(PRIVATE_CALL, -, px_MCPTT_ID_User_A, WITH_FLOORCONTROL); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 2"); … }

After Change:

function f_TC_6_2_18_MCPTT_MCX_IMS (…) { … v_McpttInfo := fl_MCPTT_Info_AmbientListening_RX(); v_ReferRequestWithSdpAndXml := f_CX_PreestablishedSession_CallSetupMO_ReceiveREFER_Send200OK(MCPTT, PRIVATE_CALL, -, -, AMBIENT_LISTENING_REMOTE, v_McpttInfo, -, v_SdpInfoRX); // Check extensions f_MCX_McpttInfoExt_DecodeAndMatchAmbientListeningType( v_ReferRequestWithSdpAndXm.XmlBodies.McpttInfo.mcptt_Params.anyExt, remote_init); f_MCPTT_PreestablishedSession_Connect(PRIVATE_CALL, -, px_MCPTT_ID_User_A, WITH_FLOORCONTROL); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 2"); … }