3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221353

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0171 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of POS testcase 9.3.4.2.21s |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add POS test case 9.3.4.2.21s to the IWD_22wk23 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test 9.3.4.2.21s required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.4.2.21s.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 37.571-2 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Changes in R5s221043 are required in addition to the changes mentioned in this document |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

1.1	Verification Test Summary	2

2	Changes required to verify test case 9.3.4.2.21s	2

1.1 Correction to f_TC_9_3_4_2_21s_NR5GC	2

3	Branches executed	3

4	Execution Log Files	3

4.1	Mediatek MT6879 UE	3

5	References	4

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of POS test case 9.3.4.2.21s which is part of the POS test suite in iWD_TTCN3-B2020-09_D22wk23.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	9.3.4.2.21s

ATS Version:	iwd-TTCN3-B2020-09_D22wk23

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	Mediatek MT6879 UE

Verification Status:	PASS

# Changes required to verify test case 9.3.4.2.21s

## 1.1 Correction to f_TC_9_3_4_2_21s_NR5GC

| Function name | f_TC_9_3_4_2_21s_NR5GC |
| --- | --- |
| Reason for change | Current implementation for multi cell DL PRS test cases omits the NR cell info configuration for 2nd and 3rd cell. This leads into issues, not properly activating these cells properly. In order to add the NR cell info configuration, a call to f_NR_SetCellPower needs to be added for the extra cells in the multi cell test cases. |
| Summary of change | Include a call to f_NR_SetCellPower(nr_Cell2, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2) |
| TTCN module | NR5GC_Positioning_Testcases.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

function f_TC_9_3_4_2_21s_NR5GC() runs on NR5GC_POS_PTC

{//E-SMLC Initiated Assistance Data Delivery followed by Location Information Transfer: UE-Assisted

f_NR5GC_POS_Init(NR_1);

//Create and configure cell

f_POS_NR_CellConfig_DlPrs(nr_Cell1);

f_POS_NR_CellConfig_DlPrs(nr_Cell2);

//Preamble: The UE is in 5GS state 3N-A

f_NR5GC_Preamble(nr_Cell1, STATE_CONNECTED_3A);

// Table 9.3.4.2.3.1-1: Configuration of measurement gaps for DL-TDOA

f_NR_ConfigMeasGap_OTDOA();

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_TC_9_3_4_2_21s_NR5GC() runs on NR5GC_POS_PTC

{//E-SMLC Initiated Assistance Data Delivery followed by Location Information Transfer: UE-Assisted

f_NR5GC_POS_Init(NR_1);

//Create and configure cell

f_POS_NR_CellConfig_DlPrs(nr_Cell1);

f_POS_NR_CellConfig_DlPrs(nr_Cell2);

//Preamble: The UE is in 5GS state 3N-A

f_NR5GC_Preamble(nr_Cell1, STATE_CONNECTED_3A);

f_NR_SetCellPower(nr_Cell2, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2);

// Table 9.3.4.2.3.1-1: Configuration of measurement gaps for DL-TDOA

f_NR_ConfigMeasGap_OTDOA();<<SKIPPED CODE>>

# Branches executed

## This POS TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n71” using NIA1 Integrity and NEA1 ciphering algorithms

# Execution Log Files

## Mediatek MT6879 UE

The Mediatek MT6879 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
tc_9_3_4_2_21s_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s221050: Supporting information for addition of POS testcase 9.3.4.2.21s in FR1 |
| --- | --- |