3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230060

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2902 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC Test Case 7.1.2.2.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-12-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per TTCN implementation of Prose CR R5-227420, v_RLC_BearerConfigList[0] value is passed in f_NR5GC_RRC_ConnectedState3N_RLC_UM, it points to SRB1 in the RLC Bearer Config List. But later in fl_NR_ExistingDRB_GetRlcBearerConfig the test case fails with access violation because SS tries to access DRB ID of served radio bearer in RLC Config List and it does not exist. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the test case to use DRB i.e v_RLC_BearerConfigList[2] instead of SRB1 in the funnction f_NR5GC_RRC_ConnectedState3N_RLC_UM |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.2.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_2_2_6_NR5GC() |
| --- | --- |
| Reason for change | - As per TTCN implementation of Prose CR R5-227420, v_RLC_BearerConfigList[0] value is passed in f_NR5GC_RRC_ConnectedState3N_RLC_UM, it points to SRB1 in the RLC Bearer Config List. But later in fl_NR_ExistingDRB_GetRlcBearerConfig the test case fails with access violation because SS tries to access DRB ID of served radio bearer in RLC Config List and it does not exist. |
| Summary of change | - Modified the test case to use DRB i.e v_RLC_BearerConfigList[2] instead of SRB1 in the funnction f_NR5GC_RRC_ConnectedState3N_RLC_UM |
| TTCN module | RLC_NR5GC |
| MCC160 Comment |  |

Before Changes

function f_TC_7_1_2_2_6_NR5GC() runs on NR5GC_PTC { // UM RLC / RLC re-establishment procedure var NR_RLC_SS_State_Type v_RLC_Rec; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigListDRB_Only; var template (value) PDCP_Config v_PDCP_Config; var SN_FieldLengthUM v_DL_Len; var SN_FieldLengthUM v_UL_Len; var T_Reassembly v_T_Reassembly; var DRB_Identity v_DRBId; f_NR5GC_Init(NR_1); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); // @sic R5s190312 ch. 4.3 sic@ //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); // Initialization of RLC variables v_RLC_Rec := f_NR_InitRLC_Record(); // @sic R5s190906 ch. 1 sic@ if (pc_um_WithShortSN) { v_DL_Len := size6; v_UL_Len := size6; v_RLC_Rec.UM_SN_Size := 6; } else { v_DL_Len := size12; v_UL_Len := size12; v_RLC_Rec.UM_SN_Size := 12; } v_RLC_Rec.t_Reassembly := 0.02; // @sic R5s190632 sic@ v_SS_Drb_ConfigList := f_NR5GC_GetRadioBearerList(f_NR5GC_MobileInfo_GetDRBInfoList()); // @sic R5-2277420 sic@ v_SS_Drb_ConfigList[0] := cs_NR_SS_RadioBearer_UM_Test(v_DRBId, v_DL_Len); // @sic R5-227420 sic@ v_PDCP_Config := cs_38508_PDCP_Config(-, -, omit); // @sic R5-227518 sic@ v_T_Reassembly := ms200; // @sic R5s190632 sic@ v_RLC_BearerConfigList := f_NR5GC_GetExisting_BearerToAddModList(true_); //@sic R5-227420 sic@ v_RLC_BearerConfigListDRB_Only := f_NR_ReconfigureRLC_UM (v_DRBId, v_DL_Len, v_UL_Len, v_T_Reassembly, true_); v_RLC_BearerConfigList[2] := v_RLC_BearerConfigListDRB_Only[0]; // Get NR capabilities sent during EUTRA preamble f_NR5GC_RRC_ConnectedState3N_RLC_UM (v_DRBId, v_RLC_BearerConfigList[0], v_DL_Len); f_NR_TestBody_Set(true); f_TC_7_1_2_2_6_NR_TestBody(v_RLC_Rec, // @sic R5s190905_r1 sic@ v_DRBId, // @sic R5s220321 sic@ v_DL_Len, v_UL_Len, v_T_Reassembly, v_PDCP_Config, v_SS_Drb_ConfigList); ….

After Changes

function f_TC_7_1_2_2_6_NR5GC() runs on NR5GC_PTC { // UM RLC / RLC re-establishment procedure var NR_RLC_SS_State_Type v_RLC_Rec; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigListDRB_Only; var template (value) PDCP_Config v_PDCP_Config; var SN_FieldLengthUM v_DL_Len; var SN_FieldLengthUM v_UL_Len; var T_Reassembly v_T_Reassembly; var DRB_Identity v_DRBId; f_NR5GC_Init(NR_1); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); // @sic R5s190312 ch. 4.3 sic@ //Set null cipher Algorithm f_NR_AS_CipheringAlgorithm_Set(nea0); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); // Initialization of RLC variables v_RLC_Rec := f_NR_InitRLC_Record(); // @sic R5s190906 ch. 1 sic@ if (pc_um_WithShortSN) { v_DL_Len := size6; v_UL_Len := size6; v_RLC_Rec.UM_SN_Size := 6; } else { v_DL_Len := size12; v_UL_Len := size12; v_RLC_Rec.UM_SN_Size := 12; } v_RLC_Rec.t_Reassembly := 0.02; // @sic R5s190632 sic@ v_SS_Drb_ConfigList := f_NR5GC_GetRadioBearerList(f_NR5GC_MobileInfo_GetDRBInfoList()); // @sic R5-2277420 sic@ v_SS_Drb_ConfigList[0] := cs_NR_SS_RadioBearer_UM_Test(v_DRBId, v_DL_Len); // @sic R5-227420 sic@ v_PDCP_Config := cs_38508_PDCP_Config(-, -, omit); // @sic R5-227518 sic@ v_T_Reassembly := ms200; // @sic R5s190632 sic@ v_RLC_BearerConfigList := f_NR5GC_GetExisting_BearerToAddModList(true_); //@sic R5-227420 sic@ v_RLC_BearerConfigListDRB_Only := f_NR_ReconfigureRLC_UM (v_DRBId, v_DL_Len, v_UL_Len, v_T_Reassembly, true_); v_RLC_BearerConfigList[2] := v_RLC_BearerConfigListDRB_Only[0]; // Get NR capabilities sent during EUTRA preamble f_NR5GC_RRC_ConnectedState3N_RLC_UM (v_DRBId, v_RLC_BearerConfigList[2], v_DL_Len); f_NR_TestBody_Set(true); f_TC_7_1_2_2_6_NR_TestBody(v_RLC_Rec, // @sic R5s190905_r1 sic@ v_DRBId, // @sic R5s220321 sic@ v_DL_Len, v_UL_Len, v_T_Reassembly, v_PDCP_Config, v_SS_Drb_ConfigList); …..