3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230061

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4707 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE ESM testcase 10.7.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-01-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The protocolConfigurationOptions field in the message PDN_CONNECTIVITY_REQUEST that is sent by the UE is declared optional for the UE. The definition in the TTCN should be adapted to accomplish this behavior |  |  |  |  |  |  |  |  |
| Summary of change: |  | The definition of the variable ProtocolConfigOptions v_Pco is changed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A comformant UE may fail the case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

| Function name | fl_TC_10_7_5_Body() |
| --- | --- |
| Reason for change | The protocolConfigurationOptions field in the message PDN_CONNECTIVITY_REQUEST that is sent by the UE is declared optional for the UE. The definition in the TTCN should be adapted to accomplish this behavior. |
| Summary of change | The definition of the variable ProtocolConfigOptions v_Pco is changed |
| TTCN module | Common\EUTRA\EUTRA_CellInfoFrequency.ttcn |
| MCC160 Comment | \LTE\10\ESM_Testcases.ttcn |

Before Change:

function fl_TC_10_7_5_Body() runs on EUTRA_PTC { // @sic R5s110293 sic@ var ProcedureTransactionIdentifier v_EpsTi := tsc_PTI_Unassigned; var EPS_BearerIdentity v_EpsDefaultBearerId2 := '6'H; var EPS_BearerIdentity v_EpsDedicatedBearerId2 := '7'H; var NAS_MSG_Indication_Type v_NasInd; var ProcedureTransactionIdentifier v_PTId_UE; var ProtocolConfigOptions v_Pco; var template (value) AccessPointName v_AccessPointName; var PDN_Address v_PDN_Address2; var SRB_COMMON_IND v_ReceivedAsp; var PDN_CONNECTIVITY_REQUEST v_PDNConnReq; var NasCount_Type v_NasCountUL; var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get(); var template (value) EUTRA_FDD_TDD_CellInfo_Type v_EUTRA_FDD_TDD_Info := f_EUTRA_CellInfo_GetFDD_TDD_Info (eutra_CellA); //Get FDD or TDD mode from Cell-configuration var octetstring v_APN_Value := tsc_APN1; // APN-1(New PDN name), different from default var BEARER_RESOURCE_ALLOCATION_REQUEST v_BEARER_RESOURCE_ALLOCATION_Req; var template (value) ESM_Cause v_EsmCause43; var template (value) NAS_DL_Message_Type v_BearerResourceAllocReject; <CODE SKIPPED>

After Change:

function fl_TC_10_7_5_Body() runs on EUTRA_PTC { // @sic R5s110293 sic@ var ProcedureTransactionIdentifier v_EpsTi := tsc_PTI_Unassigned; var EPS_BearerIdentity v_EpsDefaultBearerId2 := '6'H; var EPS_BearerIdentity v_EpsDedicatedBearerId2 := '7'H; var NAS_MSG_Indication_Type v_NasInd; var ProcedureTransactionIdentifier v_PTId_UE; var template (omit) ProtocolConfigOptions v_Pco; var template (value) AccessPointName v_AccessPointName; var PDN_Address v_PDN_Address2; var SRB_COMMON_IND v_ReceivedAsp; var PDN_CONNECTIVITY_REQUEST v_PDNConnReq; var NasCount_Type v_NasCountUL; var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get(); var template (value) EUTRA_FDD_TDD_CellInfo_Type v_EUTRA_FDD_TDD_Info := f_EUTRA_CellInfo_GetFDD_TDD_Info (eutra_CellA); //Get FDD or TDD mode from Cell-configuration var octetstring v_APN_Value := tsc_APN1; // APN-1(New PDN name), different from default var BEARER_RESOURCE_ALLOCATION_REQUEST v_BEARER_RESOURCE_ALLOCATION_Req; var template (value) ESM_Cause v_EsmCause43; var template (value) NAS_DL_Message_Type v_BearerResourceAllocReject; <CODE SKIPPED>