3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230127

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4711 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for EUTRA MAC test cases 7.1.7.1.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test, LTE_4Rx_AP_DL-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per test spec 36.523-1 Table 7.1.7.1.6a.3.2-3, two dci configurations are used at step 3 and 8 respectively. At step 3, the codeword swap flag=’0’, while current implementation set it to ‘1’. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the codeword swap flag setting to ’0’ at step 3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.7.1.6a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_7_1_6a_EUTRA |
| --- | --- |
| Reason for change | As per test spec 36.523-1 Table 7.1.7.1.6a.3.2-3, two dci configurations are used at step 3 and 8 respectively. At step 3, the codeword swap flag=’0’, while current implementation set it to 1. |
| Summary of change | Corrected the codeword swap flag setting to ’0’ at step 3 |
| TTCN module | MAC_Rel11.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_7_1_6a_EUTRA () runs on EUTRA_PTC { /* L2/MAC/7.2 * 7.1.7.6a : DL-SCH transport block size selection / DCI format 2A / RA type 0 and RA type 1 / * Two transport blocks enabled / 3 and 4 Layer Spatial Multiplexing */ ........................ /////////////////////////////////////////////////////////////////////////////////// // Test Body ////////////////////////////////////////////////////////////////////////////////// f_EUTRA_TestBody_Set(true); f_EUTRA_7_1_7_1_x_4RX(dci_2A, ra_0,'1'B, '01'B); f_EUTRA_7_1_7_1_x_4RX(dci_2A, ra_1,'1'B, '10'B); f_EUTRA_TestBody_Set(false); //test body ends /////////////////////////////////////////////////////////////////////////////////// // Post Amble ////////////////////////////////////////////////////////////////////////////////// ........................ }

After Change

function f_TC_7_1_7_1_6a_EUTRA () runs on EUTRA_PTC { /* L2/MAC/7.2 * 7.1.7.6a : DL-SCH transport block size selection / DCI format 2A / RA type 0 and RA type 1 / * Two transport blocks enabled / 3 and 4 Layer Spatial Multiplexing */ ........................ /////////////////////////////////////////////////////////////////////////////////// // Test Body ////////////////////////////////////////////////////////////////////////////////// f_EUTRA_TestBody_Set(true); f_EUTRA_7_1_7_1_x_4RX(dci_2A, ra_0,'0'B, '01'B); f_EUTRA_7_1_7_1_x_4RX(dci_2A, ra_1,'1'B, '10'B); f_EUTRA_TestBody_Set(false); //test body ends /////////////////////////////////////////////////////////////////////////////////// // Post Amble ////////////////////////////////////////////////////////////////////////////////// ........................ }